3GPP TSG RAN WG2 Meeting #122                                           	R2-2305440

Incheon, Korea, May 22nd – 26th 2023

Agenda item:			7.2.2

Source:			Intel Corporation

Title:		Further considerations on sidelink positioning

Document for:	 	Discussion, Decision

# Introduction

There was extensive discussion in the last RAN2 meeting on sidelink positioning design, including discussion on SL positioning protocol transport, signaling between UE and LMF, support of different cast types and much more. Some key agreements from the meeting related to sidelink positioning are reproduced below [1]:

Agreements: LS on group positioning is postponed for a reply from next meeting. WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2. WA: At least part of the group management for group positioning is performed at upper/application layer. Agreements: Sidelink Positioning Architecture in Figure 1-4 on R2-2304301 is taken as baseline in TS 38.305 for further discussion. RAN2 understanding is that there is no impact to LTE specs from this objective. Agreement: Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2. Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor. Agreement: Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario. Agreement: Anchor UE selection can be included in the sidelink positioning procedures at least for out of coverage scenario. Agreements: RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location). The UE role information is indicated in the discovery SLPP metafield. FFS if this applies to both discovery modes and which messages. Agreements: R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities. RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request. TP in R2-2304005 is postponed. RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)” RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

In this contribution, we discuss the open aspects related to normative work needed for support of sidelink positioning and present our view.

# Discussion

## 2.1	SL Positioning Protocol

In the previous RAN2 meetings, it was finally agreed that the transport the sidelink positioning (control) signaling is done over PC5-U and there was some discussion on the protocol stack for SLPP. In our understanding, the only aspect that needs to be captured in RAN2 specification is a stage 2 depiction of the protocol stack for SLPP. The SLPP signaling between the target UE and the anchor UE and/or the positioning server UE should be over PC5-U protocol, as per SA2 agreement. The main discussion aspect in the last meeting was on the SA2 aspect and we think the only issue is what to capture in the RAN2 specification. Thus, RAN2 should confirm and capture the figure below as part of the stage 2 specification and no further discussion or confirmation with SA2 is needed.

Figure 1 User Plane protocol stack for SLPP

Proposal 1: RAN2 is proposed to agree that as per SA2 LS, SLPP is carried over the V2X/ProSe layer.

## 2.2	Session-based vs session-less operation

With respect to the session-based vs session-less approach discussed in RAN2 in the previous meetings, the following was agreed in RAN2#120 meeting [2]:

Agreement: Sidelink positioning supports a session-based concept in SLPP, in which signalling messages within a session can be associated with one another by the involved UEs. The relationship to upper-layer designs from SA2 can be discussed during normative work. FFS if there is also sessionless operation and what aspects of session-based operation would not be included. Agreement: At least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP sessionless operation can be supported. FFS if sessionless operation can be operated with security.

During the last meeting, the following agreements were made specific to the session-based case :

Agreements:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

TP in R2-2304005 is postponed.

RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)”

RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

While it was agreed that at least a one-to-one relationship exists between SLPP session and a location request, whether multiple target UEs can be supported in a single location request is still FFS. In our understanding, the main use case identified during the discussions in the last meeting by some companies was for the case when some of the UEs may not be able to directly reach the LMF and so an intermediate target UE may be included as part of the session for a single location request. However, in our understanding, this scenario sounds quite similar to the assistant UE defined in SA2. In particular, this is related to “Key Issue 2: Ranging service operation procedure with the assistance of another UE”, whereby the assistant UE(s) can help measurement and data transmission between the anchor and the target UE. However, the assistant UE has been ruled out previously from RAN2 perspective in RAN2 119bis-e meeting:

Agreement: RAN2 do not decide to support the role of assistant UE for now. FFS if there is spec impact in RAN2 from the assistant UE.

The other scenario where multiple target UEs can be applicable is for the support of group positioning, whereby location estimates for multiple target UEs per location request may be needed. However, we think that firstly session-less (rather than session-based) operation is ideally suited for this scenario, since this can avoid the signaling overhead associated with session-based operation. Secondly, even if we assume session-based approach is used and multiple target UEs per location request are assumed, this does not necessarily imply that multiple SLPP session need to be created to serve these target UEs (as was also pointed out in the previous meeting). If session-based operation is needed to support group positioning, we assume a single SLPP session can support multiple target UEs such that the SLPP session can be mapped to the L2 SRC/DST IDs as well as the group IDs in a non-ambiguous way by relying on the transaction identifiers.

Proposal 2: For the case of multiple target UEs per single location request (e.g. for group positioning), a single SLPP session can support the location request.

It is also evident that this design of SLPP and session based operation leaves little room for the sessionless operation, since it is not clear how the ranging/SL Positioning layer operates without having the concept of a well-defined session as previously discussed in RAN2. For sessionless concept, it may be related to broadcast/groupcast discussion, i.e. similar to UE based positioning, the UE may not need the interaction with location server, and therefore no session is needed. RAN2 can decide this once the support of broadcast/groupcast and the associated security aspects are clear. It is also worth noting that if group positioning may end being supported by way of session based operation, the need for session-less operation is even less motivated.

Proposal 3: Postpone the discussion on support of the sessionless operation until the handling of broadcast/groupcast (and the associated security aspects) are clear.

## 2.3	Discovery for SL positioning

With respect to the discovery procedure, there was some discussion on this aspect in the last meeting and the following was agreed:

Agreements:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and which messages.

It was agreed that some information for discovery and candidate selection of SL positioning (anchor) UEs may be included as part of the discovery messages, but what information is included was left FFS. At least the UE role may be included as per SA2 decision, but we need to discuss what information may be included to aid the discovery procedure from RAN2 perspective.

As detailed in TS 23.304, the legacy discovery mechanism relies on exchange of discovery messages (using both Model A and Model B) and may not contain AS layer information specifically related to sidelink positioning. We certainly see some benefits of including some SL positioning related information within the discovery message, e.g. related to supported SL positioning capability or supported positioning methods, etc. The PC5 discovery procedure may be enhanced to include specific information within the discovery message that may aid the support of sidelink positioning. At least the following may be considered as proposed in the summary document in [4]:

a)	SL positioning capabilities, including supported positioning methods by the anchor UE

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

For the case of model A, the target UE may broadcast the discovery announcement message which may include the SL positioning/ranging service code as well as the above information, which implies its interest in finding a suitable anchor UE. An anchor UE that supports the positioning method(s) and other information indicated by the target UE may respond by setting up a direct PC5 connection with the target UE to aid in the positioning session.

In case of model B, the anchor UEs that have been authorized and provisioned by the network to act as the anchor UE for sidelink positioning may periodically broadcast a solicitation message, which can include the service code for SL positioning/ranging as well as the above information, i.e. supported positioning methods, ability to perform location estimate, etc. The target UE which is interested in SL positioning can then respond to the discovery message and set up a PC5 connection with the anchor UE.

Observation 1: With respect to the discovery procedure for the session based approach, AS layer parameters related to sidelink positioning are useful to include as part of the discovery message (e.g. supported positioning methods).

Proposal 4: In addition to the UE role, at least the following information shall be included as part of the discovery messages (for both model A and B):

a)	SL positioning capabilities, including supported positioning methods by the anchor UE

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

## 2.4	Anchor UE selection/reselection

The process of selection of anchor UE for sidelink positioning was a point of discussion in the previous discussion and it was RAN2 assumption that at least part of the selection procedures shall be in RAN2 scope. In this context, there are two different scenarios to consider.

Firstly, when the SL positioning procedure is initiated, the target UE may need to discovery suitable candidate anchor UEs in its vicinity. Once the discovery is performed and the target UE is able to discover such UEs, the next logical step is selection of one or multiple anchor UEs for the sidelink positioning to proceed. Note that unlike the discovery procedure, this step needs to include the LMF and/or the positioning server UE. TO enable this, at least the following options may be considered:

- Option 1: The target UE may provide the list of discovered candidate anchor UEs to the LMF and/or the positioning server UE together with other information, e.g. anchor UE ID, the supported positioning methods, the location calculation capability and the quality of the signal from the anchor UE, etc. This information may be obtained as part of the discovery procedure (as discussed in section 2.3 above and can be provided by the upper layer. Based on this information, the LMF and/or server UE may perform selection of the suitable anchor UE or UEs for the target UE for further selection.

- Option 2: Based on target UE location and/or service request (e.g. QoS, requested positioning method, etc.), the LMF/server UE may perform selection of the suitable anchor UE or UEs for the target UE for further selection based on preconfigured anchor UE lists (i.e. without target UE explicitly providing a list of candidate anchor UEs beforehand, in contrast with Option 1). To enable this, the anchor UEs may directly provide their capability and other pertinent information to the LMF for pre-configuration.

- Option 3: The LMF/server UE may provide a list of candidate anchor UEs to the target UE and the target UE shall select the anchor UE within the list. The target UE may be configured (or preconfigured in case of OOC) by the LMF with selection criteria to use for selection of anchor UE from the discovered candidate anchor UEs (e.g. the quality of the signal from the anchor UE, supported positioning methods, calculation capability, etc.). Based on these criteria, the AS layer shall select and inform the upper layer of the selected anchor UE.

The three options discussed above may broadly fall under either the LMF/Server UE based or LMF/Server UE assisted anchor UE selection methods and it would be good to discuss and downselect (if needed) between the three options above.

Proposal 5: For initial anchor UE selection after discovery, RAN2 is proposed to discuss and downselect between the following options:

- The LMF/server UE based approach, where LMF/server UE may obtain information about candidate anchor UEs (either from target UE itself or from (pre-)configuration) to make the selection.

- The LMF/server UE assisted approach, whereby LMF/server UE may provide selection criteria to the target UE and target UE makes the final selection.

The other case is when the positioning procedure is ongoing and anchor UE may need to be reselected by the target UE. There can be several reasons for this occurrence, the most common being deteriorating channel conditions between the target and anchor UE. Given the nature of sidelink channel and target and anchor UE being mobile, the support of such reselection seems warranted.

Observation 2: Due to the nature of sidelink channel and the mobile nature of the anchor and the target UE, the anchor UE may need to be reselected during an ongoing SL positioning procedure.

For this scenario, the fundamental question to consider is whether the positioning procedure may still continue after the target UE reselects to a different anchor UE, or whether a new positioning session needs to be initiated afterwards. From signaling overhead and latency perspective, it seems useful to continue the ongoing positioning procedure. However, considering the anchor node is changed, the original measurement results are no longer going to be useful. Therefore, the UE may need to report the need for anchor UE reselection to the LMF. It can be based on the error reporting methodology as in LPP or it can be a new message.

Furthermore, RAN2 needs to discuss the triggers for when the anchor UE may need to be reselected. For this case, we can take inspiration from the relay reselection procedure and the following triggers may be considered:

- If there exists a unicast link between the target UE and the anchor UE, the SL link quality (e.g., based on SL-RSRP) can be used to determine whether the anchor UE is considered valid.

- The anchor UE can provide indication to the target UE based on its own criteria, e.g., in case it has to perform power saving and cannot continue with SL-PRS transmission or location estimation.

- Anchor UE can provide a bit indication on whether it can still support the required QoS for SL positioning method/session

Out of the above options, we think the most relevant one to consider is based on link quality. Since RAN1 is discussing use of both shared and dedicated resource pools for SL-PRS, it is likely that similar channel characteristics apply for data and SL-PRS transmissions. Therefore, at least SL link quality based trigger for anchor UE reselection needs to be defined. It is also worth noting that since some of the criteria pertains to the AS layer, we think it is reasonable to assume that the evaluation for (re-)selection of anchor UE shall happen at the AS layer, which then indicates the selected anchor UE to the upper/SLPP layer. The latter is then responsible for triggering the (re-)selection procedure.

Proposal 6: The evaluation for anchor UE selection/reselection is performed at the AS layer, which indicates the selected anchor UE to the upper layer.

Proposal 7:  RAN2 supports the procedure to allows the UE to report the need for anchor UE reselection to the LMF.

Proposal 8:  For anchor UE reselection during an ongoing SL positioning procedure, at least SL link quality based trigger shall be considered.

## 2.5	Signaling between UE and LMF for in coverage

There was extended discussion in RAN2#119bis-e meeting on the protocol options between UE and LMF for the in-coverage scenario and the following three options were captured, with down selection to happen during the normative phase:

Agreement:

Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work.

- Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF

- Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP

- Use of SLPP/RSPP between the UE and the LMF

There was more discussion in the last RAN2 meeting and ultimately there was no decision made. In our understanding, Option 1 seems to have more overhead and specification impact to LPP and should be excluded. Then, among options 2 and 3, we think both options are feasible, but option 2 has the advantage of smaller specification effort and allows decoupling of SLPP and LPP design, so it is preferred from our perspective.

Similarly for the case of PC5-only positioning in coverage, i.e. we assume that the LMF is still involved in the positioning procedure. If so, similar reasoning as above still applies. Even if there is no Uu based positioning and associated (LPP) signaling to consider, LPP can still be utilized to (transparently) carry the SL based positioning signaling between the target/anchor UE and the LMF. Otherwise, we have to discuss what protocol should be used to carry SLPP across the LMF, the serving gNB and the UE which may impact multiple WGs. Therefore, we think both options 2 and 3 are feasible, but we slightly prefer option 2.

Proposal 9: For the case of both hybrid PC5+Uu and PC5-only positioning in coverage, RAN2 is proposed to agree that SLPP signaling can be transported within LPP transparently (i.e. container based approach).

## 2.6	Positioning Server UE

Regarding the positioning server UE, its role has already been agreed in RAN2 and the main issue to consider is whether there is a need to introduce a new node as part of the positioning architecture. In other words, either the target UE or the anchor UE may handle the functionality of the positioning server UE.

Agreement:

Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2.  Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor.

As proposed in the discussions during the previous meetings, we think it is feasible that server UE functionality is supported by either the target UE or the anchor UE, specifically since anchor UE may indicate this capability as part of the discovery messages (as discussed in section 2.3). So, we think it would be good to confirm that in RAN2.

Proposal 10: RAN2 confirms that either the target UE or the anchor UE may handle the functionality of the SL positioning server UE. Therefore, there is no need to explicitly include server UE role in stage 2 positioning procedures.

## 2.7	Signaling Procedures

### 2.7.1	In coverage scenario

We need to consider the potential impact to the sidelink protocol design of supporting sidelink positioning for in coverage and out of coverage scenarios. For the former, we can further consider the PC5 based only positioning and Uu/PC5 based (hybrid) positioning cases. In both the cases, it can be assumed that there exists a connection between the interested UE and the LMF via the direct Uu link. Therefore, in order to ensure that the existing positioning mechanism is reused as much as possible, we think the LPP procedure between UE and LMF over NAS as in legacy should be considered as baseline.

Similar to the Uu based positioning case, the positioning procedure may be initiated by the UE or the LMF for the in-coverage scenario. Note that this is related to the LS in R2-2302260, where SA2 is asked if they will specify the triggering event for a SLPP session. We assume that the legacy location requests from LPP, both MO-LR (whereby the LMF assists the UE in sidelink positioning) and MT-LR (whereby LMF initiates the SL positioning procedure) based sidelink positioning procedures should be supported for the in-coverage case.

Proposal 11: Both MO-LR based and MT-LR based sidelink positioning procedures shall be supported for the in coverage case, using Uu based positioning design as baseline.

The target UE (i.e. the UE interested in positioning) is still responsible for exchanging the capability transfer, AD transfer and location information transfer (as in the case of Uu based positioning) with the LMF via the serving gNB. In addition, depending on whether PC5 based only or PC5 and Uu based (hybrid) positioning is being used, the LMF may also provide the relevant configuration (positioning capability, SL assistance data and SL-PRS configuration) to the anchor UE to assist in sidelink positioning. Following the positioning measurements, the target UE may forward the measurement results to the LMF for positioning calculation. The signaling design for this scenario is shown in the figure below. It is worth noting that this based on the agreements made in the previous meeting on the series of steps for the overall signaling procedure and the figure is just the depiction of the agreed steps.

Figure 2 UE sidelink positioning for in coverage/partial coverage scenario

### 2.6.2	Out of coverage scenario

Finally, in case of out of coverage when both the target UE and the peer UE are considered out of network coverage, we cannot rely on the traditional positioning architecture, specifically the LMF. Instead, the target UE may rely on anchor UE/node to provide the necessary information/configuration in order to perform positioning to obtain location estimates. The high level signaling in that case is depicted in the figure below, where the anchor UE plays the key role by handling the positioning capability exchange, assistance data and location estimation functionality. The location request is sent directly to the anchor node, which can then exchange positioning capability, positioning assistance data and is responsible for scheduling of SL-PRS transmissions to obtain positioning measurements. Also note that the target UE may need to provide positioning measurement results over PC5 to the positioning server UE to obtain the location estimate.

It is also worth noting that in the last meeting, companies are unsure on whether the SL connection establishment is needed between the target UE and anchor UE for the positioning session. In our view, supporting the kind of QoS requirements which the SL positioning use cases require naturally require the establishment of a dedicated unicast connection between the peer UEs. It seems strange to not leverage the mechanisms already in place of sidelink which allow for greater robustness and flexibility to serve different QoS requirements for SL positioning.

Figure 3 UE sidelink positioning for out of coverage scenario

Proposal 12: As per the agreed series of steps for SL positioning procedure, RAN2 confirms the signaling flow for UE based sidelink positioning for in coverage and out of coverage as captured in Figures 2 and 3 above.

## 2.7	Group Positioning

Based on the LS from SA2 [3], it seems that at least the V2X uses cases including platooning are applicable for support of group positioning. In addition, SA2 has captured the following as part of the key issues on Ranging/Sidelink Positioning:

- A Ranging/SL Positioning layer is introduced on the Target UE/Reference UE/SL Positioning Server UE under Application layer and above AS layer to handle service request received from application layer and to control the Sidelink Positioning and Ranging operation: - Functionalities supported by the Ranging/SL Positioning layer include discovery of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions and control signalling between UEs or among a group of UEs or between UE and LMF to manage and coordinate the Sidelink Positioning and Ranging operations. - The group management can be performed at application layer, and the application layer may provide group identifier information to the Ranging/SL Positioning layer. NOTE 1: Potential group management within RSPP layer is out-of-scope of SA2.

In the last RAN2 meeting, there was some discussion in RAN2 on the group positioning aspect and the following agreements were made [1]:

Agreements:

LS on group positioning is postponed for a reply from next meeting.

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.

WA: At least part of the group management for group positioning is performed at upper/application layer.

Based on the discussions in SA2, it is clear that the group management aspect shall be handled by the application layer and there is not impact to 3GPP specifications, at least from SA2 perspective. This is similar to how group operation was designed for NR Sidelink in Rel-16, whereby any group management operations (e.g. discovery for group formation, group members entering or leaving the group, etc.) were transparent to the AS layer. The application layer group ID in this case is managed by the application layer in the UE and/or the application server, which then interacts with the ProSe Layer to determine the Destination L2 ID for use over sidelink interface. From AS layer perspective, the mapping of L2 IDs to specific UEs within the group is handled by the upper layers and no impact is foreseen in RAN2 specifications. Therefore, we propose to further refine and confirm the working assumption, i.e. all group management for group positioning is handled by the upper/application layer and no impact is foreseen to RAN2 spec.

Proposal 13: Based on SA2 conclusions, it is confirmed that group management for group positioning is handled by the upper/application layer and no impact is foreseen in RAN2.

Proposal 14: The group ID and/or L2 Destination IDs for transmission of capability information, assistance information and location request/response shall be provided by the upper layer.

With respect to support of group positioning, the following was agreed in RAN2#120 meeting:

Agreement:

From RAN2 perspective, if it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

This seems to imply that in order to support group positioning across multiple UEs, the location estimation step can be performed in a distributed way, i.e. there is no need for a central node/UE to perform the location estimation and provide it to the group member UEs; each UE is able to perform estimate calculation locally. Note that RAN2 has already confirmed that the SL positioning capability and assistance data can be sent in a groupcast/broadcast way. Therefore, assuming the group management procedure is handled by the upper layers, the session based concept can be applicable for the case of group positioning as well. The signaling flow in this case can be exemplified as below:

Figure 4 Sidelink based group positioning

Proposal 15: RAN2 to confirm the procedure and signaling flow for sidelink based group positioning as captured in Figure 4 above as baseline.

# Conclusion

This contribution discusses aspects related to support of sidelink positioning and makes the following observations and proposals:

Observation 1: With respect to the discovery procedure for the session based approach, AS layer parameters related to sidelink positioning are useful to include as part of the discovery message (e.g. supported positioning methods).

Observation 2: Due to the nature of sidelink channel and the mobile nature of the anchor and the target UE, the anchor UE may need to be reselected during an ongoing SL positioning procedure.

Proposal 1: RAN2 is proposed to agree that as per SA2 LS, SLPP is carried over the V2X/ProSe layer.

Proposal 2: For the case of multiple target UEs per single location request (e.g. for group positioning), a single SLPP session can support the location request. Proposal 3: Postpone the discussion on support of the sessionless operation until the handling of broadcast/groupcast (and the associated security aspects) are clear.

Proposal 4: In addition to the UE role, at least the following information shall be included as part of the discovery messages (for both model A and B):

a)	SL positioning capabilities, including supported positioning methods by the anchor UE

b)	Ability to compute location information based on SL-PRS measurements

c)	Ability to perform absolute vs relative positioning/ranging calculation

Proposal 5: For initial anchor UE selection after discovery, RAN2 is proposed to discuss and downselect between the following options:

- The LMF/server UE based approach, where LMF/server UE may obtain information about candidate anchor UEs (either from target UE itself or from (pre-)configuration) to make the selection.

- The LMF/server UE assisted approach, whereby LMF/server UE may provide selection criteria to the target UE and target UE makes the final selection.

Proposal 6: The evaluation for anchor UE selection/reselection is performed at the AS layer, which indicates the selected anchor UE to the upper layer.

Proposal 7:  RAN2 supports the procedure to allows the UE to report the need for anchor UE reselection to the LMF.

Proposal 8:  For anchor UE reselection during an ongoing SL positioning procedure, at least SL link quality based trigger shall be considered.

Proposal 9: For the case of both hybrid PC5+Uu and PC5-only positioning in coverage, RAN2 is proposed to agree that SLPP signaling can be transported within LPP transparently (i.e. container based approach).

Proposal 10: RAN2 confirms that either the target UE or the anchor UE may handle the functionality of the SL positioning server UE. Therefore, there is no need to explicitly include server UE role in stage 2 positioning procedures.

Proposal 11: Both MO-LR based and MT-LR based sidelink positioning procedures shall be supported for the in coverage case, using Uu based positioning design as baseline.

Proposal 12: As per the agreed series of steps for SL positioning procedure, RAN2 confirms the signaling flow for UE based sidelink positioning for in coverage and out of coverage as captured in Figures 2 and 3 above.

Proposal 13: Based on SA2 conclusions, it is confirmed that group management for group positioning is handled by the upper/application layer and no impact is foreseen in RAN2.

Proposal 14: The group ID and/or L2 Destination IDs for transmission of capability information, assistance information and location request/response shall be provided by the upper layer.

Proposal 15: RAN2 to confirm the procedure and signaling flow for sidelink based group positioning as captured in Figure 4 above as baseline.

# References

| [1] | RAN2#121 meeting, Chairman Notes. |
| --- | --- |
| [2] | RAN2#120 meeting, Chairman Notes. |
| [3] | S2-2301786, Reply LS on SL positioning groupcast and broadcast. |
| [4] | R2-2301921, Summary of AI 8.2.2 Sidelink Positioning (Intel). |