3GPP TSG-RAN WG2 Meeting #122	R2-2305442

Incheon, Korea, May 22-26, 2023

Agenda item:			7.2.4

Source:			Intel Corporation

Title:		Support of LPHAP

Document for:	 	Discussion and decision

# Introduction

In RAN2#121 meeting, RAN2 discussed LPHAP related issues, and concluded:

Agreements:

When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission.

Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.

Agreements:

RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

LS to RAN3 to confirm this.

In RAN2#121bis-e, based on [6], RAN2 further discussed LPHAP related issues, and concluded:

Agreement:

Wait for RedCap progress on extending eDRX cycle (from RAN2 perspective).

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Send a reply LS to SA2 to indicate that from the perspective of RAN2, “low power or high accuracy” positioning is out of the Rel-18 WI scope.

In this contribution, we continue the discussion on leftover issues on LPHAP topic.

# Discussion

### 2.1 Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state

The objective was added by RAN1 although the leading group is RAN2. The main discussion on how to support the extended eDRX cycle beyond 10.24s in RRC_INACTIVE should be done in Rel-18 eRedCap WI. In positioning session, we only need to provide positioning specific value to Rel-18 eRedCap and ask them to capture these values. As concluded in last RAN2 meeting, no RAN2 action is foreseen. However there is no any discussion in RAN1 on eDRX value for positioning purpose, to our understanding, the value should come from RAN1, and therefore an LS to RAN1 is needed.

Proposal 1: Send LS to RAN1, ask them to provide positioning specific value for eDRX cycle beyond 10.24s in RRC_INACTIVE.

### 2.2 SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration

Objective 1:SRS for positioning activation/request procedure(s)

When cell reselection happens out of the positioning validity area, the UE should stop the SRS transmission as in legacy system. In order to maintain the continuity of positioning procedure, the UE should request to the network for SRS configuration update. To this end, we think mechanisms are needed to provide UE with new SRS configuration potentially staying in RRC inactive state in the new cell.

In RAN2#121, RAN2 agreed

Agreements:

RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

LS to RAN3 to confirm this.

In RAN2#121bis-e, RAN2 further agreed

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

For the FFS if the request is in the RRC message or an accompanying MAC CE, to our understanding, there is only 1 valuable bit left in RRCResumeRequest, therefore it would be good to use MAC CE instead as what we did for request of MG gap, and PPW. It can be used to trigger SRS configuration request in the new cell after cell reselection occurs out of the positioning validity area. Then new gNB could provide new SRS configuration/activation in a RRC release message. From power consumption perspective, it is beneficial for the new gNB to maintain the UE in RRC_INACTIVE after UE context retrieval if the request is for SRS update only.

NOTE: the UE can only use MAC CE to trigger SRS configuration request if allowed by the network.

Proposal 2: For SRS for positioning activation/request procedure(s), if allowed by the network, UE sends MAC CE to trigger SRS configuration/activation request when cell reselection occurs outside of validity area.

When the gNB provides the updated SRS configuration to the UE, the serving gNB should provide the updated SRS configuration to the LMF in order to let the LMF forward the updated SRS configuration to measured gNBs. But the details should be discussed in RAN3.

Proposal 3: For SRS for positioning activation/request procedure(s), serving gNB forwards updated SRS configuration to the LMF via NRPPa message when receiving the request from the UE, and the LMF forwards the updated SRS configuration to measured gNBs.  The details to be discussed in RAN3.

Objective 2: SRS for positioning configurations in multiple cells/Pre-configuration of one or multiple SRS for positioning configurations

For adopting a SRS configuration across multiple cells within a validity area, RAN1 is discussing how to handle TA, interference issues, pathloss, spatial relation, and common parameters across multiple cells, for these issues RAN2 can wait for the inputs from RAN1.

Proposal 4: Issues related to TA, interference issues, pathloss, spatial relation, and common parameters across multiple cells should be discussed in RAN1 first.

Based on [6], RAN2 agreed that the SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network. During the discussion, the main question is why do we need validity timer, why TA timer is not sufficient. If TA timer is expired, the UE cannot use the SRS configuration, therefore additional validity timer is not needed.

Proposal 5: Do not introduce validity timer for SRS validity area configuration.

In [6], some companies would like to avoid multiple SRS configurations for the same UE, as

Proposal 2e: to discuss whether to support multiple SRS configurations.

We do not see the benefit why for a single SRS validity area, network should provide multiple SRS configurations. But we do see the value for network to provide multiple SRS configurations for different SRS validity area, i.e. SRS configuration per SRS validity Area.

Proposal 6: For a single SRS validity area, the network can only provide one SRS configuration. The network may provide multiple SRS configurations for multiple SRS validity areas simultaneously.

Another issues in [6] were

Proposal 2c: to discuss whether SRS (pre)configuration can also be provided via posSIB. Proposal 2f: to discuss which node (gNB or LMF) determines SRS validity area.

The problem to support posSIB is, different UEs may select the same set of SRS resources in the same area, and therefore it is difficult to identify which UE is sending the SRS resources. If we want to support SRS via posSIB, we have to specify how to avoid collision. Anyway, dedicated signalling can work well, we do not see the strong need to support SRS via posSIB.

Proposal 7: Do not support SRS configuration provided via posSIB.

For the issue which node (gNB or LMF) determines SRS validity area. The LMF should be aware of the SRS configuration for different gNBs, and therefore it is in the right position to decide this based on the negotiation with gNBs.

Proposal 8: SRS validity area is determined by the LMF based on the negotiation with related gNBs.

From RAN2 perspective, we can look the requirement that how to avoid the gNB to monitor multiple SRS configuration simultaneously for a UE.

To our understanding, the spirit of the objective is, the network provides multiple configurations (per area) to the UE, and the UE can select the SRS based on camped cell. Therefore the network does not need to update the SRS configuration when the UE moves to another cell (in Rel-17, the network only configures the SRS for the cell where the UE moves to RRC_INACTIVE, and the UE will release the SRS configuration when it moves to another cell). We do agree that it can save the power caused by additional RRC signalling. However, the measured gNB has no idea on which cell the UE is camping, i.e. which SRS configuration the UE is transmitting. Therefore, the measured gNB has to measure all candidate SRS configurations for the UE which will increase the processing load of the measured gNB which should be avoided. To avoid the additional complexity on gNB side, RAN2 should find a solution to resolve the issue.

Proposal 9: The measured gNB should be aware of which SRS configuration should be used for the positioning measurement.

If RAN2 concludes the mechanism on SRS for positioning activation/request procedure(s), the similar solution can be used. That is when the UE selects different SRS configuration, e.g. upon moving to another cell or the area for different SRS configuration, the UE shall use MAC CE to indicate the selected SRS configuration (e.g. configuration ID) to gNB, and then same as SRS for positioning activation/request procedure(s), i.e. the gNB indicates this to the LMF, and LMF forwards it to measured gNBs. .

Proposal 10: For preconfigured multiple SRS configurations, if allowed by the network, UE sends MAC CE to indicate the change of SRS configuration when different SRS configuration is selected due to cell reselection.

Proposal 11: For preconfigured multiple SRS configurations,  serving gNB forwards updated SRS configuration to the LMF via NRPPa message when receiving the change indication from the UE, and the LMF forwards the updated SRS configuration to measured gNBs.  The details to be discussed in RAN3.

### 2.3 Support of positioning in RRC_IDLE

UE efficiency (power consumption) was discussed in Rel-17 SI/WI positioning enhancements. The outcome from Rel-17 SI on RRC_IDLE (captured in TR38.837) is:

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. - Note: This does not imply that measurements have to be reported in RRC_IDLE state. The following procedures are considered as feasible for DL positioning methods in RRC_IDLE: - Reporting of DL-PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_INACTIVE/RRC_CONNETED. NOTE: The following procedures are considered to have already been supported and can be reused for positioning in RRC_IDLE - On-demand SI request in RRC_IDLE for assistance data delivery by broadcast in RRC_IDLE - ProvideAssistanceData can be sent in RRC_CONNECTED for DL-PRS configuration used in RRC_IDLE downlink positioning - RequestLocationInformation can be sent in RRC_CONNECTED for DL-PRS measurement and/or location estimate performed in RRC_IDLE

In summary, nothing was introduced in Rel-17; given existing solution can work, RAN2 did not introduce the reporting in RRC_IDLE.

If positioning is supported for RRC_IDLE in Rel-18, the following procedure could be expected (based on Rel-17 conclusion).

- The UE could get assistance data via broadcast signalling or preconfiguration in Connected mode;

- The UE could perform PRS measurement in IDLE mode;

- The UE can report the results (UE based positioning methods) to internal LCS client for MO-LR

- For NI-LR and MT-LR, the UE has to move to CONNECTED mode in order to report;

During study phase, RAN2 discussed the issue and agreed:

Proposal3: DL positioning in RRC_IDLE is recommended to normative work from R2’s perspective if power saving benefits are confirmed by R1. (15/15)

	Measurement is performed in RRC_IDLE while measurement report is sent in RRC_CONNECTED

	Feasibility of measurement report in msg5 should be evaluated with SA2/3 involved.

	Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning measurement was performed in RRC_IDLE, can be evaluated in the WI phase with SA2 involved.

Regarding power saving benefits:

It has been resolved in RAN plenary since RAN plenary has agreed to support measurement in RRC_IDLE and report in RRC_CONNECTED;

Regarding “Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning measurement was performed in RRC_IDLE”:

To our understanding the CN will discard the UE context (including positioning context, positioning session) when the UE moves to IDLE. It is unclear how the AMF is aware of which LMF, GMLC, external LCS client are waiting for the results, and it is also unclear how the AMF still keep track of the positioning session and what positioning methods are used for the UE. Therefore, existing mechanism cannot work well for the positioning in RRC_IDLE. However, it should be confirmed by SA2.

Proposal 12: RAN2 to send LS to SA2, to inform them that RAN has agreed to support “DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state” and would like to check whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning was performed in RRC_IDLE for MO-LR, MT-LR and NI-LR.

### 2.4 Alignment between eDRX and PRS configurations

During SI phase, RAN2 concluded to further consider two enhancement directions:

Agreements:

Proposal2: Alignment between DRX and PRS is beneficial from power saving point of view for LPHAP and is recommended to normative work. (14/15)

	Two directions of solutions for DRX/PRS alignments are considered: (a) PRS alignment with fixed DRX (b) DRX alignment with fixed PRS

	Solutions for the PRS/DRX alignment, e.g., LMF-based/UE-based solution, is to be discussed

	Impacts to different RRC states (RRC_INACTIVE and RRC_IDLE) is to be discussed

Useful information for the alignment:

- LPHAP indication: SA2 has concluded the usage of LPHAP indication [2]

- During the positioning procedure, AMF provides the LPHAP indication to the LMF, either obtaining from the GMLC, or in the UE LCS context which received during UE registration procedure. - LMF is enhanced to receive from AMF of the LPHAP indication in the location request, and determine positioning method, by taking into account the LPHAP requirement. LMF also sends LPHAP indication to RAN in the NRPPa message.

Based on SA2 conclusion, the LMF can get the LPHAP indication from the AMF during the positioning procedure, and the gNB can also get it from the LMF via NRPPa message.

RAN2 can follow SA2 conclusion on this, the only impact to RAN is, the LMF sends the LPHAP indication to RAN in NRPPa message. It is out of RAN2 scope on how the AMF gets the indication, and how the AMF sends the indication to the LMF.

Proposal 13: Confirm SA2 conclusion that during the positioning procedure, AMF provides the LPHAP indication to the LMF, and the LMF also sends LPHAP indication to RAN in the NRPPa message (stage 2 and RAN3 impact).

Considering SA2 also agreed that “LMF also sends LPHAP indication to RAN in the NRPPa message.”, the RAN can be aware of whether a UE is performing positioning and whether LPHAP is required. If the RAN also supports the PRS transmission, based on the LPHAP indication, the RAN can decide what DRX configuration can align with PRS configuration well. Therefore it should be:

- DRX alignment with fixed PRS (handled by RAN based on LPHAP indication and available PRS configuration);

Proposal 14: For DRX alignment with fixed PRS configurations, it is up to RAN to align DRX configuration with fixed PRS based on LPHAP indication obtained from the LMF and available PRS configuration in RAN.

In RAN2#121, based on [3], RAN2 discuss the option on PRS alignment to fixed DRX. We agree with companies’ view that PRS alignment with DRX configuration can be archived by reusing on-demand PRS request. If the UE wants to change PRS configuration to align with DRX configuration, the UE can send on-Demand PRS request to the LMF, and recommend the suitable PRS configuration. And if the DRX configuration is changed, the UE can send a new On-Demand request. But there is no stage 3 impact.

Proposal 15: For PRS alignment with fixed DRX configurations, it can be archived by on-demand PRS request without stage 3 impact. If the UE wants to change PRS configuration to align with DRX configuration, the UE can send on-Demand PRS request to the LMF with suitable recommended PRS configuration.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: Send LS to RAN1, ask them to provide positioning specific value for eDRX cycle beyond 10.24s in RRC_INACTIVE.

Proposal 2: For SRS for positioning activation/request procedure(s), if allowed by the network, UE sends MAC CE to trigger SRS configuration/activation request when cell reselection occurs outside of validity area.

Proposal 3: For SRS for positioning activation/request procedure(s), serving gNB forwards updated SRS configuration to the LMF via NRPPa message when receiving the request from the UE, and the LMF forwards the updated SRS configuration to measured gNBs.  The details to be discussed in RAN3.

Proposal 4: Issues related to TA, interference issues, pathloss, spatial relation, and common parameters across multiple cells should be discussed in RAN1 first.

Proposal 5: Do not introduce validity timer for SRS validity area configuration.

Proposal 6: For a single SRS validity area, the network can only provide one SRS configuration. The network may provide multiple SRS configurations for multiple SRS validity areas simultaneously.

Proposal 7: Do not support SRS configuration provided via posSIB.

Proposal 8: SRS validity area is determined by the LMF based on the negotiation with related gNBs.

Proposal 9: The measured gNB should be aware of which SRS configuration should be used for the positioning measurement.

Proposal 10: For preconfigured multiple SRS configurations, if allowed by the network, UE sends MAC CE to indicate the change of SRS configuration when different SRS configuration is selected due to cell reselection.

Proposal 11: For preconfigured multiple SRS configurations, serving gNB forwards updated SRS configuration to the LMF via NRPPa message when receiving the change indication from the UE, and the LMF forwards the updated SRS configuration to measured gNBs.  The details to be discussed in RAN3.

Proposal 12: RAN2 to send LS to SA2, to inform them that RAN has agreed to support “DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state” and would like to check whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning was performed in RRC_IDLE for MO-LR, MT-LR and NI-LR.

Proposal 13: Confirm SA2 conclusion that during the positioning procedure, AMF provides the LPHAP indication to the LMF, and the LMF also sends LPHAP indication to RAN in the NRPPa message (stage 2 and RAN3 impact).

Proposal 14: For DRX alignment with fixed PRS configurations, it is up to RAN to align DRX configuration with fixed PRS based on LPHAP indication obtained from the LMF and available PRS configuration in RAN.

Proposal 15: For PRS alignment with fixed DRX configurations, it can be archived by on-demand PRS request without stage 3 impact. If the UE wants to change PRS configuration to align with DRX configuration, the UE can send on-Demand PRS request to the LMF with suitable recommended PRS configuration.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] R2-2211131 LS on LPHAP information delivery to RAN (S2-2209591; contact: Huawei)	SA2

[3] R2-2300713	Alignment between DRX and PRS	Apple

[4] R2-2301935	LS on the requirement on low power or high accuracy positioning (S2-2303414; contact: Huawei)	SA2

[5] R1-2302074 LS reply on the requirement for LPHAP	RAN1

[6] R2-2304197	Summary of 7.2.4 LPHAP	Apple