3GPP TSG RAN WG2 Meeting #122           	      	            	                            R2- 2305448

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	7.16.2.1

Source:	Intel Corporation

Title:	Model ID, AIML related capability and functionality mapping

Document for:	Discussion and Decision

# Introduction

In this contribution, we continue discuss RAN2 impact of model ID’s meta information, AI/ML related capabilities, and different life cycle management functionalities mapping to the entity.

# Discussion

## Model ID Meta Information

It is noted that following two terminologies are agreed in RAN1 #111 meeting:

Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality.

Based on the purpose/motivation of model identification and functionality identification in RAN1 agreement, we think that the model ID agreed in RAN2 previous agreement covers both definitions, which was agreed to be used for identifying a model used in LCM.

Observation 1: RAN2 agreed “Model ID” covers model identification and functionality identification defined by RAN1.

To avoid ambiguity, we only use RAN2 terminology on “model ID” in this contribution. In following discussion, we mainly focus on the information/meta data that can be used to identify a model’s uniqueness.

Applicable scenario/configuration/site

For training an AI/ML model, the applicable scenario of this model relies on the data collected and used for model training.

It was agreed in RAN1 that they may study the potential specification impact to enable a set of specific models.

Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

One may optimize a specific AI/ML model for a specific scenario/site/configuration, one may also train a generalized/unified model by considering multiple types of input data from different scenarios/configuration, etc. However, such generalized model may lose certain accuracy for specific scenario, but it could be useful when adapting to unexpected scenarios. Hence, when identifying an AI/ML model, the network and UE needs to know whether a model is site-specific, scenario-specific (and its supported scenario), configuration-specific (and its configuration) or a unified model.

Model and computation Complexity

When performing model activation/deactivation/switching/fallback, except the model performance and system performance, one of the important selection criteria is based on the AI/ML’s complexity. One needs to select the AI/ML model with suitable complexity that the model inference entity can handle and process. During RAN1 discussion, AI/ML complexity is further separated into model complexity and computation complexity.

- Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

Following RAN1 agreement, “number of real-value model parameters” and “number of real-value operations” could be considered as model complexity.

Agreement For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

Furthermore, different use case may also have its specific calculation of model complexity, for example, positioning use case’s model complexity is shown as below:

|  | Model complexity to support N TRPs |
| --- | --- |
| Single-TRP, same model for N TRPs | PS where PS is the model complexity for one TRP and the same model is used for N TRPs. |
| Single-TRP, N models for N TRPs | i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. |

Hence, to successfully identify a model during model activation/deactivation/switching, model and computational complexity need to be considered as meta data for an AI/ML model.

Model Performance

Similar as AI/ML complexity, to enable NW/UE select a suitable AI/ML model for specific request, the expected model performance can be shared to the entity that is responsible for model selection as part of the meta data. The model performance may include model accuracy, model bias, model variance, etc.

Use case

Based on current RAN1 discussion, following use cases are being studied:

- CSI compression (two-side model)

- CSI prediction (one-side model)

- Beam management – Case 1

- Beam management – Case 2

- AI/ML assisted positioning

- Direct AI/ML positioning

To successfully map the model with the corresponding usage and use case, the use case information can be considered as meta info for an AI/ML model.

Proposal 1: Following meta information as model description are considered for Model ID:

- Applicable scenario, configuration, site information

- Computational complexity: FLOPs, level of pre-/post-processing

- Model complexity: number of real-value model parameter, number of real-value operations

- Model size

- Model performance: Model accuracy, model bias, model variance

- Supported use case

## AI/ML related Capability

In general, there are two types of AI/ML related capability to support AI/ML model running at the UE side:

- Functionality for a given sub-use case

- UE hardware capability for successfully processing/running the AI/ML model

We discuss detailed aspects for these two types of AI/ML capability.

Type 1: Functionality for a given sub-use case

For the first type, compared with legacy physical layer processing, AI/ML model inference may have certain performance shift. If the AI/ML model is purely trained/inferenced at the UE side, e.g. UE-sided model, the network should still be able to control whether the UE is using legacy approach or using AI/ML model to make sure a good system performance. To allow network has the visibility of UE’s capability of AI/ML, the UE needs to report what use case it can support by AI/ML. It is similar to legacy new UE features, where UE capability reporting is used to report the supported feature at UE side. It was agreed in RAN1 #112 meeting with following agreement:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

Proposal 2: At least for functionality based LCM, UE should report supported AI/ML sub-use case(s) and associated functionalities/functionality to the network via existing UE capability reporting procedure, i.e. RRC reporting for CSI/BM use case and LPP reporting for positioning use case(e.g. explicitly or implicitly by supported model ID, depends on the model ID structure discussion).

Type 2: UE hardware capability for successfully processing/running the AI/ML model

Type 2  would be more relevant for two-sided model or UE-sided model where model is downloaded from network side. It is different from legacy UE capability as it requires UE has specific hardware capability to be able to process the received AI/ML model from the network. For example, to successful receive the model, the UE should have enough memory to store the trained model, therefore, the UE should report its supported maximum memory size to store a model.

Additionally, the UE should also be able to compile and execute the received AI/ML model. To support that, the network needs to understand the supported AI/ML structure by the UE, e.g. CNN, LSTM, RNN, etc.

Performing AI/ML model inference may require a lot of power consumption at UE side. It is better for the UE to report its maximum battery capacity and its current battery status to the network, so that the network may consider a proper duration for model activation and deactivation.

Different from legacy UE capability which is a fixed information, UE supported hardware capability may be dynamically changed based on UE’s current status, where UE capability reporting procedure may not be suitable.

Based on above discussion, the UE hardware capability can further split into:

- Static hardware capability

- Dynamic hardware capability

Proposal 3: For two-sided model or UE-sided model where model is transferred from network to the UE, UE reports its static hardware capability (e.g. maximum model size, maximum battery capacity, supported model structure, etc) and dynamic hardware capability (e.g. current battery status, current memory) as AI/ML related capability to the network.

## LCM functionality and entity mapping

During RAN2 #121bis-e meeting, following LCM framework components were agreed:

- The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

In this section, we further discuss the possibilities of entity mapping based on above proposed LCM framework.

#### NW-sided Model

RAN2 impact: data collection from UE to NG-RAN to support model monitoring and model inference.

#### UE-sided Model + Level x

RAN2 impact: no RAN2 impact.

#### UE-sided Model + Level y

Option 1: Model monitoring at UE side

RAN2 impact: Configuration on how UE perform model monitoring (general guidance/policy from NW).

Option 2: Model monitoring at gNB side

RAN2 impact: 1) data collection from UE to NG-RAN to support model monitoring 2) model inference outcome report from UE to NG-RAN 3) signaling support for model activation/deactivation/switching 4) configurations for data reporting

#### Two-sided Model + Level z

In this section, we show how LCM functionalities are mapped to different network entities. Note that UE-sided model + level z shares the same RAN2 impact and entity mapping, except there’s no model inference at NW side.

Observation 2: Except model inference at NW side, UE-sided model with level z share the same LCM entity mapping and RAN2 impact with two-sided model with level z.

Option 1: Model monitoring at gNB side

RAN2 impact: 1) data collection from UE to NG-RAN to support model monitoring 2) model inference outcome report from UE to NG-RAN 3) signaling support for model activation/deactivation/switching 4) configurations for data reporting 5) model transfer from NG-RAN to UE (in any format, including parameter, proprietary model format, open format, etc)

Option 2: Model monitoring at UE side

RAN2 impact: 1) data collection from NG-RAN to UE to support model monitoring 2) signaling support for model activation/deactivation/switching with NW participation in the final decision 3) configurations for data reporting, activation/deactivation/switching trigger event, etc 4) model transfer from NG-RAN to UE (in any format, including parameter, proprietary model format, open format, etc)

Proposal 4: RAN2 to adopt above functinoality entity mapping in the TR.

# Conclusion

In this contribution, we analyze the general architecture with functionality mapping , model ID related open issue and AI/ML related capability with following observation and proposal:

Observation 1: RAN2 agreed “Model ID” covers model identification and functionality identification defined by RAN1.

Proposal 1: Following meta information as model description are considered for Model ID:

- Applicable scenario, configuration, site information

- Computational complexity: FLOPs, level of pre-/post-processing

- Model complexity: number of real-value model parameter, number of real-value operations

- Model size

- Model performance: Model accuracy, model bias, model variance

- Supported use case

Proposal 2: At least for functionality based LCM, UE should report supported AI/ML sub-use case(s) and associated functionalities/functionality to the network via existing UE capability reporting procedure, i.e. RRC reporting for CSI/BM use case and LPP reporting for positioning use case(e.g. explicitly or implicitly by supported model ID, depends on the model ID structure discussion).

Proposal 3: For two-sided model or UE-sided model where model is transferred from network to the UE, UE reports its static hardware capability (e.g. maximum model size, maximum battery capacity, supported model structure, etc) and dynamic hardware capability (e.g. current battery status, current memory) as AI/ML related capability to the network.

Observation 2: Except model inference at NW side, UE-sided model with level z share the same LCM entity mapping and RAN2 impact with two-sided model with level z.

Proposal 4: RAN2 to adopt the proposed functinoality entity mapping in the TR.

# References