3GPP TSG RAN WG2 Meeting #122      	      	                	                R2-2305449

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	7.16.2.2

Source:	Intel Corporation

Title:	Further analysis on data collection framework

Document for:	Discussion and Decision

# Introduction

During RAN2 #121 meeting, the existing data collection frameworks are summarized and compared from different aspects, e.g. supported payload size, content to be collected, E2E latency, etc. However, what type of data needs to be collected for AI/ML purpose and existing data collection frameworks are sufficient enough requires further analysis. During RAN2 #121bis-e meeting, RAN2 further progressed on how to expand existing data collection table to incorporate with different use cases, LCM purposes, etc.

In this contribution, we first discuss the types of data that needs to be collected for different use cases. For each type of data, we discuss the collection/transmission requirement and analyze whether further optimization is required.

# Discussion

## Data collection requirement

### Model Inference

To generate real-time model inference output and act correspondingly in the real-time wireless system, for model inference, the data is collected and reported in real-time manner and size of collected data is usually small.

For current use cases studied in RAN1, the model inference for those use cases (e.g. CSI, BM, positioning) only runs when the UE is in RRC_CONNECTED state. Therefore, it is straightforward that the network only needs to collect model inference data when UE is in RRC_CONNECTED state.

Proposal 1: Data collection for model inference requires a framework to collect small size of data in real-time in RRC_CONNECTED state.

### Model (Offline) Training

The data collected for model training purpose not only includes input data for model training, but also includes a set of data that can be used for model validation and model testing. Depends on whether data is collected for online training or offline training, the data collection requirement could be different.

It was agreed in RAN2 #121bis-e meeting to deprioritize online training during SI phase.

- R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

In this section, we mainly discuss data collection for offline training.

In our understanding, from content-wise, the data collected by model inference are the same used for model (offline) training, i.e. the same input for an AI/ML model. The fundamental difference between data collection for model inference and model (offline) training is the latency requirement.

As discussed above, since the model inference needs to make real-time decision to execute in the wireless system, the input for model inference (i.e. data collection of model inference) needs to be real-time. For model training, especially when training procedure is offline, the data collection can be done in a non-real time manner or even offline manner.

Observation 1: data collection for model training can be either non-real-time or offline.

As for the data size, for offline training, the dataset is usually used once throughout the entire model for training model weights and parameters. Normally, the dataset is considered large to contain various scenario to generate a suitable model, either unified model or site/configuration/scenario-specific model. However, since model training is offline, there’s also no need to transmit the large size of dataset over the air interface in real-time, since such sharing and synchronization between network and UE can also be done in an offline manner. For continuous model training offline, one network entity can continuously store the data used for model inference, then send the bundle of collected data to offline for model update, which is also perform offline. For offline model training which is transparent to 3GPP network, it is up to implementation how to collect data from the network, as well as the frequency of performing data collection.

Proposal 2: Large size of dataset for offline training is transmitted between network and UE in an offline manner, which is outside of 3GPP.

### Model Monitoring

As captured in R1-2304148 [1], it can be observed that the model monitoring can be summarized in following two aspects:

1) monitoring wireless related system performance or measurement: it can be defined per use case. For example, throughput, BLER, NACK/ACK for CSI compression, beam measurement for beam management, UE location for positioning.

2) monitoring model performance feedback: This information is mainly generated by model inference function based on the accuracy of the model output. For example, SGCS/NMSE/data drift between training dataset and observed dataset/out-of-distribution detection for CSI compression, confidence/probability information related to the output for beam management, statistics of the difference between output and ground truth label for positioning.

Since data collection for model monitoring is used for network to observe the performance of one operating AI/ML model and decide to activate/deactivate/fallback in real-time, data collection for model monitoring shall also be real-time. Though some model performance feedback may require a longer period to generate, for example, collecting data and then generate distribution, statistics and/or probability, it still requires the network to collect/report such information immediately to guarantee the system performance.

Proposal 3: Data collection for model monitoring requires a framework to collect small size of data in real-time in RRC_CONNECTED state.

## Use Cases

### CSI feedback enhancement

Based on R1-2304148 [1], we here summarized the agreed data to be collected in RAN1 for different LCM purposes:

- Time-domain CSI prediction (UE sided model)

| Model inference | Alt 1) raw channel matrices; Alt 2) Eigenvector(s) |
| --- | --- |

- CSI compression (two-sided model)

| Model inference | - CSI-RS measurement - SRS and/or CSI-RS measurement, e.g. Precoding matrix, channel matrix (NW sided) |
| --- | --- |
| Model monitoring | - Intermediate KPIs: SGCS and/or NMSE to evaluate the accuracy of AI/ML output CSI - Eventual KPIs: throughput, BLER, NACK/ACK - Data drift between training data set and observed dataset - Out-of-distribution detection - output of CSI reconstruction model (UE side monitoring) - realistic channel estimation reported by the UE (network side monitoring) |

As discussed in the companion contribution [2] and [3], gNB and UE will hold model inference functionality of CSI feedback enhancement use case, and gNB may make decision for UE-side and NW-side performance monitoring. Therefore, to avoid overhead caused by data transfer between different network entities, it is suggested that the termination of data collection for model inference and model monitoring for CSI feedback enhancement should be the same as the network entity holding model inference and model monitoring functionality.

It is observed that the model inference data, e.g. precoding matrix, eigenvector, can be supported by existing or enhanced L1 signaling (e.g. CSI reporting). Furthermore, for CSI compression use case, RAN1 also requires higher accuracy measurement for CSI-RS configuration, SRS and/or CSI-RS measurement, CSI reporting, etc. However, we believe those enhancements is the scope of RAN1, which may be built on existing CSI reporting mechanism.

Observation 2: Enhancement to support higher accuracy measurement and model inference data will be studied by RAN1.

It was agreed in RAN1 #110bis-e meeting that, network is responsible to make decisions of model activation/deactivation/update/switching no matter it is network or the UE monitors the performance. For UE-side monitoring with network decision, since CSI reconstruction model is located at network side, it first requires the network first sends the model inference output to the UE for UE-side data collection, then UE sends back the model monitoring performance indicator to the network for model control decisions. Based on existing data collection framework, only CSI reporting framework can be designed as bi-directional data collection. Therefore, we propose:

Proposal 4: For CSI feedback enhancement use case, CSI reporting is used for UE-side data collection to collect output of CSI reconstruction model..

For other data collected for model monitoring at network side, model monitoring could be terminated at gNB. Additionally, since model performance feedback is model-specific and does not touch user privacy issue, there’s no need to consider user privacy when reporting model performance feedback. Hence, RRM measurement could be considered to enhance for model performance feedback data collection.

Proposal 5: For CSI feedback enhancement NW-side data collection, RRM measurement framework is considered as baseline for model monitoring model performance feedback data collection.

### Beam management

The agreed data to be collected in RAN1 for different LCM purposes for beam management is summarized as below:

- UE-side model

| Model inference | - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception, which may or may not have additional specification impact (e.g., legacy mechanism may be reused) |
| --- | --- |
| Model monitoring | - Beam measurement (both UE side monitoring and NW side monitoring) |

- NW side model

| Model monitoring | - Beam measurement based on a set of beams indicated by gNB |
| --- | --- |

Furthermore, RAN1 also agreed to use L1 signaling to report the beam related information to the network (e.g. beams that is based on the output of model inference, beams of N future time instances, measurement results of more than 4 beams in one reporting instance, etc)

L1 signalling: For BM-Case1 with a UE-side AI/ML model: - L1 signalling to report the following information of AI/ML model inference to NW: - The beam(s) that is based on the output of AI/ML model inference. For BM-Case2 with a UE-side AI/ML model: - L1 signalling to report the following information of AI/ML model inference to NW: - The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference. For BM-Case1 and BM-Case2 with a network-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams)

Hence, since model monitoring for beam management is the existing beam measurement, the existing RRM measurement and L1 signaling can be reused as data collection framework for beam management.

Proposal 6: L1 and L3 measurement can be considered as data collection framework for beam management model inference and model monitoring.

### Positioning accuracy enhancement

The agreed data to be collected in RAN1 for different LCM purposes for positioning accuracy enhancement is summarized as below:

- Direct AI/ML positioning

| Model inference | - New measurement: CIR, PDP - Existing measurement: RSRP, RSRPP, RSTD |
| --- | --- |

- AI/ML assisted positioning

| Model inference | - New measurement: ToA, path phase (model output to LMF) - Existing measurement: RSTD, LOS NLOS indicator, RSRPP |
| --- | --- |
| Model monitoring | - estimated UE location - estimated intermediate parameters - ground truth label - other metrics, e.g. statistics of measurement, relative displacement, inference output inconsistency, statistics of the difference between output and ground truth label |

Different from CSI feedback enhancement and beam management use case, data collected for positioning depends on the side-ness of model inference, e.g. UE-sided model, NW-sided model, LMF-sided model.

For UE-sided model, as captured in [1], the UE may need assistance signaling from the network to configure what data to be collected. Since model inference and model monitoring for UE-sided model is also located at the UE-side, there’s no need for the UE to report the measurement data to the network. Instead of using any existing data collection framework (which require data to be reported to the network), the configuration of data collection at UE side may be consider as part of model configuration.

Observation 3: For positioning UE-sided model, data collection framework is not needed as model inference and model monitoring is performed at UE side.

For NW-sided model and LMF-side model, based on the termination of the data collection, it is straightforward to use existing LPP measurement procedure, respectively, for model inference data collection and model monitoring data collection.

Proposal 7: For positioning accuracy enhancement, LPP procedure are considered as data collection framework for model inference and model monitoring.

## Summary

|  | Omitted existing columns | Model (Offline) Training | Model Inference | Model Monitoring |
| --- | --- | --- | --- | --- |
| Logged MDT | … | N/A (out scope of 3GPP) | N/A (reason: termination of data collection is not suitable) | N/A (reason: termination of data collection is not suitable) |
| Immediate MDT | … | N/A (out scope of 3GPP) | N/A (reason: termination of data collection is not suitable) | N/A (reason: termination of data collection is not suitable) |
| L3 measurements | … | N/A (out scope of 3GPP) | Suitable for: - CSI feedback enhancement NW-side data collection - Beam management - Positioning accuracy enhancement (NW-side model) | Suitable for: - CSI feedback enhancement NW-side data collection - Beam management - Positioning accuracy enhancement (NW-side model) |
| L1 measurement (CSI reporting) | … | N/A (out scope of 3GPP) | Suitable for: - CSI feedback UE-side data collection - CSI feedback NW-side data collection - Beam management | Suitable for: - CSI feedback NW-side data collection - Beam management |
| UAI | … | N/A (out scope of 3GPP) | N/A (reason: content/data type to be collected is not suitable) | N/A (reason: content/data type to be collected is not suitable) |
| Early measurements | … | N/A (out scope of 3GPP) | N/A (reason: not for RRC_CONNECTED state) | N/A (reason: not for RRC_CONNECTED state) |
| LPP | … | N/A (out scope of 3GPP) | - Positioning accuracy enhancement (LMF-side model) | - Positioning accuracy enhancement (LMF-side model) |

Proposal 8: RAN2 to consider above summary table of framework used for different types of data collection as baseline in the TR.

# Conclusion

In this contribution, we analyze different types of data to be collected for different LCM purpose and select the suitable data collection framework for transmission, with following observations and proposals:

Proposal 1: Data collection for model inference requires a framework to collect small size of data in real-time in RRC_CONNECTED state.

Observation 1: data collection for model training can be either non-real-time or offline.

Proposal 2: Large size of dataset for offline training is transmitted between network and UE in an offline manner, which is outside of 3GPP.

Proposal 3: Data collection for model monitoring requires a framework to collect small size of data in real-time in RRC_CONNECTED state.

Observation 2: Enhancement to support higher accuracy measurement and model inference data will be studied by RAN1.

Proposal 4: For CSI feedback enhancement use case, CSI reporting is used for UE-side data collection to collect output of CSI reconstruction model..

Proposal 5: For CSI feedback enhancement NW-side data collection, RRM measurement framework is considered as baseline for model monitoring model performance feedback data collection.

Proposal 6: L1 and L3 measurement can be considered as data collection framework for beam management model inference and model monitoring.

Observation 3: For positioning UE-sided model, data collection framework is not needed as model inference and model monitoring is performed at UE side.

Proposal 7: For positioning accuracy enhancement, LPP procedure are considered as data collection framework for model inference and model monitoring.

Proposal 8: RAN2 to consider above summary table of framework used for different types of data collection as baseline in the TR.

|  | Omitted existing columns | Model (Offline) Training | Model Inference | Model Monitoring |
| --- | --- | --- | --- | --- |
| Logged MDT | … | N/A (out scope of 3GPP) | N/A (reason: termination of data collection is not suitable) | N/A (reason: termination of data collection is not suitable) |
| Immediate MDT | … | N/A (out scope of 3GPP) | N/A (reason: termination of data collection is not suitable) | N/A (reason: termination of data collection is not suitable) |
| L3 measurements | … | N/A (out scope of 3GPP) | Suitable for: - CSI feedback enhancement NW-side data collection - Beam management - Positioning accuracy enhancement (NW-side model) | Suitable for: - CSI feedback enhancement NW-side data collection - Beam management - Positioning accuracy enhancement (NW-side model) |
| L1 measurement (CSI reporting) | … | N/A (out scope of 3GPP) | Suitable for: - CSI feedback UE-side data collection - CSI feedback NW-side data collection - Beam management | Suitable for: - CSI feedback NW-side data collection - Beam management |
| UAI | … | N/A (out scope of 3GPP) | N/A (reason: content/data type to be collected is not suitable) | N/A (reason: content/data type to be collected is not suitable) |
| Early measurements | … | N/A (out scope of 3GPP) | N/A (reason: not for RRC_CONNECTED state) | N/A (reason: not for RRC_CONNECTED state) |
| LPP | … | N/A (out scope of 3GPP) | - Positioning accuracy enhancement (LMF-side model) | - Positioning accuracy enhancement (LMF-side model) |

# References

[1] R1-2304148, TP for TR for Study on AI-ML for NR air interface

[2] R2-2305448, Model ID, AIML related capability and functionality mapping, Intel

[3] R2-2305451, model control procedure: RAN2 impact, Intel