3GPP TSG-RAN WG2 Meeting #122	R2-2305455

Incheon, May 22-26, 2023

Source: 	Vodafone

Title: 		Definition of NES and barring on cell DTX/DRX cells

Document for: Decision

Agenda Item: 7.3.4

## Discussion

RAN WG2 decided during the last meeting that:

Agreements:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

We believe that this agreement needs to be enhanced as it is not clear what non-NES UEs means and what should be understood under backwards compatibility.

NON-NES UEs:

It seems straightforward to assume that all pre-release 18 UEs are non-NES. At the same time Rel-18 UEs might not be NES-UE either as it can be assumed that not all UEs will implement every NES feature.

The main question under discussion is if there is a need to provide any access restriction based on the fact that a particular NES feature is not implemented by the UE. As per now, the only NES feature discussed in terms of access restrictions is the cell DRX/DTX which assumes a common (long) CELL DRX/DTX cycle. Therefore it is proposed to change non-NES UEs to non-CELL DRX/DTX capable UEs

Proposal 1: It is proposed to call non-NES UEs, Non-CELL DRX/DTX capable UEs

NES solution is backwards compatible:

From the wording of the agreement it is not very clear what “NES solution is backwards compatible” means. Normally, backwards compatible means that a new UE can operate in a cell of previous release. Anyway, we agreed that:

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

This agreement means that legacy UEs and non-cell DTX/DRX capable UEs can in principle work in the cell where CELL DRX/DTX is switched on.

Proposal 2: Based on the above, we would like to propose the following rewording of the agreement:

- RAN2 confirms that non-cell DTX/DRX capable UEs can access to the cell where cell DTX/DRX is activated

Barring of the UEs from the cells implemented CELL DRX/DTX:

As it can be seen from RAN 1 NES Study TR 38.864, the benefit in terms of energy savings using cell DRX/DTX can only be achieved without service degradation if the load of the cell is low (between 15-30%). This means that the amount of the UEs entering such a cell should be minimized as much as possible. We could also observe that due to the SPS, CG and SR limitations, the support of especially real time traffic might be a balance between energy savings and ensuring of QoS.

Once more and more cell DTX/DRX capable UEs come to the network, those UEs might select/reselect to DTX/DRX capable cells. To be able to control the amount of UEs selecting the cell with CELL DRX/DTX, a barring mechanism need to be provided.

Observation 1: Operator has to have the ability to bar UEs from entering cells which deployed cell DRX/DTX and such a mechanism should be designed in a flexible way from the beginning allowing to control the amount of UEs supporting and not supporting cell DTX/DRX operation to enter the cell supporting/activated cell DTX/DRX.

Legacy UEs and Barring:

There are multiple ways to bar the UEs in NR from selecting a particular cell. This can be achieved (some examples are below) by

The other mechanism is to bar completely or partially, where UEs belonging to a particular Access Identity:

Observation 2: There are sufficient mechanisms to bar legacy UEs from accessing cell DRX/DTX.

At the same time, the current mechanisms do not allow to distinguish the UEs between the UEs supporting and not supporting cell DTX/DRX and therefore we see as beneficial to introduce a new barring mechanism for UEs supporting cell DTX/DRX. It would also be beneficial to introduce it in such a way it does not impact other working groups and could also be used with barring factor.

Proposal 3: It is proposed to introduce an additional barring mechanism which could be activated for NES capable UEs only.

Reselections:

Some companies e.g. in R2-2301857 proposed e.g.  “Proposal 5: RAN2 is encouraged to find a solution where NES capable cells, using new cell selection and cell re-selection signaling, can deprioritize cells not upgraded with NES features from being selected for camping”.

As explained above, in order to achieve energy savings with DRX/DTX functionalities the load and also the number of UEs using such cells shall be minimized and therefore it is contra productive to  “deprioritize cells not upgraded with NES features from being selected for camping”.

The most practical deployment would be the deployment on particular a frequency layer. We also assume there is normally other frequency layers available once Cell DTX/DRX is activated.

At the current stage, considering the Note from the WI: “The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided”. We do not see enough motivation to introduce specific reselection mechanism.

It should also be considered that any additional mechanisms would need to be implemented on the UE side and would introduce an additional market segmentation and one single barring mechanism should be sufficient.

Proposal 4: At the current stage we do not see enough motivation to introduce specific reselection mechanism and propose to de-prioritize these discussions

## Conclusions:

Proposal 1: It is proposed to call non-NES UEs, Non-CELL DRX/DTX capable UEs and re-define it only if there are any other NES features identified which have an impact to access control.

Proposal 2: Based on the above, we would like to propose the following re-definition of the agreement:

- RAN2 confirms that non-cell DTX/DRX capable UEs can access to the cell where cell DTX/DRX is activated

Proposal 3: It is proposed to introduce an additional barring mechanism which could be activated for NES capable UEs only.

Proposal 4: At the current stage we do not see enough motivation to introduce specific reselection mechanism and propose to de-prioritize these discussions

## References

R2-2301857	Legacy UEs and NES UEs accessing to NES cells	BT plc, NTT DOCOMO, Turkcell	discussion	Rel-18

R2-2300703   Mechanism of legacy UE barring in NES cell	Apple	discussion