3GPP TSG-RAN WG2 Meeting #122          	          R2-2305470

Incheon, Korea, May 22-26, 2023

Agenda Item:	6.1.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on the TRS availability

Document for:	Discussion and decision

Introduction

For handling the TRS availability during TRS configuration modification, it is specified that the UE considers the configured TRS(s) to be unavailable until the new L1 TRS availability indication is received. However, regarding the handling of L1 indication, there still can be ambiguity during such a SI change procedure.

In this contribution, we discuss the issue of handling L1-based availability indication during SI change for SIB17 and propose possible clarifications.

Discussion

Issue analysis

According to TS 38.331 [1] highlighted below, upon receiving SIB1 indicating that SIB17 has changed, the UE should consider its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s).

#### – SIB17 SIB17 contains configurations of TRS resources for idle/inactive UEs. SIB17 information element -- ASN1START -- TAG-SIB17-START SIB17-r17 ::= SEQUENCE { segmentNumber-r17 INTEGER (0..63), segmentType-r17 ENUMERATED {notLastSegment, lastSegment}, segmentContainer-r17 OCTET STRING } SIB17-IEs-r17 ::= SEQUENCE { trs-ResourceSetConfig-r17 SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17, validityDuration-r17 ENUMERATED {t1, t2, t4, t8, t16, t32, t64, t128, t256, t512, infinity, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Need S lateNonCriticalExtension OCTET STRING OPTIONAL, ... } ...... -- TAG-SIB17-STOP -- ASN1STOP SIB17 field descriptions. trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17 with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17 has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s)..

Since there is a time gap between the transmission of SIB1 and SIB17, there can be two different instances where the first L1 based availability indication can be received by the UE. The timeline of SIB17 change and L1-based availability indication transmission is illustrated in Fig.1, and it can be seen from the figure that there can be two distinct cases based on the current spec:

- Case 1: the UE receives the first L1-based availability indication between SIB1 and SIB17 (t1), i.e. after receiving the SIB1 indicating that SIB17 has changed while before acquiring the updated SIB17.

- Case 2: the UE receives the first L1-based availability indication after receiving the updated SIB17 (t2).

Fig.1 Cases of UE receiving SIB17 and the first L1-based availability indication

For case 2, the UE behaviour is clear. Even though the updated TRS configuration has been acquired, the UE will consider the TRS(s) to be unavailable until the associated validity duration is determined based on the indication received at t2.

For case 1, however, it is ambiguous whether/how the UE should handle this indication:

- If such an indication sent before the updated SIB17 is not considered to be valid, the UE should wait until it receives a new indication sent after the updated SIB17 (i.e. the same as Case 2, at t2).

- Or if this indication should be applied, the UE needs to determine and maintain the validity duration upon receiving the indication at t1, thus may start to utilize the TRS(s) after SIB17 is acquired without waiting for another indication.

If the UE and the network are not aligned, different TRS validity durations will be maintained and thus UE paging monitoring may be impacted due to failed background operations (e.g. time/frequency synchronization) before paging.

Observation 1: Based on the current TRS mechanism and procedures, it is possible that the L1-based availability indication is sent after SIB1 indicating that SIB17 has changed but before updated SIB17 during a SI change.

Observation 2: It is not clear whether the L1-based TRS availability indication sent between SIB1 indicating that SIB17 has changed and updated SIB17 should be applied. Different understandings will result in UE and network maintaining different TRS validity durations, which causes UE synchronization issues and impacts paging monitoring.

Possible ways for clarification

To resolve the ambiguity of TRS availability indication handling during SI change, there are two possible ways.

- Option 1: the L1-based TRS availability indication received before the updated SIB17 is ignored

For simpler handling of the TRS availability, one straightforward way is that the new validity duration is determined and maintained only after the updated SIB17 is already in use. The UE would not need to calculate the validity duration in advance before there is a valid TRS configuration.

Therefore, it should be clarified that during the SI change procedure, the L1-based TRS availability indication received before the updated SIB17, if any, should be ignored by the UE and the UE only applies associated L1 indications after it has received SIB17.

An example for clarification is given below.

SIB17 field descriptions

trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17 with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17 has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s) after the changed SIB17 has been acquired.

- Option 2: the L1-based TRS availability indication received before the updated SIB17 is applied

Considering that both the network and the UE know the timing of SIB and indication transmission, another understanding may be that the L1 indication can be sent in advance of the updated SIB17 and the validity duration is maintained accordingly, such that the UE would have a chance to utilize the configured TRS(s) sooner after it receives the new TRS configuration.

In this option, the TRS validity duration can be determined before the UE receives the updated TRS configuration and is applied immediately to the new TRS(s) when the configuration is in use.

The current field description can be improved for clarity as follows.

SIB17 field descriptions

trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17 with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17 has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s) which may be sent before or after the changed SIB17.

For simplicity, we think Option 1 is a better way.

Proposal 1: Clarify how to handle the L1-based TRS availability indication sent between SIB1 and updated SIB17 during the SI change procedure as follows:

- (Option 1) The L1-based TRS availability indication received before the updated SIB17, if any, should be ignored by the UE and the UE only applies associated L1 indications after the updated SIB17 has been acquired.

Conclusion

In this paper, we discuss the issue of handling TRS availability indication during the SI change procedure. Observations and proposals are summarized as follows.

Observation 1: Based on the current TRS mechanism and procedures, it is possible that the L1-based availability indication is sent after SIB1 indicating that SIB17 has changed while before updated SIB17 during a SI change.

Observation 2: It is not clear whether the L1-based TRS availability indication sent between SIB1 indicating that SIB17 has changed and updated SIB17 should be applied. Different understandings will result in UE and network maintaining different TRS validity durations, which causes UE synchronization issues and impacts paging monitoring.

Proposal 1: Clarify how to handle the L1-based TRS availability indication sent between SIB1 and updated SIB17 during the SI change procedure as follows:

- (Option 1) The L1-based TRS availability indication received before the updated SIB17, if any, should be ignored by the UE and the UE only applies associated L1 indications after the updated SIB17 has been acquired.

References

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification (Release 17).