3GPP TSG-RAN WG2 Meeting #122	R2-2305472

Incheon, Korea, May 22-26, 2023

Agenda item:	7.19.2

Source:	Huawei, HiSilicon

Title:	Discussion on enhanced eDRX in RRC_INACTIVE

Document for:	Discussion and Decision

- Introduction

In RAN2#121e, the following agreements on enhanced eDRX in RRC_INACITVE have been achieved.

Introduce an optional UE capability with signalling for Rel-18 enhanced eDRX in RRC_INACTIVE. UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_IDLE eDRX. TBD if it must also support Rel-17 RRC_INACTIVE eDRX. A cell can allow Rel-18 INACTIVE eDRX, only if eDRX-AllowedIdle is configured. TBD if it must also configure Rel-17 RRC_INACTIVE eDRX. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell. Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX. Introduce a new IE for INACTIVE eDRX to include the eDRX cycle values larger than 10.24s. Following cases are invalid: - Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle. - Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle. RAN PTW length is mandatorily present within Rel-18 INACTIVE eDRX’s configuration. Use the same UE_ID_H as IDLE eDRX for calculating the PH for RAN paging when INACTIVE eDRX is longer than 10.24s. Use TeDRX_RAN instead of TeDRX_CN to calculate the PH for RAN paging when TeDRX_RAN is longer than 10.24s. For the overlapping PH, RAN PTW starting location is determined based on CN eDRX cycle. For the non-overlapping PH, PTW starting location for RAN PTW is determined based on the CN eDRX cycle. In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}. Legacy systemInfoModification-eDRX indication in Short message and eDRX modification boundaries are also applicable for Rel-18 UEs configured with INACTIVE eDRX > 10.24sec, and in this case, the CN eDRX cycle is used to compare with the modification period.

In this paper, we will discuss some INACTIVE eDRX related open issues.

- Discussion

  - UE capability and NW configuration

An optional UE capability with signaling for Rel-18 enhanced eDRX in RRC_INACTIVE is introduced. And it is agreed that UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_IDLE eDRX.

The relationship between the new Rel-18 enhanced INACTIVE eDRX indicator and existing extendedDRX-CycleInactive-r17 should be identified. One possible understanding is that the UE may indicate support for Rel-18 enhanced INACTIVE eDRX only if it supports extendedDRX-CycleInactive-r17. In this case, supporting Rel-18 enhanced INACTIVE eDRX implies that the UE also supports the eDRX in RRC_INACTIVE with values of 256, 512 and 1024 radio frames. Another possible understanding is that the new Rel-18 enhanced INACTIVE eDRX indicator is independent from existing extendedDRX-CycleInactive-r17. If the UE indicates support of Rel-18 enhanced INACTIVE eDRX, it means UE only supports the eDRX in RRC_INACTIVE with values from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024. We think the former understanding is more realistic, i.e., UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_INACTIVE eDRX.

Proposal 1: UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_INACTIVE eDRX.

Correspondingly, a cell can allow Rel-18 enhanced INACTIVE eDRX, only if Rel-17 RRC_INACTIVE eDRX is configured.

Proposal 2: A cell can allow Rel-18 enhanced INACTIVE eDRX, only if Rel-17 RRC_INACTIVE eDRX is allowed.

  - PTW mechanism

We first discuss the PTW length for CN PTW and RAN PTW, according to agreements in RAN2#121, the RAN PTW length can be different from the CN PTW length.

RAN2 confirms the R17 agreements made at RAN2#114 for enhanced INACTIVE eDRX: - - It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length.

Observation 1: RAN2 (at RAN2#121 meeting) has already agreed that the RAN PTW length can be different from the CN PTW length.

Besides, the determination of PTW length is usually related to DRX cycle value, for example, the PTW length is 4 times of DRX cycle. As specified that the INACTIVE DRX cycle is shorter than or equal to IDLE DRX cycle, the corresponding RAN PTW should be shorter than or equal to the CN PTW, in this case, the same number of paging reception opportunities can be provided. A restriction on RAN PTW length always being same as the CN PTW length is not preferred considering RAN implementation flexibility, especially considering that RAN PTW length is MANDATORILY present within Rel-18 INACTIVE eDRX’s configuration which was agreed.

Proposal 3: The RAN configured PTW length can be shorter or equal to the CN configured PTW length.

If Proposal 3 is agreed, for a Rel-18 UE in RRC_INACTIVE, if both CN eDRX cycle and RAN eDRX cycle are longer than 10.24s, there may be three kinds of PTW as illustrated in Fig.1. In a non-overlapped PH, there is only RAN PTW; in an overlapped PH, there are can be CN PTW and overlapping PTW. To be noted, the CN PTW only case occurs in overlapped PH when the RAN PTW is shorter than the CN PTW.

Fig.1 Three kinds of PTW in case that both CN eDRX cycle and RAN eDRX cycle are longer than 10.24s

The T for different PTWs are given as below:

- In a non-overlapped PH, for RAN PTW only, i.e. within RAN PTW, T = RAN configured DRX cycle;

- In an overlapped PH, for CN PTW only, i.e. within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information};

- In an overlapped PH, for overlapping PTW, i.e. within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

Thus, it is a valid case: in an overlapped PH: within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}.

Proposal 4: RAN2 to confirm it is a valid case: in an overlapped PH, within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}.

  - Fallback behaviour

A new optional IE is introduced for INACTIVE eDRX to include the eDRX cycle values larger than 10.24s. Thus, from signalling perspective, a Rel-18 UE can be configured with both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX. If the gNB determines to let UE (if capable) be able to perform fallback NACTIVE eDRX, the gNB can configure both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX to the UE; otherwise, the gNB can configure only Rel-18 enhanced INACTIVE eDRX to the UE. It is up to NW implementation whether to configure Rel-18 enhanced INACTIVE eDRX along with Rel-17 INACTIVE eDRX.

Proposal 5: gNB should configure both Rel-17 INACTIVE eDRX cycle value and Rel-18 enhanced INACTIVE eDRX cycle value to the UE, if it intends to allow the UE to fall back to use the Rel-17 INACTIVE eDRX cycle value. It is up to NW implementation whether to configure Rel-17 INACTIVE eDRX cycle along with Rel-18 enhanced INACTIVE eDRX cycle.

If the anchor gNB configures both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX to the UE, when performs RAN paging, the anchor gNB shall deliver both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration to the receiving gNBs in RAN area.

Observation 2a: The anchor gNB delivers both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration over XnAP when performing RAN paging for a UE configured with both Rel-17 INACTIVE eDRX cycle value and Rel-18 enhanced INACTIVE eDRX cycle value.

Observation 2b: A new R18 gNB, upon receiving this XnAP message, can use the Rel-17 INACTIVE eDRX cycle value to page the UE if fallback is supported.

Observation 2c: A legacy gNB, upon receiving this XnAP message, will only use the Rel-17 INACTIVE eDRX cycle value to page the UE, if allowed by this legacy gNB, and ignore the Rel-18 enhanced INACTIVE eDRX cycle value.

Suppose the UE enters into a cell supporting Rel-18 enhanced INACTIVE eDRX, the UE is agreed to apply Rel-18 enhanced INACTIVE eDRX, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell. Correspondingly, the receiving Rel-18 gNB shall apply Rel-18 enhanced INACTIVE eDRX, regardless of whether Rel-17 INACTIVE eDRX is received from the anchor gNB.

Suppose the UE enters into a cell without supporting Rel-18 enhanced INACTIVE eDRX, like a cell belonging to Rel-17 gNB, the Rel-18 UE shall stop using the Rel-18 enhanced INACTIVE eDRX. If eDRX-AllowedInactive-r17 is present in the cell, considering the UE configured with Rel-17 INACTIVE eDRX additionally, the UE shall use Rel-17 INACTIVE eDRX (<= 10.24s) for RAN paging in RRC_INACTIVE. Correspondingly, the Rel-17 gNB is only aware of the Rel-17 INACTIVE eDRX configuration since it could not recognize the new introduced IE for Rel-18 enhanced INACTIVE eDRX configuration. As a result, the Rel-17 gNB use Rel-17 INACTIVE eDRX (<= 10.24s) for RAN paging in RRC_INACTIVE;

Proposal 6: RAN2 to confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell.

- Conclusion

The corresponding observations and proposals are listed as below:

UE capability and NW configuration

Proposal 1: UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_INACTIVE eDRX.

Proposal 2: A cell can allow Rel-18 enhanced INACTIVE eDRX, only if Rel-17 RRC_INACTIVE eDRX is allowed.

PTW mechanism

Observation 1: RAN2 (at RAN2#121 meeting) has already agreed that the RAN PTW length can be different from the CN PTW length.

Proposal 3: The RAN configured PTW length can be shorter or equal to the CN configured PTW length.

Proposal 4: RAN2 to confirm it is a valid case: in an overlapped PH, within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}.

Fallback behaviour

Proposal 5: gNB should configure both Rel-17 INACTIVE eDRX cycle value and Rel-18 enhanced INACTIVE eDRX cycle value to the UE, if it intends to allow the UE to fall back to use the Rel-17 INACTIVE eDRX cycle value. It is up to NW implementation whether to configure Rel-17 INACTIVE eDRX cycle along with Rel-18 enhanced INACTIVE eDRX cycle.

Observation 2a: The anchor gNB delivers both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration over XnAP when performing RAN paging for a UE configured with both Rel-17 INACTIVE eDRX cycle value and Rel-18 enhanced INACTIVE eDRX cycle value.

Observation 2b: A new R18 gNB, upon receiving this XnAP message, can use the Rel-17 INACTIVE eDRX cycle value to page the UE if fallback is supported.

Observation 2c: A legacy gNB, upon receiving this XnAP message, will only use the Rel-17 INACTIVE eDRX cycle value to page the UE, if allowed by this legacy gNB, and ignore the Rel-18 enhanced INACTIVE eDRX cycle value.

Proposal 6: RAN2 to confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell.