3GPP TSG-RAN WG2 Meeting #122			     R2-2305476

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.25.1

Source: 	LG Electronics Inc.

Title:         	Measurement reporting for FR2 unknown SCell

Document for: 	Discussion and Decision

Introduction

In the R18 WI NR_RRM_enh3, RAN4 has achieved some agreements on RRM enhancements for FR2 unknown SCell activation and an LS [1] has been sent to RAN2, as follows:

- How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement - Technical enhancement goal: - network configures UE to report L3 measurement result when triggering conditions in (2) are met. - It was RAN4 understanding that a one-time report may be configured. The type of report configuration may be RAN2 decision. - Action request to RAN2: - RAN4 requests RAN2 to design corresponding signalling for the technical enhancement goal above - How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement - Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering. - Action request to RAN2: - RAN4 requests RAN2 to decide: - How the measurement report gets triggered by SCell activation command. RAN4 request RAN2 to design the corresponding signaling and decide whether a new event is needed to achieve above technical enhancement goal. - Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report - Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - Agreement (in 2nd round issue 1-1-5 of R4-2306315) - UE reports L3 measurement result with SSB index for unknown FR2 SCell activation enhancement. - Action request to RAN2: - RAN4 requests RAN2 to design corresponding signaling for “report of L3 measurement result” after SCell activation command for unknown FR2 SCell activation enhancement. - It’s up to RAN2’s decision on which layer’s signaling shall be used to send the report to NW

In this contribution, we discuss the impact to specifications from RAN2 perspective based on the LS from RAN4.

Discussion

According to the LS [1], UE needs to reports L3 measurement result with SSB index for unknown FR2 SCell after SCell activation command. RAN4 also agreed it’s up to RAN2’s decision on which layer’s signaling shall be used to send the report to network.

We already have the measurement reporting procedure which is optimized for reporting L3 measurement results from the UE to the network. It was designed to report the measurement results per cell based on SS/PBCH block(s) and SS/PBCH block(s) indexes. Additionally, the measurement results per SS/PBCH block also can be reported via the measurement reporting procedure, if configured by network. Since no TU is allocated to RAN2 for this WI, we should try to use the existing procedure, if there are no critical issues, to minimize the standardization efforts of RAN2. Therefore, we propose to use the measurement reporting procedure to report L3 measurement result of FR2 unknown SCell.

Proposal 1  	The measurement reporting procedure is used to report L3 measurement result of the FR2 unknown SCell.

When an unknown SCell is activated, the measurement results on other serving cells than the activated SCell may not need to be reported. However, in the existing measurement report procedure, the measurement results on all serving cells for which servingCellMO is configured are mandatorily reported in Measurement Report message. Considering the SCell activation does not occur very often, it does not seem to be desirable to modify the measurement reporting procedure to exclude the measurement results on other serving cells just for the purpose of signalling reduction. Therefore, it is proposed that when the measurement report is triggered by SCell activation command, the UE reports SSB based measurement results on all serving cells, i.e. cell results and measurement information per beam, for which servingCellMO is configured.

Proposal 2	If the measurement report is triggered by SCell activation command, UE reports SSB based measurement results on all serving cells, i.e. cell results and measurement information per beam, for which servingCellMO is configured without measurement results on neighbour cells.

UE doesn’t know which SCell is unknown cell from network perspective. To avoid the UE reporting measurement results when it is not needed, i.e. when a known SCell is activated, the network should explicitly inform the UE whether the L3 measurement reporting is necessary or not, when transmitting the SCell activation command.

Proposal 3	When activating unknown SCell, network explicitly indicates whether to report the L3 measurement results of unknown SCell or not, e.g. via enhanced SCell activation command.

The purpose of the L3 measurement report for unknown FR2 SCell is to shorten the SCell activation delay, hence the measurement reporting should be initiated as soon as the SCell activation command is received. If the measurement report is delayed to satisfy another condition, it would no longer be useful. However, it is still FFS in RAN4 whether to have additional events for report triggering, so RAN2 should wait further input from RAN4 to decide which report type to use or whether to introduce a new report type for measurement reporting of FR2 unknown SCell.

Proposal 4	Wait further input from RAN4 to decide which report type to use or whether to introduce a new report type for measurement reporting of FR2 unknown SCell.

Though a single SCell is activated, if the measurement report procedure is used to report L3 measurement results of the activated SCell, the measurement results of other SCells will be reported together. Therefore, the measurement identity does not need to be separately configured per each SCell, and network only needs to configure at most one measurement identity associated with the new report type across all CGs.

Proposal 5	Network can configure at most one measurement identity associated with the new report type across all CGs.

Conclusion

Proposal 1  	The measurement reporting procedure is used to report L3 measurement result of the FR2 unknown SCell.

Proposal 2	If the measurement report is triggered by SCell activation command, UE reports SSB based measurement results on all serving cells, i.e. cell results and measurement information per beam, for which servingCellMO is configured without measurement results on neighbour cells.

Proposal 3	When activating unknown SCell, network explicitly indicates whether to report the L3 measurement results of unknown SCell or not, e.g. via enhanced SCell activation command.

Proposal 4	Wait further input from RAN4 to decide which report type to use or whether to introduce a new report type for measurement reporting of FR2 unknown SCell.

Proposal 5	Network can configure at most one measurement identity associated with the new report type across all CGs.

Reference

[1] R4-2306321 LS on FR2 SCell enh