3GPP TSG-RAN WG2 Meeting #122                                                        R2-2305484

Incheon, Korea, May 22-26, 2023

Source:	CATT

Title:	Further discussion on inter-RAT SHR and SPR

Agenda Item:	7.13.4

Document for:	Discussion and Decision

Introduction

In RAN2#121 meeting, inter-RAT SHR and SPR related issues were discussed and achieved the following agreements [1].

Agreement:

1: For Q1 in the LS R2-2211160, RAN2 agrees to reduce/avoid the impact on LTE specification to support inter-RAT SHR.

2: For handover from NR to LTE,UE generates the NR SHR when SHR for inter-RAT mobility is triggered due to T310 or T312 trigger threshold is fulfilled.

3: For HO from NR to LTE, UE records the SHR for inter-RAT mobility in the VarSuccessHO-Report.

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

5:  A new EUTRA target cell CGI is introduced in inter-RAT SHR.

6: For HO from NR to LTE, the T310 and T312 threshold is provided to the UE by source gNB in the otherConfig.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

Agreement

1: UE includes available location information in SPR .

2: UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

3:  At least the following options are needed for releasing  SPR report:

a. New SPR is initiated

b. Upon retrieval of SPR

c. Detach is initiated.

4:  In SPR, reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

In this contribution, we will continue to discuss the leftover issues about inter-RAT SHR and SPR in section 2.

Discussion

SPR

- SPR configuration

In RAN2#120 meeting, SPR configuration was discussed and made the following agreements [2]:

4 For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.: For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN. T304 trigger needs to be configured by the target SN node. 1 UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

Based the agreements above, the UE only stores one SPR configuration, for legacy PSCell change procedure, we consider the following issues which need to be further discussed and addressed.

If SN hopes to configure SPR configuration (T310/T312 triggering conditions) and trigger PSCell change procedure, SN will send the SPR configuration within the container through MN before sending SN-initiated PSCell change command, then UE could receive MN-initiated PSCell change command, UE has no idea about the PSCell change type (i.e. MN-initiated PSCell change or SN-initiated PSCell change), UE cannot determine whether it should record SPR or not. Furthermore, UE could record the SPR and report it to SN based on SN configured SPR configuration even if the PSCell change is triggered by MN which is not what we expect.

Observation 1: The UE cannot determine whether it should record SPR or not and furthermore it could record the SPR and report the SPR to SN based on SN configured SPR configuration when receives MN-initiated PSCell change command as UE has no idea about the PSCell change type.

If the UE can get to know the PSCell change type (e.g. by network indication), the UE will record SPR based MN/SN configured SPR configuration only when MN/SN triggers the PSCell change. In case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command, the UE will not record the SPR even if the SPR trigger conditions are met. The SPR configuration will be released after successful PSCell change which means the SPR configuration is inefficient.

Observation 2: If the UE can get to know the PSCell change type, the SPR configuration is inefficient in case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command.

In addition, if SN sends SPR configuration to UE, the MN could also send SPR configuration to UE since the MN doesn’t know the UE has stored SN configured SPR configuration, the MN configured SPR configuration will override SN configured SPR configuration, and vice versa.

Observation 3: MN/SN configured SPR configuration can override SN/MN configured SPR configuration as MN/SN doesn’t know the UE has stored SN/MN configured SPR configuration.

For CPC scenario, MN-initiated CPC and SN initiated CPC could be configured to UE simultaneously, if both MN and SN hope to configure SPR configuration to UE, it cannot be achieved based current agreement, i.e.  the UE only stores one SPR configuration. Furthermore, whether MN-initiated CPC or SN-initiated CPC is performed is unpredictable, if only one SPR configuration is stored in UE, there is a possibility that the SPR configuration is useless, e.g. the SPR configuration stored in UE is configured by MN, but the UE executes SN-initiated CPC.

Observation 4: MN and SN cannot configure SPR configuration to UE simultaneously in case of both MN-initiated CPC and SN-initiated CPC are configured to UE.

Observation 5: The SPR configuration could be useless as it is unpredictable for MN-initiated CPC or SN-initiated CPC is performed.

Considering above, we prefer to store two SPR configurations in UE to address the issues mentioned above.

Proposal 1: RAN2 to agree that UE stores two SPR configurations configured by MN and SN respectively.

- PSCell change/CPC type indication information

In RAN3#119bis-e meeting, RAN3 discussed the PSCell change type indication information issue and achieved the follow agreement.

Agreements: To assist in the forwarding of SPR, UE may include the following in SPR - CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed) - WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

From RAN2’s perspective, for legacy PSCell change, MN needs to indicate the PSCell change type to UE as the UE may not know the PSCell change type (i.e. MN-initiated inter-SN PSCell change or SN-initiated inter-SN PSCell change). For intra-SN PSCell change without MN involvement, the source SN sends the PSCell change command by SRB3. The UE could get to know the PSCell change type based on the receiving path.

Observation 6: For legacy PSCell change, the PSCell change type indication information needs to be indicated to UE.

For MN-initiated CPC and SN-initiated CPC, the UE can get to know the CPC type by measId as the execution conditions configured for MN-initiated CPC are based on MN configured measConfig and the execution conditions configured for SN-initiated CPC are based on SN configured measConfig. UE can differentiate SN-initiated intra-SN CPC from SN-initiated inter-SN CPC by the receiving path, i.e. SN-initiated inter-SN CPC configuration is configured in MN RRCReconfiguration message, however, SN-initiated intra-SN CPC configuration is configured in SN RRCReconfiguration meesage which as container in MN RRCReconfiguration message. Therefore, no extra indication information is needed for UE to identify the CPC type.

Observation 7: For CPC, no extra indication information is needed for UE to identify the CPC type as the UE can get to know the CPC type by implicit way.

Based on the above, we propose,

Proposal 2: RAN2 to confirm that for legacy PSCell change, the PSCell change type indication information is introduced to inform UE the executed PSCell change type.

Inter-RAT SHR

- Inter-RAT HO from NR to LTE

In RAN2#121 meeting, whether the following two contents need to be included in inter-RAT SHR when HO from NR to LTE was discussed but no consensus was achieved.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE: a. C-RNTI (FFS target or source) c. FFS: Time between report generating and fetching

C-RNTI

In R17 intra-NR SHR, C-RNTI was discussed and C-RNTI of target cell was introduced to correlate the SHR and RLF report in case of RLF at target cell soon after successful handover to target cell. In R18 inter-RAT SHR from NR to LTE, if the association is supported by network, it seems to be difficult for network to achieve it as it needs to wait for another report (SHR/RLF report) when receiving one of the report (RLF report/SHR) at most 48 hours, considering that the source NR node cannot know whether there is SHR/RLF report is related to the received RLF/SHR report. In addition, if the C-RNTI is included in inter-RAT SHR from NR to LTE, it will be encoded as NR format, but the RLF report will be encoded as LTE format, we are not sure whether the NR node can decode and get the C-RNTI in LTE RLF report. Therefore, we prefer not to include C-RNTI in inter-RAT SHR from NR to LTE.

Proposal 3: C-RNTI is not included in inter-RAT SHR from NR to LTE.

Time between report generating and fetching

In RAN2#121 meeting, whether “time between report generating and fetching” should be included in inter-RAT SHR from NR to LTE was discussed but no consensus was achieved. Considering that the similar timer (timeSinceFailure) has been supported to report in RLF report to help failed/target node to search context information for optimization, therefore, we can also include it in R18 inter-RAT SHR from NR to LTE.

Proposal 4: Time between report generating and fetching is included in R18 inter-RAT SHR from NR to LTE.

- Inter-RAT HO from LTE to NR

After RAN3#119bis-e meeting, an LS was sent to RAN2 from RAN3 about inter-RAT SHR as follow [3].

Inter-RAT SHR RAN3 has agreed to support collection of SHR during successful inter-RAT HO (LTE to NR) but only if there is no impact on LTE. With this in mind, RAN3 agreed to only consider the T304 trigger. This agreement will be revisited if RAN2 detects impact on LTE. This decision further relied on the following assumptions which needs to be confirmed by RAN2: - Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand - UE stores this SHR configuration in NR format - If T304 trigger is met, UE records SHR in NR format - UE reports this SHR only to an gNB (either the target gNB or another gNB) In case the above scenario is also considered feasible for RAN2, RAN3 has identified that at least the following information is beneficial to be reported from the UE: - Source LTE cell identity - Target NR cell identity

Based on the RAN3’s LS above, it is nature for RAN2 to agree to support inter-RAT SHR from LTE to NR for T304 trigger.

Proposal 5: RAN2 confirms to support inter-RAT SHR from LTE to NR for T304 trigger.

Furthermore, RAN1 has pointed out the assumptions (highlighted in yellow) which need to be confirmed by RAN2. From RAN2’s perspective, for inter-RAT handover from LTE to NR, in order to avoid the impact on LTE specification which has been agreed in RAN2#121 meeting, it is reasonable that the UE will generate the NR SHR when SHR for inter-RAT mobility is triggered due to T304 trigger threshold is fulfilled, and the UE will only report the inter-RAT SHR report to the NR network. In addition, it is fine to target gNB to send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand.

Therefore, we propose,

Proposal 6: RAN2 confirms the following assumptions which included in RAN1’s LS:

- Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand

- UE stores this SHR configuration in NR format

- If T304 trigger is met, UE records SHR in NR format

- UE reports this SHR only to an gNB (either the target gNB or another gNB)

In [3], RAN3 has identified that at least source LTE cell identity and target NR cell identity are beneficial to be reported from the UE. Based on this, for inter-RAT handover from LTE to NR, considering the source cell is EUTRA cell, a new EUTRA source cell CGI needs to be introduced in inter-RAT SHR.

Proposal 7: Source LTE cell identity and target NR cell identity are included in inter-RAT SHR from LTE to NR.

Proposal 8: For inter-RAT HO from LTE to NR, a new EUTRA source cell CGI is introduced in inter-RAT SHR.

Conclusion

According to the discussion in section 2, we propose:

Observation 1: The UE cannot determine whether it should record SPR or not and furthermore it could record the SPR and report the SPR to SN based on SN configured SPR configuration when receives MN-initiated PSCell change command as UE has no idea about the PSCell change type.

Observation 2: If the UE can get to know the PSCell change type, the SPR configuration is inefficient in case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command.

Observation 3: MN/SN configured SPR configuration can override SN/MN configured SPR configuration as MN/SN doesn’t know the UE has stored SN/MN configured SPR configuration.

Observation 4: MN and SN cannot configure SPR configuration to UE simultaneously in case of both MN-initiated CPC and SN-initiated CPC are configured to UE.

Observation 5: The SPR configuration could be useless as it is unpredictable for MN-initiated CPC or SN-initiated CPC is performed.

Observation 6: For legacy PSCell change, the PSCell change type indication information needs to be indicated to UE.

Observation 7: For CPC, no extra indication information is needed for UE to identify the CPC type as the UE can get to know the CPC type by implicit way.

Proposal 1: RAN2 to agree that UE stores two SPR configurations configured by MN and SN respectively.

Proposal 2: RAN2 to confirm that for legacy PSCell change, the PSCell change type indication information is introduced to inform UE the executed PSCell change type.

Proposal 3: C-RNTI is not included in inter-RAT SHR from NR to LTE.

Proposal 4: Time between report generating and fetching is included in R18 inter-RAT SHR from NR to LTE.

Proposal 5: RAN2 confirms to support inter-RAT SHR from LTE to NR for T304 trigger.

Proposal 6: RAN2 confirms the following assumptions which included in RAN1’s LS:

- Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand

- UE stores this SHR configuration in NR format

- If T304 trigger is met, UE records SHR in NR format

- UE reports this SHR only to an gNB (either the target gNB or another gNB)

Proposal 7: Source LTE cell identity and target NR cell identity are included in inter-RAT SHR from LTE to NR.

Proposal 8: For inter-RAT HO from LTE to NR, a new EUTRA source cell CGI is introduced in inter-RAT SHR.

Reference

- R2-2302401 Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece

- R2-2300002 Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France

- R3-232140 LS on intra-system inter-RAT SHR and SPR