3GPP TSG-RAN WG2 Meeting #122		R2-2305486

Incheon, Korea, 22nd– 26th May, 2023

Source:	CATT

Title:	RACH enhancement for SON

Agenda Item:	7.13.6

Document for:	Discussion and Decision

Introduction

For RACH enhancement, there is a summarized document [1] in last RAN2 meeting, and the following agreements are achieved:

Agreements: => 1 FFS: Include the actual number of msg3 repetitions in RA report. 2 FFS: Include NSAG priority in RA report. 3b FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1. 3c FFS: Include S-NSSAI(s) in RA report.

In this contribution, we discuss the rest issues for SgNB RACH report, and also the issues on RACH report for RACH partitioning.

Discussion

SgNB RACH report for MR-DC scenarios

In RAN2#121 meeting, SgNB RACH report for MR-DC scenarios has below agreement for the report structure:

1: To have “a list of SN RA report entries as a single NR container (i.e. NR RA-ReportList)”.

An LS [2] is sent to RAN3 to ask whether the UE should carry the last PSCell ID or a PSCell list to the network node together with the NR container. The reply LS has been received after the last RAN3 meeting [3] which mentioned two issues:

For the two alternatives of air-interface SN RACH report information:

- Alt 1: Includes unique PSCell identities, i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities

- Alt 2: Includes the last PSCell identity (in NR RA-ReportList)

RAN3 concluded that the preferred option is Alt 1.

And for the MR-DC RACH report scenario:

Furthermore, RAN3 noticed the following RAN2 agreement: => It is not supported in R18 that UE reports NR RACH Report to LTE cell when the UE is in standalone LTE. RAN3 is unclear why such a restriction was agreed by RAN2. If there are no additional RRC specification impacts, RAN3 see no reason to restrict the reporting of NR RA Report when UE is in standalone LTE mode. RAN3 therefore kindly asks RAN2 to clarify the above agreement.

Therefore for the first issue, RAN2 should follow RAN3’s preference to include all the involved NR PSCell IDs without duplication together with the NR container in the LTE RACH report. And then the LTE MN should transmit the NR container to the involved NR PSCell.

Proposal 1: UE includes all the involved NR PSCell IDs without duplication together with the NR container in the LTE RACH report.

For the second issue, RAN2 has agreed only the EN-DC and (NG)EN-DC can be considered for the MR-DC SN RACH information recording. And for the report scenario, we have some analysis in our document of [4]. In summary, we think the SN RACH information is only used to optimize the RACH parameters of a specific node, so the scenario of recording the SN RACH information has no relationship with the scenario of reporting. If the UE recorded the NR SN RACH information in EN-DC and (NG)EN-DC and then connected to an NR MN, the NR SN RACH information can also be reported since the format of  RACH information is exactly of the NR format.

Therefore we suggest re-discuss this scenario of report issues, and we propose to report SN NR RACH information to NR MN in NR format, if UE enters NR-DC, NE-DC or NR SA scenarios, and to the LTE/(e)LTE MN node for EN-DC/(NG)EN-DC/LTE SA scenarios.

Proposal 2: Report SN NR RACH information to NR MN in NR format, if UE enters NR-DC, NE-DC or NR SA scenarios, and to the LTE/(e)LTE MN node for EN-DC/(NG)EN-DC/LTE SA scenarios.

Based on the proposal above, a reply LS to RAN3 can be sent to inform the update scenarios agreed by RAN2.

Proposal 3: Send a reply LS to RAN3 to inform the updated RAN2 agreed SN NR RACH report scenarios.

Another issue needs to be considered is whether the NR RACH information should be separately reported from the legacy LTE RACH report. For legacy LTE RACH report, there is no available indicator since UE should always have the RACH info recorded in CONNECTED mode, the network just needs to request for it. We think the same mechanism can be directly reused for NR SN RACH container fetch. No available indicator needs to be introduced in LTE specification. If the network requests for the RACH information, the UE should send both the LTE and NR RACH information to the network if any.

Proposal 4: UE could reports both the LTE MN and the NR SN RACH information together to the MN node by network request without new available indicator for NR SN RACH report.

For NR-DC scenario, there is no additional UE capability introduced for the SgNB RACH report since the RACH information of MN and SN use the same encode format and share the same mechanism and the UE variable. But for EN-DC and NG-EN-DC scenarios, if the NR container is introduced, additional UE capability may be needed to identify whether the inter-RAT record and report of RACH information could be supported.

Proposal 5: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

RACH report about RACH partitioning information

In RAN2#120 meeting [2], some preliminary agreements were made on RACH report about RACH portioning information.

Agreements:

4	UE includes RA and SDT information in RA report when an SDT operation fails.

But there are some other issues which are needed to be further studied. In the following, we analyse SDT issues in RA report as well as some other issues related to RACH partitioning.

SDT

For SDT, it has been agreed to included RA and SDT information in RA report when an SDT operation fails. But it is it still unclear on how to associate SDT feature and information for failed SDT operation.

In Rel-17, onDemandSISuccess was introduced [3]. When on-demand SI request is successful, onDemandSISuccess is set to be true. When on-demand SI request fails, onDemandSISuccess is absent. And the network can figure out whether SI request procedure is successful or not by onDemandSISuccess together with raPurpose. For example, when raPurpose is set to requestForOtherSI or msg3RequestForOtherSI, and onDemandSISuccess is set to be true, it means this on-demand SI procedure is successful; when raPurpose is set to requestForOtherSI or msg3RequestForOtherSI, and onDemandSISuccess is absent, it means this on-demand SI procedure failed.

onDemandSISuccess This field is set to true when the RA report entry is included because of either msg1 based on demand SI request or msg3 based on demand SI request and if the on-demand SI request is successful. Otherwise, the field is absent.

Similarly, the same mechanism can be reused for SDT. I.e. defining a flag which indicates whether the SDT procedure is successful or not.

Proposal 6: Define a flag to indicate whether RA-SDT procedure is successful or not.

But there is no corresponding raPurpose associating with this flag.

In RAN2#119bis [4], it was agreed that as follows:

1	Agree to add the following parameters into RACH report for RACH partitioning:

-	Feature or the combination of features that triggered the RACH

-	Used feature combination

So, the feature or the feature combination that triggered the RACH can be used together with this new defined flag to indicate whether SDT is successful or not.

Or we define one new RA purpose for SDT which is similar to consistent LBT failures in the SpCell which was agreed in RAN2#119bis meeting. RAN2 can down select solutions in both options.

Proposal 7: RAN2 to down select one of the options to associate SDT and the new flag to indicate whether RA-SDT procedure is successful or not:

- Associating with feature or feature combination triggering the RACH and or used for RACH;

- Defining one new RA purpose for RA-SDT.

RACH resource configuration for RACH partitioning

In Rel-17 RACH partitioning, multiple RACH resources are configured for different RACH partitioning in order to enable early identification of the RACH feature or RACH feature combination on the network side. But it may be happened that the UE triggers one feature or feature combination, but there is no corresponding RACH resource. Based on current spec procedure, the UE will select another RACH resource to initiate RACH procedure based on the used feature priority. So, it is useful to report the RACH resources for the configured RACH feature/feature combination, i.e., the RACH resource configuration for RACH partitioning.

Proposal 8: The UE reports the RACH resource configuration for feature/feature combination for SON.

Field featurePriorities for RACH partitioning

In RACH partitioning procedure, the UE will select the RACH resources with a feature which has the highest priority among the features applicable to the RACH procedure. For example, when the SDT (Small data transmission), Msg3 repetition and NSAG (Network Slice AS Group) 1 are applicable to the RACH procedure, the UE will compare the featurePriorities of each feature. And assuming the featurePriorities for NSAG1 is the feature with highest priority, the UE will select the RACH resource corresponding to NSAG1 feature. Hence, it is useful to report the featurePriorities to the network to figure out why the mismatch happens between the feature/feature combination that triggered the RACH and the feature/feature combinations that used in the RACH (i.e. intended feature/feature combination).

Hence, we propose that:

Proposal 9: The UE indicates the featurePriorities information for feature /feature combination for SON.

Slicing

RAN2#121bie-e meeting => 2 FFS: Include NSAG priority in RA report. 3b FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1. 3c FFS: Include S-NSSAI(s) in RA report.

In RAN2#121bis-e meeting, as above, RAN2 discussed whether/what NSAG associated information to be reported in RA report. According to below agreement, for slicing, UE could report the used NSAG#n1 to the network, meanwhile, the UE could also report the network the other NSAG, such as NSAG#n2, which trigger the same access attempt and has higher priority than NSAG#n1, but this NSAG#n2 is not used because of the restriction in SIB1, this could help network understanding the RACH resource requiring for different NSAG(s) and make some optimization on the resource configuration.

RAN2#119bis Agreements: 1 Agree to add the following parameters into RACH report for RACH partitioning: - Feature or the combination of features that triggered the RACH - Used feature combination

Proposal 10: It is suggested that NSAG-ID(s) in RA report at least includes the applied NSAG-ID(s) and not applied NSAG-ID(s) which triggered the same random access procedure and has higher priority than applied NSAG-ID(s).

Conclusion

According to the analysis in section 2, we propose:

For SgNB RACH report for MR-DC

Proposal 1: UE includes all the involved NR PSCell IDs without duplication together with the NR container in the LTE RACH report.

Proposal 2: Report SN NR RACH information to NR MN in NR format, if UE enters NR-DC, NE-DC or NR SA scenarios, and to the LTE/(e)LTE MN node for EN-DC/(NG)EN-DC/LTE SA scenarios.

Proposal 3: Send a reply LS to RAN3 to inform the updated RAN2 agreed SN NR RACH report scenarios.

Proposal 4: UE could reports both the LTE MN and the NR SN RACH information together to the MN node by network request without new available indicator for NR SN RACH report.

Proposal 5: Additional capability may be needed for NR RACH Report enhancement in LTE for EN-DC and NG-EN-DC scenarios.

For RACH report about RACH partitioning

Proposal 6: Define a flag to indicate whether RA-SDT procedure is successful or not.

Proposal 7: RAN2 to down select one of the options to associate SDT and the new flag to indicate whether RA-SDT procedure is successful or not:

- Associating with feature or feature combination triggering the RACH and or used for RACH;

- Defining one new RA purpose for RA-SDT.

Proposal 8: The UE reports the RACH resource configuration for feature/feature combination for SON.

Proposal 9: The UE indicates the featurePriorities information for feature /feature combination for SON.

Proposal 10: It is suggested that NSAG-ID(s) in RA report at least includes the applied NSAG-ID(s) and not applied NSAG-ID(s) which triggered the same random access procedure and has higher priority than applied NSAG-ID(s).

Reference

- R2-2302069 Report of [AT121][803][R18 SONMDT] RACH enhancement (Huawei)

- R2-2302066, Reply LS on RACH enhancement for R18 SONMDT, RAN2

- R3-232144 Reply LS on RACH enhancement for R18 SONMDT RAN3

- R2-2300296 RACH enhancement for SON CATT