3GPP TSG-RAN WG2 Meeting #122		R2-2305488

Incheon, Korea, May 22-26, 2023

Source:	CATT

Title:	Discussion on Fast MCG recovery and MHI Enhancement

Agenda Item:	7.13.8

Document for:	Discussion and Decision

Introduction

The agreements about Fast MCG recovery MRO enhancement in RAN2#120 meeting are [1]:

Agreements

1	For fast MCG recovery MRO, prioritize NR-DC scenario. if time allows, study whether the same solution can be extended for others DC scenarios.

2	Consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316). The “upon fast MCG recovery case” is FFS.

3	RLF report is enhanced to support fast MCG recovery MRO.

4	Fast MCG recovery failure cause shall be included for fast MCG recovery optimization. FFS details

In RAN2#121 meeting, some scenarios are discussed but no convergence is achieved.

=> FFS the scenario to “SCG deactivation before fast MCG recovery (when UE detects MCG failure)”.

Meanwhile, RAN3 has sent an LS to RAN2 [2] about the UE report content of Fast MCG recovery Enhancement, which includes:

In the scope of MRO for the fast MCG recovery, RAN3 has agreed that it is beneficial if the UE reports at least: - PSCell where SCG failure happened, and - the cause of the fast MCG recovery failure containing at least: - T316 expiry, - SCG failure, and - SCG was deactivated or other cases where SCG is not available - SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx) if the cause of the fast MCG recovery is SCG failure

RAN2 has not treat the fast MCG recovery related issues in RAN2#121bis, but RAN3 has some further discussions in RAN3#119bis [3] and reached the agreements:

MRO for the fast MCG recovery: Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

From the RAN3 suggested UE report contents in the incoming LS and in the agreements of last RAN3 meeting, we consider the corresponding scenarios could be clearly decided in RAN2. Therefore in this contribution, first we want to continue clarify the possible scenarios, and then we intend to discuss the detailed enhanced content about Fast MCG recovery MRO.

In this contribution, besides the discussion of fast MCG recovery, we also propose the issue we observed caused by mobility history information for SCG and SCG deactivation/activation mechanism coexistence and give the initial analysis from our perspective.

Discussion

Fast MCG recovery

Scenarios clarification

Successful scenario

The fast MCG link recovery is introduced in R16. The specification mentioned that:

- If radio link failure occurred in MCG, first the MCG failure should be recorded in VarRLF-Report, and then UE should initiate the MCG failure information procedure if condition fulfilled;

- If MCG failure is sent to the network and if any of the response message of RRCRelease/ MobilityFromNRCommand/RRCReconfiguration with reconfigurationwithSync for the PCell is received before T316 expired, the VarRLF-Report will be cleared.

Observation 1: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

The UE action of clearing the VarRLF-Report upon successful fast MCG recovery is agreed in R16. The failure scenarios can have emergency problem which needs to be solved first and foremost. Furthermore, in RAN3 LS [2] it is only mentioned the failure information by UE reporting. Therefore, we consider the enhancement for failure scenarios of fast MCG recovery should be discussed firstly in R18.

Proposal 1: Scenarios about fast MCG recovery failure are prioritized in R18.

Thus, we analyze the failure cases and corresponding content need to report in the subsequent discussion. Besides the T316 expires and the SCG failure during fast MCG recovery, we think the already agreed SCG deactivation scenario needs to be clarified first.

Already agreed SCG deactivation scenario

In RAN2 one of the agreed scenarios is “SCG failure/deactivation during fast MCG recovery (i.e., running of T316)”. It can be mapped to the RAN3 agreed ‘Case a’ in RAN3#117e [4]:

RAN3#117-e Agreement SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure)

But in RAN3#119bis meeting, it is clear that the SCG deactivation scenario has been removed for this Case a:

RAN3#119bis Agreement MRO for the fast MCG recovery: Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

And in the incoming RAN3 LS [2], besides the T316 expiry and SCG failure, there is no SCG deactivation during fast MCG recovery case, another cause of the fast MCG recovery failure is “SCG was deactivated or other cases where SCG is not available” which means the SCG is deactivated before MCG failure. From the agreements above we can deduce that “SCG deactivation during fast MCG recovery” case has been removed in RAN3, but “SCG deactivation before fast MCG recovery” is one of the agreed cases in RAN3.

Observation 2: “SCG deactivation before fast MCG recovery” scenario is corresponding to part of the RAN3 agreed case f1 of the fast MCG recovery failure:

- “Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed.”

From RAN2 technical point of view, for SCG deactivation case, it is the MN’s responsibility to send the SCG deactivation configuration, and the SCG deactivation cannot be configured to the UE if the MCG link is broken:

- If the MCG failure occurs, the UE shall suspend all RBs except SRB0 and reset MCG MAC, therefore any transmissions in UL and DL cannot be performed based on the descriptions in TS38.331:

5.7.3b MCG failure information [Omitted 5.7.3b.1] 5.7.3b.2 Initiation A UE configured with split SRB1 or SRB3 initiates the procedure to report MCG failures when neither MCG nor SCG transmission is suspended, the SCG is not deactivated, t316 is configured, and when the following condition is met: 1> upon detecting radio link failure of the MCG, in accordance with 5.3.10.3, while T316 is not running. Upon initiating the procedure, the UE shall: 1> stop timer T310 for the PCell, if running; 1> stop timer T312 for the PCell, if running; 1> suspend MCG transmission for all SRBs, DRBs, multicast MRBs, except SRB0, and, if any, BH RLC channels; 1> reset MCG MAC; [Omitted part]

- It is also not allowed to use split SRB1 to transmit the SCG deactivation command due to the descriptions in TS37.340:

7.13 Activation and Deactivation of SCG To enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured, an activation/deactivation mechanism of SCG is supported. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated. Upon SCG deactivation and while the SCG is deactivated, the network ensures that there is no uplink control PDU transmission to the deactivated SCG (e.g. the network releases statusReportRequired from PDCP entities of SCG bearers if configured, the network does not perform QoS flow remapping from a DRB associated to the deactivated SCG to another DRB). The network ensures the SCG is activated while PDCP duplication is activated for SCG RLC entities associated with a PDCP entity. NOTE: Upon SCG (de)activation, it is up to the network to ensure there is no pending SDUs or PDUs in SCG RLC entity (e.g. instructs the UE to perform PDCP data recovery and RLC re-establishment/release, if needed). [Omitted part]

The timer T316 can only be started if the UE send the MCGFailureInformation message, so “SCG deactivation during fast MCG recovery (i.e., running of T316)” is not a valid scenario based on current spec.

Observation 3: “SCG deactivation during fast MCG recovery (i.e., running of T316)” scenario has been removed from the RAN3 suggest cases, and it is also not a valid scenario from technical point of view in RAN2.

Therefore the SCG deactivation can only be configured before the fast MCG recovery, not during the fast MCG recovery. For the SCG deactivation before the fast MCG recovery, if the SCG has already been configured of deactivation, the UE may have chance suffering RLF in MCG, and then to perform RRC re-establishment. Since the MCG failure content recorded by the UE cannot be sent to the network because of the deactivated SCG, it can be considered as a possible cause which results in the fast MCG recovery failure.

Based on all the analysis above, it is suggest including the SCG deactivation before fast MCG recovery scenario instead of SCG deactivation during fast MCG recovery.

Proposal 2: RAN2 to clarify the SCG deactivation cannot be performed during fast MCG recovery, and change corresponding scenario to “SCG deactivation before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 agreement.

PSCell change /addition ongoing scenario

The MCG failure may occur when PSCell change/addition ongoing. For PSCell change/addition the UE may receive the RRCReconfiguration message, and then to apply the corresponding action. Since the addition or change of the PSCell need some processing delay time, it is possible that the UE needs to send the MCGFailureInformation message but the SCG link has not been prepared completely. This scenario can be considered as the same type as the “SCG deactivation before fast MCG recovery” scenario, and it is corresponding to the case of “other cases where SCG is not available” in RAN3 LS [2].

Observation 4: “PSCell change/addition ongoing before fast MCG recovery” scenario is corresponding to “other cases where SCG is not available” cause of the fast MCG recovery failure in RAN3 LS.

If the UE receives the PSCell change after T316 is running, the UE may still not receive the HO command for recovery. But in this case, the T316 will run to expire, so it is a sub-case for “T316 expire” scenario.

Proposal 3: Add the scenario “PSCell change /addition ongoing before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 LS.

SCG failure before MCG failure scenario

Both the scenarios of SCG deactivation, PSCell change /addition ongoing mentioned above are normal SCG related procedure without failure. For these scenarios, even the MCG failure information cannot be reported to the network, almost no optimization can be performed for MRO purpose. But if the MCG recovery failure is caused by SCG failure before MCG failure, the MCG recovery failure maybe avoid by some kind of mobility robustness optimization. Therefore the SCG failure case should be distinguished from the other cases without SCG failure. In RAN3 LS [2], “SCG failure” is one of the causes of the fast MCG recovery failure which can be mapped to both scenarios of SCG failure before/during fast MCG recovery. And it is part of the RAN3 agreed case f1 of the fast MCG recovery failure.

Proposal 4: Add the scenario “SCG failure before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 agreement.

HO command for recovery failure after the MCG failure scenario

In addition, the network may send a HO command after receiving the MCGFailureInformation message for recovery. But if the UE cannot successfully apply the received HO command, the fast MCG recovery failure may also occur. Since the UE can only store one entry of RLF report content, the HOF information will override the previous MCG failure information. To keep the information integrity for network side analysis, the consecutive failures are necessary to be recorded by the UE for the MRO usage.

Proposal 5: Add the consecutive failure scenario of “HO command for recovery failure after MCG RLF”.

Detailed reported content for fast MCG recovery MRO

For current agreed scenarios at last meeting and the suggested scenarios which need to be align with RAN3 LS [2] and agreements [3] in above section 2.1, we think the UE recorded content can be discussed for 1) only report MCG failure and for 2) report both MCG and SCG failure. The figure below illustrates the classification of different scenarios:

Figure 1 Classification of different scenarios and the MCG/SCG failure information

1) Report only MCG failure

For all “Report only MCG failure” cases, there is no SCG failure related information recorded in the UE, so the legacy MCG RLF/HOF content could cover most of the failure information. Only MCG failure includes possible cases as below:

- T316 expires;

For this case, the UE could store the VarRLF-Report based on HOF or RLF, and has already sent the MCGFailureInformation message. Based on current specification, if no response message such as RRCRelease/MobilityFromNRCommand/RRCReconfiguration with reconfigurationwithSync for the PCell is received before T316 expired, the VarRLF-Report will be maintained, and the UE will initiate the re-establishment procedure. So to distinguish the legacy RLF/HOF report and the fast MCG recovery failure report, some information should be added into the VarRLF-Report, e.g. running time of T316;

- SCG cannot be used, i.e. SCG deactivated, or PSCell addition/change is ongoing when UE detects MCG failure;

For this case, the SCG is not failed but cannot be used due to some other causes. UE will not start the “Fast MCG recovery procedure”, and directly perform the re-establishment procedure. If T316 value has been configured to the UE, it is the network’s opinion to recover the MCG link via SCG. Therefore it is better to report to the network the MCG failure with the cause that why the SCG link is unusable.

Proposal 6: UE report only MCG failure related content to the network for the scenarios below:

- T316 expires;

- SCG cannot be used without failure, i.e. SCG deactivated, or PSCell addition/change is ongoing.

Proposal 7: For only MCG failure, record the possible content to distinguish with the legacy RLF/HOF in MCG, e.g. to record and report the running time of T316, the SCG unusable cause and the PSCell ID if any.

- In addition, the HO command for recovery failure after the MCG RLF scenario.

For this case, the UE has successfully sent the MCGFailureInformation message, and the network also sends the HO command to the UE. If the UE cannot successfully apply the received HO command, the fast MCG recovery failure may also occur. Since the UE can only store one entry of RLF report content, the HOF information will override the previous MCG failure information. To keep the information integrity for network side analysis, the consecutive failures are necessary to be recorded by the UE for the MRO usage. Based on current MCG RLF report information, at least the cell ID in the HO command should be recorded and reported together with the legacy MCG RLF.

Proposal 8: Record the consecutive failure information for “HO command for recovery failure after MCG RLF” scenario.

2) Report both MCG and SCG failures

Report both MCG and SCG failure includes 2 cases as below:

- SCG failure during fast MCG recovery (i.e., running of T316). The MCG link cannot be successfully recovered since SCG is unusable, even if the MCGFailureInformation message has been successfully sent to the network. The already recorded MCG failure related information needs to be report to the network, maybe binding with the failed SCG information for further MRO usage;

- SCG failure before MCG failure. This scenario includes the cases that MCG failure occurs after e.g. SCG RLF, PSCell change failure. In this case, the already recorded SCG failure related information needs to be report to the network, maybe binding with the failed MCG information.

For both cases, to distinguish the scenarios of whether the SCG failure is occurred before/during fast MCG recovery, it is proposed also to include other information besides the MCG/SCG failure information itself to indicate the relationship between the two failures, e.g. which leg failure first indicator, the time elapsed from one failure to the other failure.

Proposal 9: UE report both MCG and SCG failure related content to the network for the scenarios below:

- SCG failure during fast MCG recovery (i.e., running of T316);

- SCG failure before MCG failure.

Proposal 10: For both MCG and SCG failures, record and report the PSCell ID, SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Proposal 11: For both MCG and SCG failures, record the relationship between the two failures in RLF report for both MCG and SCG failure case, e.g. indicate which leg failed first, and the time elapsed between the two failures.

MHI Enhancement for SCG Deactivation/Activation

Mobility history report which includes the mobility history information of maximum of 16 most recently visited primary cells and/or time spent is introduced in release 16, and in release 17, in case of dual connectivity, the mobility history information of recently visited primary secondary cell group cells and/or time spent is introduced and nested in PCell mobility history information.

In R17, SCG activation/deactivation mechanism is introduced to enable reasonable UE battery consumption while having fast usage of SCG when (NG) EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

Observed issue of SCG MHI and SCG deactivation/ activation coexistence

In 38.331 spec, upon addtion/change/release of a PSCell, the UE logs the mobility history information for SCG in a new entry while being connected to the current PCell. These entries generated and stored for SCG while being connected to the current PCell will be included in the current PCell mobility history information when PCell change occurs. In details, the UE logs the time spent with no PSCell since entering the current PCell or since last PSCell release since entering the current PCell in RRC_CONNECTED upon addition of a PSCell, and logs global cell identity or physical cell identity and carrier frequency of that cell and the time spent in the previous PSCell while being connected to the current PCell upon change/release of a PSCell (highlighted yellow below).

Excerpt from 38.331-h40

5.7.9.2 Initiation 1> If the UE supports PSCell mobility history information and upon addition of a PSCell: <…skip…> 2> for the included entry: 3> set the field timeSpent of the entry according to following: 4> if this is the first PSCell entry for the current PCell since entering the current PCell in RRC_CONNECTED: 5> include the entry as the time spent with no PSCell since entering the current PCell in RRC_CONNECTED; 4> else: 5> include the time spent with no PSCell since last PSCell release since entering the current PCell in RRC_CONNECTED; 1> If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell: <…skip…> 2> for the included entry: 3> if the global cell identity of the previous PSCell is available: 4> include the global cell identity of that cell in the field visitedCellId of the entry; 3> else: 4> include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry; 3> set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell;

In current SCG deactivation/activation mechanism, at PSCell addition/change, in case the SCG state is configured as deactivated, the UE does not perform random access. If the network wants the UE to perform random access, it can indicate the SCG as activated and deactivate it after the random access by RRC. It means that if the UE receives PSCell addition/change command with SCG state is deactivated, the UE will not perform random access to target PSCell until receiving SCG activation command. However, in this case, the UE will start synchronising to the DL of the target PSCell upon apply the configuration of PSCell addition/change.

Excerpt from 38.331-h40

5.3.5.3 Reception of an RRCReconfiguration by the UE <…skip…> 1> if the RRCReconfiguration includes the secondaryCellGroup: 2> perform the cell group configuration for the SCG according to 5.3.5.5; <…skip…> 2> if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message: 3> perform SCG activation as specified in 5.3.5.13a; 3> if reconfigurationWithSync was included in spCellConfig in nr-SCG: 4> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 3> else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message: 4> if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or 4> if lower layers indicate that a Random Access procedure is needed for SCG activation: 5> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 4> else the procedure ends; 3> else the procedure ends; 2> else 3> perform SCG deactivation as specified in 5.3.5.13b; 3> the procedure ends;

Excerpt from 38.331-h40

5.3.5.5.2 Reconfiguration with sync <…skip…> 1> else (sl-PathSwitchConfig is not included): 2> if this procedure is executed for the MCG or if this procedure is executed for an SCG not indicated as deactivated in the E-UTRA or NR RRC message in which the RRCReconfiguration message is embedded: 3> start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync; 2> if the frequencyInfoDL is included: 3> consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId; 2> else: 3> consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId; 2> start synchronising to the DL of the target SpCell; 2> apply the specified BCCH configuration defined in 9.1.1.1 for the target SpCell; 2> acquire the MIB of the target SpCell, which is scheduled as specified in TS 38.213 [13]; <…skip…>

Observation 5: Upon UE receives PSCell addition/change command with SCG state is deactivated, the UE will not perform random access to target PSCell until SCG activation, but will start synchronizing to the DL of the target PSCell.

Based on Observation 5, upon UE receives SCG addition/change command with SCG state is deactivated, the UE can consider it has accessed to the target PSCell as the UE starts synchronizing to the DL of the target PSCell or the UE can consider it has not accessed to the target PSCell as the UE will not perform random access to the target PSCell, which will affect the SCG mobility history information logging.

Furthermore, when the UE receives SCG deactivation command while connected to current PSCell, if the UE is not configured to perform RLM and BFD for SCG in SCG deactivation state, there is a high possibility of DL out of sync, and there is also a high possibility of UL out of sync due to TAT will not be maintained while SCG deactivation. If the UE doesn’t consider SCG deactivation and logs SCG mobility history information as legacy (i.e. the UE logs the current PSCell and time spent from accessing to the current PSCell to receive PSCell change or PSCell release), this may make the network mistakenly think that the UE has DL/UL coverage always in the PSCell.

Observation 6: SCG deactivation/activation mechanism may have impact on the SCG mobility history information.

Initial analysis of the impact of SCG deactivation/activation on SCG MHI

Based on the discussion above, we give the initial analysis from our perspective below case by case.

PSCell addition with SCG is deactivated

If both SCG MHI and SCG deactivation/activation are supported for UE, considering the case that the UE being connected to a PCell in standalone, and receives a PSCell addition command to add a SCG with scg-state is configured (i.e. SCG is deactivated), the UE will perform DL sync to the target PSCell, but do not perform random access to the target PSCell. In this case, how the UE to log the mobility history information for SCG should be considered. E.g., as shown in the Figure 2 below.

Figure 1: PSCell addition with SCG is deactivated

In Figure 2 above, the UE accesses to a PCell or releases SCG at T1, receiving the PSCell addition command with scg-state is configured (i.e. SCG is deactivated) at T2, receiving SCG activation command at T3, and receiving PSCell change command with scg-state is not configured (i.e. SCG is activated) or SCG release command at T4, there are the following methods which can be considered to log mobility history information for SCG for the procedure:

Method 1: 2 entries: (T3-T1), (PSCell 1 or PSCell 2, T4-T3)

Method 2: 3 entries: (T2-T1), (PSCell 1, T3-T2), (PSCell 1or PSCell 2, T4-T3)

For method1, when UE receives PSCell addition command with scg-state is configured at T2, the UE believes there is still no PSCell as no random access to PSCell 1 to be performed at T2, then UE receives SCG activation command at T3, there are two cases, one is the activated target PSCell is PSCell 1 (i.e. previous deactivated PSCell), another is the activated target PSCell is PSCell 2 (i.e. a new PSCell), the UE will log time spent (T3-T1) in a new entry. At T4, the UE receives PSCell change command with scg-state is not configured or SCG release command, the UE will log PSCell 1or PSCell 2 and time spent (T4-T3) in a new entry.

For method 2, when UE receives PSCell addition command with scg-state is configured at T2, the UE believes that the UE is connected to PSCell 1 as UE will perform DL sync to PSCell 1 at T2, the UE logs time spent (T2-T1) in a new entry. At T3, UE receives SCG activation command, similar as method 1, the activated PSCell can be PSCell 1 or PSCell 2, the UE logs PSCell 1 and time spent (T3-T2) in a new entry. At T4, the UE receives PSCell change command with scg-state is not configured or SCG release command, the UE will log PSCell 1or PSCell 2 and time spent (T4-T3) in a new entry.

PSCell change with SCG is deactivated

Similar as PSCell addition with SCG is deactivated, if UE receives PSCell change command with SCG is deactivated, the UE also performs DL sync to the target PSCell and not perform random access to the target PSCell. The UE has no idea how to log the mobility history information for SCG. E.g., as shown in the Figure 3 below.

Figure 3 PSCell change with SCG is deactivated

In Figure 3 above, UE has activated PSCell 1 at T1, UE receives PSCell change command with scg-state is configured (i.e. SCG is deactivated) at T2, receiving SCG activation command at T3, and receiving PSCell change command with scg-state is not configured (i.e. SCG is activated) or SCG release command at T4, there are the following methods which can be considered to log mobility history information for SCG for the procedure, which is similar as PSCell addition with SCG is deactivated case.

Method 1: 2 entries: (PSCell 1, T3-T1), (PSCell 2 or PSCell 3, T4-T3)

Method 2: 3 entries: (PSCell 1, T2-T1), (PSCell 2, T3-T2), (PSCell 2 or PSCell 3, T4-T3)

If UE has deactivated PSCell at T1, then, for method 1, the UE will log previous latest activated PSCell and the time spent from accessing the previous latest activated PSCell to T3 if previous latest activated PSCell is existing or the UE will log the time spent from accessing the PCell/releasing SCG to T3. For method2, it is same as UE has activated PSCell 1 at T1 case.

SCG is deactivated while being connected to a PSCell

If UE has activated PSCell and receives the SCG deactivation command, the UE will deactivate the SCG. Then, the UE receives the SCG activation command to activate SCG if needed, there are two cases, one is the UE performs random access to activate SCG, another is the UE perform rach-less to activate SCG. In the two cases, how the UE logs the mobility history information for SCG needs to be discussed. E.g., as shown in the Figure 4 below.

Figure 4: SCG is deactivated while being connected to a PSCell

In Figure 4 above, the UE has activated PSCell 1 at T1, receiving SCG deactivation command at T2, receiving SCG activation command at T3, and receiving PSCell change command with scg-state is not configured (i.e. SCG is activated) at T4. There are the following methods which can be considered to log mobility history information for SCG for the procedure.

Method 1: 1 entry: (PSCell 1, T4-T1)

Method 2: 2 entries: (PSCell 1, T3-T1), (PSCell 2, T4-T3)

Method 3: 3 entries: (PSCell 1, T2-T1), (T3-T2), (PSCell 1 or PSCell 2, T4-T3)

For method1, the UE receives SCG deactivation command at T2, receiving SCG activation command at T3 and UE performs rach-less SCG activation, i.e. the UE still is connected to PSCell 1 until UE receives PSCell change command with scg-state is not configured at T4, the UE accesses to PSCell 3. UE considers it is connected to PSCell 1 from T1 to T4, therefore, UE logs PSCell 1 and time spent (T4-T1) in a new entry.

Based on method 1, if UE receives SCG activation command at T3 and UE performs random access to PSCell 2, the UE considers it is connected to PSCell 1 from T1 to T3 and is connected to PSCell 2 from T4 to T3. Then, the UE logs PSCell 1 and (T3-T1) in a new entry, logs PSCell 2 and (T4-T3) in another new entry (i.e. method 2).

If UE considers it is not connected to PSCell 1 at T2 (i.e. receiving SCG deactivation command), then, the interval of SCG deactivation should be logged separately. In this case, three entries should be logged in UE, i.e. (PSCell 1, T2-T1), (T3-T2), (PSCell 1 or PSCell 2, T4-T3).

Based on the analysis above, it can be seen that SCG deactivation/activation will affect the SCG mobility history information logging; RAN2 should consider MHI enhancement for SCG deactivation/activation.

Proposal 12: RAN2 considers MHI enhancement for SCG deactivation/activation.

Conclusion

According to the analysis in section 2, we propose:

Scenarios clarification for fast MCG recovery MRO:

Observation 1: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

Proposal 1: Scenarios about fast MCG recovery failure are prioritized in R18.

Observation 2: “SCG deactivation before fast MCG recovery” scenario is corresponding to part of the RAN3 agreed case f1 of the fast MCG recovery failure:

- “Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed.”

Observation 3: “SCG deactivation during fast MCG recovery (i.e., running of T316)” scenario has been removed from the RAN3 suggest cases, and it is also not a valid scenario from technical point of view in RAN2.

Proposal 2: RAN2 to clarify the SCG deactivation cannot be performed during fast MCG recovery, and change corresponding scenario to “SCG deactivation before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 agreement.

Observation 4: “PSCell change/addition ongoing before fast MCG recovery” scenario is corresponding to “other cases where SCG is not available” cause of the fast MCG recovery failure in RAN3 LS.

Proposal 3: Add the scenario “PSCell change /addition ongoing before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 LS.

Proposal 4: Add the scenario “SCG failure before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 agreement.

Proposal 5: Add the consecutive failure scenario of “HO command for recovery failure after MCG RLF”.

Detailed reported content for fast MCG recovery MRO:

Proposal 6: UE report only MCG failure related content to the network for the scenarios below:

- T316 expires;

- SCG cannot be used without failure, i.e. SCG deactivated, or PSCell addition/change is ongoing.

Proposal 7: For only MCG failure, record the possible content to distinguish with the legacy RLF/HOF in MCG, e.g. to record and report the running time of T316, the SCG unusable cause and the PSCell ID if any.

Proposal 8: Record the consecutive failure information for “HO command for recovery failure after MCG RLF” scenario.

Proposal 9: UE report both MCG and SCG failure related content to the network for the scenarios below:

- SCG failure during fast MCG recovery (i.e., running of T316);

- SCG failure before MCG failure.

Proposal 10: For both MCG and SCG failures, record and report the PSCell ID, SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Proposal 11: For both MCG and SCG failures, record the relationship between the two failures in RLF report for both MCG and SCG failure case, e.g. indicate which leg failed first, and the time elapsed between the two failures.

SCG deactivation/activation on SCG MHI:

Observation 5: Upon UE receives PSCell addition/change command with SCG state is deactivated, the UE will not perform random access to target PSCell until SCG activation, but will start synchronizing to the DL of the target PSCell.

Observation 6: SCG deactivation/activation mechanism may have impact on the SCG mobility history information.

Proposal 12: RAN2 considers MHI enhancement for SCG deactivation/activation.

Reference

- Draft_R2_120_meeting_report_v1.docx

- R3-230992 LS on MRO for CPC and CPA and fast MCG recovery RAN3

- RAN3_119bis-e_agenda_20230426_EOM.doc

- RAN3_117-e_agenda_20220825_EOM.doc