3GPP TSG RAN WG2 Meeting #122                                                         R2-2305494

# Incheon, Korea, May 22nd-26th, 2023

Agenda item:	7.5.3

Source:	Intel Corporation

Title:	C-DRX Enhancements for XR Traffic

Document for:	Discussion and decision

# Introduction

This document provides further analysis details on the down-selected options to enhance C-DRX support of XR frame rates corresponding to non-integer and non-uniform periodicities.

# Discussion

Before discussing the solution details, it might be good to clarify on our understanding that network would inform the UE when DRX enhancement are to be enabled in order to address issue associated to XR traffic with frame rates corresponding to non-integer and non-uniform periodicities.

C-DRX enhancements discussed here target traffic with non-integer periodicities aiming to address the issues associated to the misalignment of DRX/XR periodicities and the SFN wrap around and UE would only use them when configured by the network.

## Misalingment of DRX and XR traffic periodicites

The solution space of multiple active DRX configurations is not considered as captured in the following agreement:

- RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

From RAN2#121bis-e discussion, there was a common understanding that following two solutions space still could be considered, e.g. as explained in [1]-[3]:

- Solution space 1) Rational value of DRX cycles

- Solution space 2) Integer DRX cycle with a draft adjustment on the start offset of the ON duration and/or the DRX cycle length

### (Sol.1) Rational value of DRX cycles

The solution space (1) of rational (i.e., non-integer) value of the DRX cycle aims to better align with XR traffic periodicities with rational values (e.g. 50/3 msec for 60 frames per sec, 100/9 msec for 90 frames per sec, etc). These new non-integer values would be added for both short and long DRX cycles. To support such non-integer periodicities (rational numbers), however, the legacy DRX formulas need to be updated, e.g., to include the floor operation. It is also explained in some of above referenced TDoc how the direct application of modulo operation on rational numbers can result in rounding errors and that there exists methods to implement the operation without any rounding errors, for example, A modulo B can be implemented as A – [ B × floor (A / B)]

#### (Sol.1.1) Rational value of DRX cycles

An equivalent equation for DRX formula is creating by using the floor operation. An example of the corresponding TP is shown below [2]:

***** Example TP to 38.321 - START *****

1>	if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)):

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset:

NOTE:  If drx-ShortCycle or drx-LongCycle is not an integer, the modulo operation should be implemented by a method that does not introduce rounding errors. For example, (drx-StartOffset) modulo (drx-ShortCycle) can be implemented as drx-StartOffset – drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle).

***** Example TP to 38.321 - END *****

#### (Sol.1.2) Rational value of DRX cycles in cadence (fps)

New DRX parameter is defined to indicate the frames per second, fps (or cadence) rather than the periodicity to calculate the start offset of the DRX.  Example of the corresponding TP is shown below [3]:

***** Example TP to 38.321 - START *****

floor{M – floor(M/1000*drx_fps) * 1000/drx_fps}= drx-StartOffset

where M = (SFN × 10) + subframe number, and drx_fps is configured according to the FPS of XR traffic by network.

***** Example TP to 38.321 - END *****

### (Sol.2) Integer DRX cycle with ‘some’ adjustment

The solution space (2) of integer DRX cycle with ‘some’ adjustment aims for UE to compensate the accumulated drift due to the misalignment of XR and DRX periodicities by adjusting the value (or configuration) used for the DRX start offset and/or for the DRX cycle.

#### (Sol.2.1) DRX cycle adjustment

For this solution space (2.1), two levels of DRX cycle are supported. For example, assume XR frame rate with 60 FPS, configuring an outer DRX cycle length with 50ms, a DRX start offset and 3 inner cycles {17ms,17ms, 16ms} within the outer DRX cycle [5]. This option could imply support of multiple values for the DRX cycle associated with one DRX configuration [6]or single DRX cycle with multiple ON durations [7].

In [8] , the concept of “Super cycle + Short cycle” is used to limit the calculation of OnDurationTimer starting time within new cycle (e.g., SuperCycle), as shown in below figure. A similar kind of operation is also referred to as multiple DRX sub cycles within a DRX cycle in [5] or as multiple DRX cycles used in a round in [9].

***** Example TP to 38.321 - START *****

1>	if the Short DRX cycle is used for a DRX group, and{ [(SFN × 10) + subframe number] modulo (SuperCycle) }modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle):

2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

***** Example TP to 38.321 - END *****

Another example of the corresponding TP is shown below [5]:

***** Example TP to 38.331 - START *****

DRX-ConfigExt-v1800 ::=					SEQUENXE {

drx-SubCycleList-r18          Drx-SubCycleList-r18				OPTIONAL	-- Need R

}

Drx-SubCycleList-r18	::=	  SEQUENCE (SIZE (1..maxNrofDrxSubCycle)) OF Drx-SubCycle-r18

Drx-SubCycle-r18	::= INTEGER (0..34)

…

drx-SubCycleList

Indicates a list of one or more drx-SubCycle(s) in order within the Short DRX cycle or Long DRX cycle. The value of the drx-SubCycle in ms.

***** Example TP to 38.331 - END *****

***** Example TP to 38.321 - START *****

drx-SubCycleList(optional): the multiple DRX sub cycles in order within the Short DRX cycle or Long DRX cycle;

…

1>	if the Short DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

- if the drx-SubCycleList and the Short DRX cycle is used for a DRX group, except the first DRX sub cycle of the first Short DRX cycle, the beginning subframe of each DRX sub cycle follows the end of the previous DRX Sub cycle:

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

***** Example TP to 38.321 - END *****

#### (Sol.2.2) Start offset adjustment

For this solution space (2.2), a single DRX cycle is associated with multiple/different start offsets to allow that DRX configuration can have non-uniform DRX start offsets across each DRX cycle, i.e. UE is configured with an integer DRX cycle which is closest to the periodicity of XR traffic, and a pattern of DRX start offsets which matches/aligns the start time of ON durations with the start of each XR data burst, for example: drx-ShortCycle = 16 ms with drx-StartOffset = [0ms, 16ms, 17ms], resulting in a non-uniform CDRX cycle pattern of {16ms, 16ms, 17ms}.

Example of the corresponding TP is  shown below [10]:

***  Example TP to 38.331 - START ***

DRX-ConfigExt-v1800 ::=					SEQUENCE {

drx-StartOffsetList-r18     drx-StartOffsetList-r18				OPTIONAL	-- Need R

}

drx-StartOffsetList-r18 ::=

SEQUENCE (SIZE (1..maxNrofDrxStartOffset)) OF drx-StartOffset

…

drx-StartOffsetList

Indicates a list of one or more drx-StartOffset(s). If this field is present, the field drx-StartOffset and the DRX start offset configured in drx-LongCycleStartOffset shall be ignored.

**** Example TP to 38.331 - END ***

**** Example TP to 38.321 - START ***

drx-StartOffsetList (optional): the multiple drx-StartOffsets which define the subframes where the multiple DRX cycles start;

…

For each drx-StartOffset configured within the list drx-StartOffsetList (if configured) or configured by drx-SrartOffset or drx-LongCycleStartOffset:

1>	if the Short DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle):

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

- if the Long DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

…

*** Example TP to 38.321 - END ***

Example with a new time offset is used to shift the start offset of the DRX after a given number of DRX cycles [11]:

***** Example TP to 38.321 - START *****

    - traffic_time_offset: indicates a fixed time shift for the start of drx-onDurationTimer;

    - drx_offset: a number of cycles after which the new shift traffic_time_offset should be added

…

1> if the Long DRX cycle is used for a DRX group, and [SFN × 10 + subframe number] modulo (drx-LongCycle) = (drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle)

***** Example TP to 38.321 - START *****

### Forseen impacts of possible solution spaces

The following table summarizes whether the option of solutions can address by itself the mismatch issue between DRX and XR traffic periodicities, and the foreseen impacts to the configuration and the DRX calculation.

| Solution spaces | Impact to configuration | Impact to DRX calculation |
| --- | --- | --- |
| (1.1) Rational value of DRX cycle (ms) | - New DRX cycle parameter with non-integer values | - Update formula to use floor operation when/how applicable |
| (1.2) Rational value using cadence (fps) | - New DRX parameter with cadence periodicity based on fps | - New formula based on cadence in fps |
| (2.1) DRX cycle adjustment | - New value(s) for the DRX cycle - New frequency when to use a different DRX cycle value | - Update required to select applicable value of the DRX cycle |
| (2.2) DRX start offset adjustment | - New value(s) for the start offset associated with a DRX config - New frequency when to use a different start offset value | - Update required to select applicable value(s) of the start offset |

All options above are feasible with some implications, considerations and with different levels of specification impact. Solutions (2.1) and (2.2) require the network to predict the exact change of the DRX configuration in advance based on the XR traffic behaviour. Solutions (1.1) and (1.2) suggest using the actual XR traffic cadence/periodicity which might be more generic approach than (2.1) and (2.2) but it would require to specify a new definition of the DRX formula (in relation to the XR traffic pattern) and subsequent new un-expected issues that may come out of them (e.g., synchronization or rounding). Considering these points and companies’ support, we understand that RAN2 should try to further down-select. In our understanding solution (2.1) seems to provide enough network flexibility to accommodate the DRX to XR traffic periodic behaviour without having to change the way DRX is currently defined. Moreover, this solution (2.1) allows keeping the principle that DRX is configured independently of one specific service.

While all proposed solution spaces can address the issue on the mismatch between DRX and XR periodicities seems technically feasible, the open question is the implication, applicability and corresponding specification impacts foreseen.

- To address the mismatch between DRX and XR periodicities, it is proposed to adopt solution space (2.2) “DRX start offset adjustment” for which network pre-configures multiple DRX start offset values (i.e. a list of values) to be used.

## SFN wrap around

RAN2 agreed on the following approach to address the SFN wraparound, when DRX cycle is not a divisor of 10.24 sec. This could happen under two scenarios (a) after SFN wrap around due to the mismatch between the XR traffic arrival time and the ON duration of the DRX and scenario (v) when UE is reconfigured close to the boundary of the Hyper-SFN which could result in ON-duration timing mismatch between UE and network.

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

### Counter which increments at every SFN wrap around

The following options were proposed to enable the counter approach which increments at every SFN wrap around:

- Define a new counter. Example of the corresponding TP is shown below [11]:

***** Example TP to 38.321 - START *****

1> if SFN=0,

2> m = m+1

1> if the Long DRX cycle is used for a DRX group, and [(1024×m + SFN) × 10 + subframe number] modulo (drx-LongCycle) = drx-StartOffset

***** Example TP to 38.321 - END *****

- Adding (2.a) new “E-SFN” (with max value of 9 is defined and incorporated on DRX formulas which meets that  0 ≤ [(E-SFN × 10240) + (SFN × 10) + subframe number] < 102400) [12][13] or (2.b) H-SFN [2] when calculating the start offset of the DRX.

Figure on E-SFN wraparound with the updated C-DRX formulas  (E-SFN with max value 9) [13]

- Define a new state variable to replace/extend SFN in a UE, which increments/updates when number of SFN reaches a configurable/specific value [14][8]. For example, SFN in the current DRX formula is replaced by a new state variable, SFN_M which is updated by SFN_M = (SFN_M + 1) mod M whenever SFN changes [14]. M is typically configured with a value of 1,000 for XR applications. If SFN_M is configured, the formula for the start time of DRX on duration uses SFN_M instead of SFN. Since SFN_M and subframe number have the range of 0~999 frames and 0~9 subframes respectively, the new DRX reference time of [(SFN_M × 10) + subframe number] is repeated every 10,000ms. That ensures boundaries at which SFN_M wraps around would be always aligned with boundaries of XR’s traffic cycles and no partial frame would cross the boundaries of SFN_M

Like in previous section all options are feasible with different levels of impacts or implications. H-SFN might be the one that requires the least impact understanding that H-SFN is already defined in specification and could easily be reused or a new timer.

- To enable a counter in DRX formula that increments at every SFN wrap, discuss whether to use a new counter (option 1) or H-SFN (option 2.b) to update the DRX formula.

### DRX reference SFN

Legacy reference SFN is currently defined as follows:

timeReferenceSFN-r16          ENUMERATED {sfn512}       OPTIONAL    -- Need S

…

timeReferenceSFN   Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

The simplest approach is to reuse similar definition for the new DRX specific reference SFN and update the DRX calculation accordingly. Example of the corresponding TP is shown below [15]:

***** Example TP to 38.321 - START *****

[([SFN + N x 1024]× 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset + drxReferenceSFN x 10) modulo (drx-ShortCycle)

…

[([SFN + N x 1024] × 10) + subframe number] modulo (drx-LongCycle) = (drx-StartOffset + drxReferenceSFN x 10) modulo (drx-LongCycle)

***** Example TP to 38.321 - END *****

On summary, legacy reference SFN formulation could be reused and the corresponding new drxReferenceSFN IE could be defined.

- To enable a DRX reference SFN (i.e., drxReferenceSFN), define a new parameter as part of C-DRX configuration reusing same definition as for legacy timeReferenceSFN and add it to the C-DRX formula as shown in TP.

# Conclusion

The observations captured are the following:

Observation 1.	C-DRX enhancements discussed here target traffic with non-integer periodicities aiming to address the issues associated to the misalignment of DRX/XR periodicities and the SFN wrap around and UE would only use them when configured by the network.

Observation 2.	While all proposed solution spaces can address the issue on the mismatch between DRX and XR periodicities seems technically feasible, the open question is the implication, applicability and corresponding specification impacts foreseen.

The proposals captured are the following:

Proposal 1.	To address the mismatch between DRX and XR periodicities, it is proposed to adopt solution space (2.2) “DRX start offset adjustment” for which network pre-configures multiple DRX start offset values (i.e. a list of values) to be used.

Proposal 2.	To enable a counter in DRX formula that increments at every SFN wrap, discuss whether to use a new counter (option 1) or H-SFN (option 2.b) to update the DRX formula.

Proposal 3.	To enable a DRX reference SFN (i.e., drxReferenceSFN), define a new parameter as part of C-DRX configuration reusing same definition as for legacy timeReferenceSFN and add it to the C-DRX formula as shown in TP.

# Reference

- R2-2302910, Summary of DRX enhancements for XR traffic, Intel Corporation, Sony, April 2023.

- R2-2302514, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, April 2023.

- R2-2300324, Discussion on DRX Enhancements for XR Power Saving, vivo.

- R2-2303861, DRX cycle alignment for XR, Nokia, Nokia Shanghai Bell, CMCC, China Unicom, DENSO CORPORATION, Ericsson, Intel, Google Inc., Huawei, HiSilicon, Samsung, Xiaomi, April 2023.

- R2-2300945, Discussion of DRX enhancement, Lenovo.

- R2-2300565, XR-Specific power saving, ZTE Corporation, Sanechips.

- R2-2300423, Discussing on XR-specific DRX enhancement, Xiaomi Communications.

- R2-2301323, Discussion on power saving scheme for XR, Samsung.

- R2-2301237, Discussion on DRX enhancements, CMCC.

- R2-2300118, Discussion on XR power saving, Huawei, HiSilicon.

- R2-2301508, Discussion on XR-specific power saving, Ericsson.

- R2-2300843, Discussion on DRX enhancement for XR, NEC Corporation.

- R2-2301372, DRX enhancements for XR, MediaTek Inc.

- R2-2300188, DRX enhancements for XR, Qualcomm Incorporated.

- R2-2300432, DRX enhancements for XR traffic, Intel Corporation.