3GPP TSG RAN WG2 Meeting #122                                                                     R2-2305498

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	7.12.2.1

Source:	Intel Corporation

Title:	Mobile IAB mobility enhancement for connected UEs

Document for:	Discussion and Decision

# Introduction

Mobility enhancements, e.g. CHO, RACH-less for RRC_CONNECTED UEs were discussed in RAN2 #121bis-e meeting with following agreements:

CHO: - FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN. RACH-less: - Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB). - RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command). - RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

In this contribution, we mainly focus on the remaining open issues on whether and how to support mobility enhancements for connected mode served UEs.

# Discussion

## CHO

During RAN2 #121bis-e meeting, it was proposed to consider CHO with CondEventT1 (which is specified for NTN scenario) to mobile IAB. As per the proponents, the motivation for considering this is to stagger the peak signalling load.

In Rel-17 NTN, timer-based CHO trigger event was introduced as a new conditional trigger event for NTN-capable UEs. As agreed in RAN2 #115e meeting [1], t1-Threshold in CondEventT1 is defined to use UTC, and the UE shall also be GNSS-capable to calculate UTC.

- RAN2 adopts Option 1: UTC time + duration/timer, e.g. 00:00:01 + 40s for representing T1 and T2 for CHO time event.

Mobile IAB-node is supposed and designed to serve the normal UEs, hence, there’s no need to mandate such UE to be GNSS-capable. During Rel-17 NTN discussion, there were some other solutions proposed that doesn’t require utilizing UTC, for example, “reference time + duration/timer, e.g. SFN=0+40s”, “two timers, 301s+341s”. However, based on TS 38.331 [2], t1-Threshold is designed as integer from 0 to 5497555813887. Expanding t1-Threshold format from UTC to either SFN or timer will require specification change, for example, adding types of t1-Threshold format, e.g. UTC/SFN/timer and define the range of time window for SFN/timer.

Observation 1: Existing solution for CondEventT1 is only applicable for NTN capable UEs.  Further, it cannot be directly applied for mIAB as a mobile IAB-node served UE may not be GNSS-capable. Adopting new format, e.g. SFN/timer, as t1-Threshold requires additional specification work in TS 38.331 to define new entering/leaving condition and the corresponding value range for CondEventT1.

Additionally, it was captured in TS 38.300 [3], CondT1 shall always be configured together with another measurement-based trigger condition.

##### 16.14.3.2.2 Conditional Handover The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified. NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]: - The RRM measurement-based event A4; - A time-based trigger condition; - A location-based trigger condition. A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12]. It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

This is because, during Rel-17 NTN discussion, it was questioned the trigger of CHO cannot purely reply on the timer/location information, as single timer/location information cannot be trusted for handover decision. CHO mechanism needs to employ radio measurement to make sure the target candidate CHO cell has a good channel quality as well. In mobile IAB scenario, though the source and target logical DU are co-located, it is still difficult to solely rely on timer-based CHO trigger event, as the configured time window may not accurately reflect the operating mode (ON/OFF) of the target logical DU as well as its channel condition. Hence, the same principle as NTN should still be followed in mobile IAB, that is, timer-based CHO trigger event needs to be used in combination with the existing CHO measurement-based event (i.e. CondA3 and CondA5 agreed for eIAB in Rel-17).

As agreed in RAN2 #114e meeting, the timer trigger event can only be used when the measurement-based event is met.

- CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.

Based on the above observation, it seems configuring CHO to the served UEs with CondEventT1 only benefits to distribute CHO execution timing across served UEs to avoid handover congestion of the UE group. However, the same purpose can be achieved by normal handover via network implementation, where the IAB-donor CU can gradually handover the group of served UEs from the first logical DU to the second one. There’s no need to configure CHO as an extra step, since normal handover can achieve the same goal.

Observation 2: For mobile IAB served UEs, timer-based CHO trigger event will be activated only if the measurement-based trigger event condition is met. It will only benefit to distribute group UEs’ handover to a certain period, which can be equally achieved by normal handover via network implementation.

Proposal 1: For mobile IAB-node’s served UEs, CHO is not enhanced in Rel-18 mIAB, including introducing CondEventT1 for served UEs.

## RACH-less Handover

### General UE procedure for RACH-less HO

The mobile IAB-node can operate in both FR1 and FR2. In our understanding, if a mobile IAB-node operating in FR1, there’s no need to specify beam handling for mIAB RACH-less HO, since it is similar to LTE RACH-less HO. The beam handling issue during RACH-less HO is specifically for FR2.

As discussed in NR NTN, RACH-less HO may be helpful when the group of UEs perform intra and inter satellite handover, where RACH congestion and delay can be avoided. It was agreed in RAN2 #121 meeting that LTE RACH-less was taken as baseline for NR NTN:

Agreements:

- Support RACH-less Handover in Rel-18.

- RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition

Recalling that NR NTN is also operating in FR1, the same assumption can be reused for mIAB-nodes that are operating in FR1. For example, RACH-less configuration in RRCReconfiguration message for TA, pre-allocated UL grant, trigger of security key change during RACH-less HO, HO confirmation, etc, could be common for NTN and mobile IAB as well, which is similar to the LTE RACH-less design.

Observation 3: For mIAB operating in FR1, much of the procedure of RACH-less HO can be common between mobile IAB WI and NTN WI (if RACH-less HO is adopted). It is feasibility to adopt RACH-less HO in the mIAB WI in FR1, regardless of beam handling.

During RAN2 #121bis-e meeting, following agreements on detailed procedure of RACH-less HO was agreed in Rel-18 NTN:

- RAN2 confirms the general UE procedure for NTN RACH-less HO

1.	receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

2.	start timer T304 for the target cell (RRC)

3.	perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

4.	start time alignment timer (MAC)

5.	monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

6.	send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

7.	consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

8.	stop timer T304 for the target cell. (RRC)

FFS whether to release UL grant if pre-allocated after RACH-less HO completion

FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

4.	The pre-allocated grant is provided as type-1 CG

5.	Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

1. whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

2. to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

3. power control for initial UL transmission

Agreements online:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

Based on above observation, we therefore propose:

Proposal 2: At least for FR1, RACH-less HO is supported for served UEs during mIAB DU migration, including:

- receive a RACH-less HO command which can include pre-allocated grant optionally, indicating the same TA is used.

- Start T304 for target logical DU

- Pre-allocated UL grant can optionally be provided as type-1 CG

- Support of dynamic grant by monitoring target cell PDCCH if pre-allocated grant is not configured in RACH-less HO command

- Using available UL grant for initial UL transmission including RRCReconfigurationComplete message

- Confirm completion of RACH-less HO by receiving UE Contention Resolution Identity MAC CE.

- Upon completion of RACH-less HO, stop T304

### Wayforward for Beam Handling for mIAB served UEs

During RAN2 #121bis-e meeting, following two options were agreed to be further discussed to support beam operation if RACH-less handover is supported for mIAB-served UEs.

- RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

In legacy handover, upon handover, the UE and the gNB performs initial beam detection procedure via RACH operation. The UE detects the best SSB beam from gNB and inform the selection by using a specific PRACH resource mapped to the corresponding DL SSB beam. After that, the gNB and the UE perform further beam management process to use finer granularity of beams by using SSB/CSI-RS measurement/reporting. To be more specific, following is a list of information how beam information is provided for different channels:

- PDCCH: PDCCH beam indicated with TCI state either configured by RRC or MAC CE if multiple TCI states are configured.

- PUSCH: Spatial relation info is indicated on DCI format.

- PDSCH: 128 TCI states are configured by RRC and MAC-CE is used to activate up to 8 TCI state and DCI selects one of them.

However, since the finer granularity of beam management will only take place after RACH procedure, if RACH-less HO is used, the network only needs to provide the wide beam information together with the HO command.

Observation 4: Finer granularity of beam management will take place after RACH. The network only needs to provide wide beam information together with HO command if RACH-less HO is used.

During mobile IAB DU migration, since the relative location between the served UE and the mobile IAB-node is not changed, it is possible that the same/similar physical beam with similar direction and angle could be reused by network and the served UEs before and after UE’s handover. Therefore, even there’s a small difference in the direction and/or angle of a narrow beam, the same wide beam of the source logical DU may still be reused by target logical DU and the served UE. Then, the question is how to provide configuration of such wide beam towards the served UEs during RACH-less HO.

For Option 1, if beam related configurations, e.g. PDSCH-Config, PDCCH-Config, etc at the source logical DU is known by the target logical DU, the wide beam information may be provided by the target logical DU to the served UEs based on the knowledge of the QCL relationship between the narrow and wide beam (e.g. CSI-RS beam and SSB beam).

However, different from Option 1, Option 2 assumes the target logical DU reuses the CellGroupConfig of the source logical DU and  the target logical DU may maintain the same ServingCellConfig for the served UEs, and the served UEs could continue using the same beam even without changing the corresponding configuration. In this case, the same TCI state for PDCCH and PDSCH and spatial relation info of PUSCH can be provided in RACH-less HO command.  While this may not be possible in all scenarios, it could be possible in many deployments to justify this approach.

However, both options require further confirmation with RAN1 regarding to the feasibility of providing wide beam information for initial UL transmission and/or DL reception based on original beams used for PDCCH/PDSCH/PUCCH in source logical DU. Hence, we suggest:

Proposal 3: For FR2, RAN2 to check feasibility with RAN1 about providing wide beam information for initial UL transmission and DL reception in HO command based on QCL relationship between narrow beam and wide beam of the source logical DU.

# Conclusion

In this contribution, we first discuss the specification changes might required by adopting CondEventT1 to mobile IAB. Then discuss the way-forward for RACH-less handover for served UEs, e.g. adopting RACH-less HO in FR1, and checking beam handling feasibility with RAN1.

In summary, we have following proposals and observations:

CHO

Observation 1: Existing solution for CondEventT1 is only applicable for NTN capable UEs.  Further, it cannot be directly applied for mIAB as a mobile IAB-node served UE may not be GNSS-capable. Adopting new format, e.g. SFN/timer, as t1-Threshold requires additional specification work in TS 38.331 to define new entering/leaving condition and the corresponding value range for CondEventT1.

Observation 2: For mobile IAB served UEs, timer-based CHO trigger event will be activated only if the measurement-based trigger event condition is met. It will only benefit to distribute group UEs’ handover to a certain period, which can be equally achieved by normal handover via network implementation.

Proposal 1: For mobile IAB-node’s served UEs, CHO is not enhanced in Rel-18 mIAB, including introducing CondEventT1 for served UEs.

RACH-less HO

Observation 3: For mIAB operating in FR1, much of the procedure of RACH-less HO can be common between mobile IAB WI and NTN WI (if RACH-less HO is adopted). It is feasibility to adopt RACH-less HO in the mIAB WI in FR1, regardless of beam handling.

Proposal 2: At least for FR1, RACH-less HO is supported for served UEs during mIAB DU migration, including:

- receive a RACH-less HO command which can include pre-allocated grant optionally, indicating the same TA is used.

- Start T304 for target logical DU

- Pre-allocated UL grant can optionally be provided as type-1 CG

- Support of dynamic grant by monitoring target cell PDCCH if pre-allocated grant is not configured in RACH-less HO command

- Using available UL grant for initial UL transmission including RRCReconfigurationComplete message

- Confirm completion of RACH-less HO by receiving UE Contention Resolution Identity MAC CE.

- Upon completion of RACH-less HO, stop T304

Observation 4: Finer granularity of beam management will take place after RACH. The network only needs to provide wide beam information together with HO command if RACH-less HO is used.

Proposal 3: For FR2, RAN2 to check feasibility with RAN1 about providing wide beam information for initial UL transmission and DL reception in HO command based on QCL relationship between narrow beam and wide beam of the source logical DU.

# References

[1] R2-2109301	RAN2#115-e Meeting Report

[2] TS 38.331 Radio Resource Control (RRC) protocol specification

[3] TS 38.300 NR and NG-RAN Overall Description; Stage 2