3GPP TSG-RAN WG2 Meeting #122	                                     R2-2305506

Incheon, Korea, May 22 – May 26, 2023

Agenda item: 	7.7.1

Source:	Intel Corporation

Title: 	Discussion on NR NTN UE capabilities

Document for:	Discussion and Decision

# Introduction

For Rel-18 NR NTN enhancements [1], some new features have been agreed, e.g., network verified UE location, location based measurement initiation in Earth-moving cells, and RACH-less handover.

In this paper, we further discuss which UE capabilities can be defined accordingly.

# Discussion

## 2.1 Network verified UE location

The following descriptions are excerpted from WID [1] as below:

Optional feature: Note 7 : Network verified UE location is an optional UE feature Target positioning solution: RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view Other candidate positioning solutions: DL-TDoA methods for verification may be considered as lower priority and if time permits Note 4 : Multiple satellite in view by the UE may be considered if time allows

Based on the descriptions above, the network verified UE location is an optional UE feature. There are under consideration one target positioning solution (i.e., single satellite multi-RTT), and two candidate positioning solutions if time allows (i.e., DL-TDoA and multiple satellite in view (e.g., for multi-RTT)).

So, one thing that could be confirmed is single satellite multi-RTT for the purpose of network verified UE location is an optional feature. Whether multiple satellite multi-RTT or DL-TDOA positioning method can also be supported is still FFS.

Observation 1: single satellite multi-RTT is the target positioning method for the purpose of network verified UE location, and whether multiple satellite multi-RTT or DL-TDOA positioning method can also be supported is still FFS.

In legacy positioning, the multi-RTT UE capability is reported to LMF from UE directly (e.g., the reporting procedure is as shown in Figure 5.1.1-1 of LPP TS 37.355) without having any UE capability defined for this on 38.306. On other hand, positioning inter-related features with RAN impact may require new UE features defined in TS 38.306 (e.g., enhancements to reference signals for positioning).

Figure 5.1.1-1: LPP Capability Transfer procedure

Since the RAN1 discussion on single satellite multi-RTT is still ongoing, e.g., it’s not clear whether some enhancements on positioning reference signals are needed for NTN according to the following RAN1 agreements.

Agreement Existing DL/UL reference signals for positioning are used for supporting Network verified UE location in NTN. FFS: Whether some enhancements on these reference signals are needed for NTN

Moreover, the key parts of multi-RTT positioning method are the measurements/calculation of UE Rx-Tx time difference and gNB Rx-Tx time difference. According to RAN1 agreements, there are still 3 options on the table for further discussion, and it is possible that one or more options may be adopted. Therefore, it is still not clear what kind of new single satellite multi-RTT UE capability from UE to LMF is needed.

Agreement For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements: - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted - Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS - Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4 FFS: One or multiple SRS can be used in determining the arrival time FFS: Additional enhancement including additional information to be reported, if justified Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

Observation 2: according to current RAN1 progress, it is still not clear whether some enhancements on positioning reference signals need to be specified as part of UE capabilities (in TS 38.306) and what kind of new single satellite multi-RTT UE capability from UE to LMF is needed (in 37.355).

To sum up, single satellite multi-RTT for the purpose of network verified UE location is an optional feature, but RAN2 still needs to wait for RAN1 progress to discuss the spec impact on TS 37.355 and TS 38.306/38.331.

Proposal 1: Single satellite multi-RTT for the purpose of network verified UE location is an optional feature. FFS on the spec impact on TS 37.355 and TS 38.306/38.331 (RAN2 needs to wait for RAN1 progress).

## 2.2 TN-NTN cell reselection

Regarding the enhancements to TN-NTN cell reselection, the following agreement was made:

- UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

The intention is to save idle/inactive UE power consumption on TN neighbour cell measurements. And network needs to provide assistance information to help UE identify the TN area as agreed below:

Agreements:

- TN coverage area information will be associated to the frequency information.

- RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information):

Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

Agreements:

1.	For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

To sum up, a UE supporting this feature needs to decode the corresponding assistance information to identify the TN area, and the UE can skip the neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. Since this is for idle/inactive UEs, it could be an optional UE capability without signalling.

Proposal 2: define an optional without signalling UE capability for skipping the neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

## 2.3 NTN-NTN cell reselection

Regarding NTN-NTN cell reselection enhancements, location based measurement initiation is supported in Earth-moving cell in R18 NR NTN, while location and time based measurement initiation in Quasi-earth fixed cell was supported in R17 NR NTN.

- For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell

Similar to R17 UE capability of location based measurement initiation in Quasi-earth fixed cell, an optional UE capability without signalling can be defined for location based measurement initiation in Earth-moving cell.

Proposal 3: define an optional without signalling UE capability for location based measurement initiation in Earth-moving cell for cell selection/reselection.

In RAN2#121bis, the time based measurement initiation is also agreed as below. The corresponding UE capability can also be defined.

- For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

Proposal 4: define an optional without signalling UE capability for time based measurement initiation in Earth-moving cell for cell selection/reselection.

## 2.4 RACH-less handover

Regarding R18 NR NTN handover enhancements, one confirmed new feature is RACH-less handover.

Agreements:

- Support RACH-less Handover in Rel-18.

And considering in RAN2#116bis-e meeting, a general agreement on FRX/XDD differentiation was made as follows.

From Rel-17 onwards, at least for new capabilities, if a UE capability requires at least FRx or at least xDD differentiation, it is defined with both FRx and xDD differentiation in per band signaling, i.e. no new UE capabilities will be defined in the FRX and XDD capability signaling branches.

Similar to CHO UE capabilities which has been defined as a per band capability, the RACH-less handover UE capability should be a per band capability. In addition to per band capability, whether the per UE capabilities need to be defined (like condHandoverFDD-TDD-r16 and condHandoverFR1-FR2-r16 for CHO), which indicate whether RACH-less handover between FR1 and FR2 is supported, and whether RACH-less handover between TDD and FDD is supported. The support of these cases is still FFS relying on further RAN4 work on NTN bands above 10GHz.

The latest RAN4 agreements are as shown below; it seems that NTN bands above 10GHz could be in FR2. Meanwhile in the WID [1], it’s already stated that “FDD mode is assumed for satellite operation above 10 GHz”. So, we may need to define RACH-LessHandover-NTN-FR1-FR2-r18 to indicate whether RACH-less handover between FR1 and FR2 is supported. But the corresponding RAN2 discussion can be started after RAN4 work on this is concluded.

- Band definition above 10GHz NTN operating band; UL; DL. n512; 27.5 – 30.0 GHz; 17.3 – 20.2 GHz. n511; 28.35 – 30.0 GHz; 17.3 – 20.2 GHz. n510; 27.5 – 28.35 GHz; 17.3 – 20.2 GHz. - NTN sub-frequency range options - Option 1: 17300-24250 MHz as extension of the existing FR2 range Designation; Frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-0-NTN; 17300 MHz – 24250 MHz. FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz. - Option 2: 17300-30000 MHz Designation; Frequency range. FR2-NTN; 17300-30000 MHz. - Option 3: Other options not precluded

Observation 3: whether RACH-less handover between FR1 and FR2/between TDD and FDD need to be considered, relies on the RAN4 work on NTN bands above 10GHz.

Proposal 5: define an optional per band UE capability for NTN RACH-less handover.

Proposal 6: Wait for RAN4 conclusion before discussing whether NTN RACH-less handover between FR1 and FR2/between TDD and FDD need to be indicated.

# Conclusion

In this paper, we discuss NR NTN UE capabilities, and we have the following observations:

Observation 1: single satellite multi-RTT is the target positioning method for the purpose of network verified UE location, and whether multiple satellite multi-RTT or DL-TDOA positioning method can also be supported is still FFS.

Observation 2: according to current RAN1 progress, it is still not clear whether some enhancements on positioning reference signals need to be specified as part of UE capabilities (in TS 38.306) and what kind of new single satellite multi-RTT UE capability from UE to LMF is needed (in 37.355).

Observation 3: whether RACH-less handover between FR1 and FR2/between TDD and FDD need to be considered, relies on the RAN4 work on NTN bands above 10GHz.

And we propose:

Proposal 1: Single satellite multi-RTT for the purpose of network verified UE location is an optional feature. FFS on the spec impact on TS 37.355 and TS 38.306/38.331 (RAN2 needs to wait for RAN1 progress).

Proposal 2: define an optional without signalling UE capability for skipping the neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

Proposal 3: define an optional without signalling UE capability for location based measurement initiation in Earth-moving cell for cell selection/reselection.

Proposal 4: define an optional without signalling UE capability for time based measurement initiation in Earth-moving cell for cell selection/reselection.

Proposal 5: define an optional per band UE capability for NTN RACH-less handover.

Proposal 6: Wait for RAN4 conclusion before discussing whether NTN RACH-less handover between FR1 and FR2/between TDD and FDD need to be indicated.

# References

[1] RP-223534		Revised WID: NR NTN (Non-Terrestrial Networks) enhancements	Thales