3GPP TSG-RAN WG2 Meeting #122                                              	          R2-2305516

Incheon, Republic of Korea, May 22 - 26, 2023

Agenda Item: 	7.5.4.3

Source: 	Sony

Title: 	Configured Grant enhancements for XR

Document for: 	Discussion / Decision

# Introduction

A new work item on XR enhancements for NR was approved [1]. This is basically the normative phase based on the outcome of the XR Rel-18 study item [2]. The objective on XR-specific capacity improvements is to specify the enhancements related to capacity:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

-	BSR enhancements including at least new BS Table(s); (RAN2);

-	Delay reporting of buffered data in uplink; (RAN2);

-	Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2);

-	Discard operation of PDU Sets (RAN2);

During the study item phase, various enhancement techniques were proposed. The selected capacity improvements techniques were divided into physical layer enhancements and layer 2 enhancements. On physical layer enhancements, the following enhancements for configured grant-based transmission are recommended [2]:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration.

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE.

This contribution discusses our views on the mechanisms to improve XR capacity, particularly on the configured grant aspects as one of the recommended items during study item phase.

# Discussion

eXtended Reality (XR) and Cloud Gaming refer to various types of augmented, virtual, and mixed environments, where human-to-machine and human-to-human communications are performed with the assistance of handheld and wearable end user devices (UEs).   XR and Cloud Gaming are two applications that are considered important for NR Rel-18 and beyond (also known as 5G Advanced).  Hence, a Rel-18 Study Item on eXtended Reality (XR) has been approved in 3GPP [1] to study potential enhancements to the legacy 5G system for support of XR traffic.

XR traffic is rich in video especially in the downlink with a typical frame rate of 60 Hz [2], which leads to a data transmission with non-integer periodicity in NR, i.e., the periodicity is not an integer number of subframes and in this example, the periodicity is 16.67 ms.  Due to varying frame encoding delay and network transfer time, the packet arrival at the gNB may experience random jitter. The frame rate and jitter of a DL traffic is illustrated in Figure 1. The non-integer and jitter characteristics of XR traffic is known as quasi-periodic traffic.  The same transmission characteristic could also occur in uplink transmission.

Figure 1: Illustration of DL traffic model [TR 38.838]

In the legacy 5G system, traffic with known periodicity and packet size, e.g., voice, is supported using Semi-Persistent Scheduling (SPS) PDSCH and Configured Grant PUSCH (CG-PUSCH).

In legacy system SPS and CG-PUSCH assumes that the Transport Block Size (TBS) of the PDSCH and PUSCH of the traffic are identical in every period.  However, in XR traffic, the payload of a quasi-periodic traffic may not be the same but varies within a range.

In legacy CG-PUSCH, a single CG-PUSCH occasion within a period is supported. However, there are issues with a single CG-PUSCH occasion in a period:

- In a single CG occasion, the periodicity is always aligned with the arrival periodicity of new data. So, if all the XR data is not transmitted on that occasion, it will be delayed to the next occasion in which another new data is supposed to be transmitted, hence it may cause some delays. One solution could be to overprovision the periodicity of the CG occasions (i.e., with smaller periodicity) in such a way that some of these CG occasions are used and some are unused, and UE can indicate the used/unused occasions in the UCI. Although it may work, but XR data may have different characteristics, for example heavy data transmission in a small period followed by a silent time until a new data arrives from the application layer.

- There could be an application layer jitter at the UE where the generation of the new data may slightly shift in time and cannot be delivered on the intended occasion. For example, in XR application, the jitter can shift the intended occasion within the range of +4 to -4 ms. Hence, if periodicity is long, the data may sit in the buffer for a period that is longer than the XR delay budget. Hence, the data will be discarded at the end.

As mentioned above, the new work item (WI) that has been agreed for XR [1] is to specify multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE. This may mean that whenever a new data arrives/generated, there is a number of dense/compact occasions available for the XR data in a period so that the delay is minimized for PDU set transmission. The compact occasions comprise one main occasion and one or more supplementary occasions.

The operation of multiple CG-PUSCH occasions in a period of a single CG-PUSCH configuration is illustrated in Figure 2. This operation supports the transmission of XR traffics in which the packet size can vary. In this example, the UE buffer varies from 4 Mbps to 2 Mbps and within a CG-PUCH period, the UE is configured with up to 4 CG-PUSCH occasions. In case, the UE buffer reduces then the UE can use some of the CG-PUSCH occasions. On the other hand, in case the UE buffer is full then all the CG-PUSCH occasions can be used.

RAN1 made a progress that a new UCI will be defined to provide information about unused CG PUSCH transmission occasions in a single CG configuration, and this UCI will be carried in every transmitted CG PUSCH (i.e., UCI + data). Therefore, RAN2 should specify RRC parameters for multiple CG-PUSCH occasions in a period of a single CG-PUSCH configuration.

From RRC configuration perspective, the configuration should be done in such a way that there is main CG occasion and one or more supplementary occasions within a period of a single CG configuration. The number of supplementary occasions should be semi-static and RRC configured regardless of whether they are activated (used) or not activated (unused) and all should have an equal offset/gap between them, starting from the main occasion, to simplify the specification. The offset/gap could be zero (i.e., contiguous).

Proposal 1: From RRC configuration perspective, there should be two configurations: the configuration of the main occasion (i.e., the legacy configuration) and the configuration of the number of supplementary occasions within a period of a single CG configuration.

Proposal 2: From RRC configuration perspective, the number of supplementary occasions should be RRC configured (i.e., semi-static) regardless of whether they are used or unused and all should have an equal offset/gap between them, starting from the main occasion. The offset/gap could be zero (i.e., contiguous).

Figure 2: Multiple CG PUSCH transmission occasions in a period of a single CG configuration.

Timeline for indicating unused CG occasions: The benefits of indicating unused CG occasions is to reuse the resources for other UEs at the gNB, as well as reducing the interference. However, gNB can only schedule these resources to other UEs if it gets the information/indication early from the UE. In order to help gNB scheduler, we think RAN2 and RAN1 should specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration (i.e., minimum indication time for unused CG occasion).

Proposal 3: Specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration.

# Conclusion

In this contribution, we have provided our views on the mechanisms to improve XR capacity, particularly on the configured grant aspects. We have the following proposals:

Proposal 1: From RRC configuration perspective, there should be two configurations: the configuration of the main occasion (i.e., the legacy configuration) and the configuration of the number of supplementary occasions within a period of a single CG configuration.

Proposal 2: From RRC configuration perspective, the number of supplementary occasions should be RRC configured (i.e., semi-static) regardless of whether they are used or unused and all should have an equal offset/gap between them, starting from the main occasion. The offset/gap could be zero (i.e., contiguous).

Proposal 3: Specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration.

# References

- RP-223502, “New WID on XR Enhancements for NR”, RANP #98e, December 2022

- RP-223187, “Study on XR enhancements for NR (TR 38.835 V1.0.0)”, December 2022