3GPP TSG-RAN WG2 Meeting #122											   		R2-2305517

Incheon, Republic of Korea, May 22 - 26, 2023

Agenda Item:	7.5.4.3

Source: 	SONY

Title:	Options for Retransmission-less CG for XR traffic

Document for:	Discussion and Decision

# Introduction

RAN#99 agreed an updated WID on XR Enhancements for NR [1] with the following objectives:

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g., RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g., periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

RAN1 Conclusion:

“Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue.

Note: how to capture evaluation results and findings will be separately discussed.

RAN2 Agreement:

- Whether the issue of retransmission-less CG for UL pose transmission is addressed in the WI needs to be discussed in RAN

RAN2#121bis Agenda:

#### 7.5.4.3 Configured Grant enhancements for XR

Including RAN2-specific aspects of Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration.

Including RAN2-specific aspects of dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE.

Including discussion on how retransmission-less CG defined for NTN could work with XR (as per RAN#99 discussion).

RAN2#121bis discussion:

For the power saving of transmitting pose control information, RAN2 to down-select from the following two options:

Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG. Sample TP in Annex A of R2-2304391 (13/24)

Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG. Sample TP in Annex B of R2-2304391. (6/24)

Agreements:

There is support to adopt NTN solution for the retransmission-less CG.

If adopted, RAN2 aims to only consider option1 or option 2:

- Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

- Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG.

FFS how the solution ensures consistent HARQ operation.

RAN1#112bis discussion:

Agreement

- Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112)

  - FFS details

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Agreement

- The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement).

RAN2#122 Agenda:

7.5.4.3 Configured Grant enhancements for XR

Including discussion on retransmission-less CG, e.g. how does the solution discussed in RAN2#121bis-e ensure consistent HARQ operation?

In this contribution, we discuss solutions for the retransmission-less CG and whether the short-listed options in RAN2 are suitable for CG in terms of power saving and capacity enhancement.

## Retransmission-less CG for XR

The main motivations of retransmission-less CG-PUSCH are from power saving perspective and capacity enhancements as follows:

- From power saving perspective, if there are frequent transmissions of UL pose/control information (e.g., 4ms periodicity), then a UE should monitor PDCCH for retransmissions (based on drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL) and this will increase UE power consumption significantly.

- From capacity enhancement perspective, if HARQ retransmissions are avoided, some resources can be saved, for example the DL PDCCH resource as well as the scheduled retransmission resource for the uplink PUSCH, hence the system capacity can be improved in practice.

During RAN2-121bis-e meeting, RAN2 discussed solutions for retransmission-less CG, and two options were short listed. We believe that there are some drawbacks for both short-listed options and there is a better solution when considering from the capacity enhancement and power saving perspectives. Hence, we propose a third option (option 3) and compare all three options/solutions in detail:

Option 1. Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration: This option introduces a new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration. In the current 3GPP system, there are a maximum of 12 CG configurations, and in this option, each of these CG configurations can independently configured the retransmission-less parameter, that is the gNB can decide which CG is HARQ retransmission-less and which one requires HARQ retransmission. This option will limit the flexibility of using this CG for other transmissions/services that require for retransmissions. Another issue is that when the CG is not used due retransmission-less data is not available, the resources are wasted, hence reducing the capacity in the system.

Option 2. Adapt the NTN solution by disabling the HARQ RTT timer per HARQ process for both CG and DG: In NTN system, HARQ Mode B was introduced in NTN where for a set of HARQ Process Numbers (HPN), HARQ retransmissions are disabled. It is proposed in [2] that HARQ Mode B is supported thereby allowing CG-PUSCH to operate without HARQ retransmission. However, HARQ Mode B is configured on a HPN basis, that is whether HARQ retransmission is used depends on the HPN of the PUSCH. This may be acceptable for dynamic PUSCH where the HPN is indicated by the gNB, however, for CG configuration in licensed spectrum, the HPN depends on the starting symbol of the CG-PUSCH occasion and its periodicity which is not easily controlled by the gNB nor the UE, and so HARQ Mode B is not suitable for CG-PUSCH operation. The consequence/issue is that a UE must wait the next available CG occasion with retransmission-less HARQ process, and that introduces a significant delay for UL pose/control transmissions.

Option 3. Dynamic indication of the Retransmission-less CG-PUSCH: Another option is to indicate the HARQ retransmission-less dynamically in the UCI (e.g., CG-UCI) of a CG-PUSCH that denotes whether the current CG-PUSCH is HARQ retransmission-less or requires HARQ retransmission. This enables the UE to indicate per CG-PUSCH occasion whether that CG-PUSCH transmission requires HARQ retransmission or not. Note that RAN1 already agreed that a new UCI will be defined to provide information about unused CG PUSCH transmission occasions in a single CG configuration, and this UCI will be carried in every transmitted CG PUSCH. Therefore, RAN2 should consider this option 3 where the retransmission-less indicator (just 1-bit) is included in the UCI dynamically for every transmitted CG PUSCH. The benefit is that it provides full flexibility of allowing any CG occasion to operate with or without HARQ retransmissions. In addition, compared to Option 1 and 2 above, this option 3 does not waste resources and does not introduce delays for UL pose/control transmissions. Even if the probability of miss-detection of the UCI is very low, the UCI miss-detection can already happen for NR-U in Rel-16/17. Hence, RAN2 should consider dynamic Retransmission-less indicator included in every transmitted CG-PUSCH (via UCI).

Furthermore, the network/gNB can schedule HARQ retransmission whenever the UCI that carries the Retransmission-less indicator cannot be decoded at the gNB. If gNB cannot decode the UCI, the gNB can always schedule for retransmission for the PUSCH as it does not know whether UE has triggered HARQ retransmission or retransmission-less for the PUSCH. In this case, if the UE triggered HARQ retransmission then it will monitor for retransmission which is aligned with the gNB behaviour. However, if the UE triggered retransmission-less then it will not monitor for retransmission which still saves its power consumption. Note that gNB can determine whether there was a transmission on any occasion based on previous UCI indicating whether this occasion is unused or used. This way, any out-of-sync between gNB and UE is certainly avoided.

Observation 1: Option 1 will limit the flexibility of using the CG for other transmissions/services that require for retransmissions. In addition, when the CG is not used due retransmission-less data is not available, the resources are wasted, hence reducing the capacity in the system.

Observation 2: For Option 2, for CG configuration in licensed spectrum, HARQ Process Number (HPN) depends on the starting symbol of the CG-PUSCH occasion and its periodicity which is not easily controlled by the gNB nor the UE. Hence the UE must wait the next available CG occasion with retransmission-less HARQ process, and that introduces a significant delay for UL pose/control transmissions.

Observation 3: Option 3 provides full flexibility of allowing any CG occasion to operate with or without HARQ retransmissions. In addition, Option-3 does not waste resources and does not introduce delays for UL pose/control transmissions.

Proposal 1: RAN2 should consider Retransmission-less indicator included in every transmitted CG-PUSCH via UCI (Option 3). RAN2 should send an LS to RAN1 to do the final specification.

Note that from RRC perspective for Option 3, it can reuse the parameter “allowedHARQ-mode” in “The IE LogicalChannelConfig” that provides the mapping restriction whether a logical channel is configured for HARQ retransmission or HARQ retransmission-less. Hence, a UE knows from the RRC parameter for a logical channel whether to indicate HARQ retransmission-less or normal HARQ retransmission in the UCI dynamically.

## Conclusions

In this contribution, we have discussed solutions for the retransmission-less CG and whether the short-listed options/solutions in RAN2 are suitable for CG in terms of power saving and capacity enhancement.

We have the following observations and proposal.

Observation 1: Option 1 will limit the flexibility of using the CG for other transmissions/services that require for retransmissions. In addition, when the CG is not used due retransmission-less data is not available, the resources are wasted, hence reducing the capacity in the system.

Observation 2: For Option 2, for CG configuration in licensed spectrum, HARQ Process Number (HPN) depends on the starting symbol of the CG-PUSCH occasion and its periodicity which is not easily controlled by the gNB nor the UE. Hence the UE must wait the next available CG occasion with retransmission-less HARQ process, and that introduces a significant delay for UL pose/control transmissions.

Observation 3: Option 3 provides full flexibility of allowing any CG occasion to operate with or without HARQ retransmissions. In addition, Option-3 does not waste resources and does not introduce delays for UL pose/control transmissions.

Proposal 1: Proposal 1: RAN2 should consider Retransmission-less indicator included in every transmitted CG-PUSCH via UCI (Option 3). RAN2 should send an LS to RAN1 to do the final specification.

## References

# [1] RP-230786, “Updated WID on XR Enhancements for NR”, RAN#99.

[2] R1-2210002, “Power Saving Techniques for XR,” Qualcomm, RAN1#110bis-e.