3GPP TSG-RAN WG2 Meeting #122						R2-2305525

Incheon, Republic of Korea, May 22 – 26, 2023

Agenda Item:	7.16.2.2

Source: 	Sony

Title:	Some considerations about data collection

Document for:	Discussion

# Introduction

RAN2_121 made following agreement:

- R2 may consider including the existing EVEX framework for this SI, FFS exactly what this means, can discuss next meeting.

And RAN2_121bis agreements on data collection:

- The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage - Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s). - P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information. - P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement. FFS how we do the formatting/presentation of the results. - P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results. - P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training) - - Model sidedness (UE side, NW side, two sided) FFS - - Use case mapping FFS - P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS - Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM).

And then the table for data collection methods was appended with three new columns

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy | Inference | Monitoring | Offline Training |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Framework |  |  |  |  |  |  |  |  |  |  |

MDT and EVEX framework were discussed for data collection during RAN2_121bise meeting and there was no conclusion. In this contribution, we attempt to compare both EVEX and MDT and provide a comparison table to start with.

# Discussion

## 2.1 EVEX

RAN2_121 discussed a contribution in [3] which argued to use EVEX framework for offline training. EVEX framework is spread across different SA WGs, as indicated by the LS on “Alignment of activities on UE data collection reporting and event exposure” from SAP#99 in [4].  UE data collection reporting and event exposure work is spread between SA4/6/2 WG specifications and this LS from SA plenary is tasking SA4 to either take the ownership of EVEX framework specifications or leave it for SA2 to maintain EVEX specifications.

SA4 specifications TS 26.531 has a reference architecture for data collection and reporting. In our understanding, RAN2 focus may be on reference point R2, between Direct Data Collection Client and Data Collection AF, as shown in figure below:

Figure 4.2-1: Reference architecture for data collection and reporting in reference point notation

Data collection from UE application is further specified in SA2 specification TS 23.288 section 6.2.8 and a summary of this procedure was also provided in [3].

## 2.2 MDT

MDT is specified mainly by RAN2 and RAN3.

## 2.3 Comparison

Offline training

In our understanding, offline training may require data collection over a period of time. [Qualcommpaper] proposed to use EVEX for offline training for a UE sided model. The assumption is that offline training may collect data which is not standardised.

We think MDT could also be used for UE sided model but then the handling of proprietary data may not work well. But then a standardised UE sided model should have standardised data collection methods, in our opinion.

Proposal 1: RAN2 to discuss if a Standardised UE sided AI/ML model should have a standardised data collection method.

Data collection spanning over different RRC states

It was pointed out by many companies that EVEX does not support RAN involvement in data collection procedure. Both EVEX and MDT need enhancements in collecting data spanning over different RRC states. If a UE state transition takes place, then MDT data continuity is not maintained. EVEX is user plane based so data collection only in connected mode may be supported and FFS for SDT. In our understanding, the effort for MDT might be small compared to EVEX because MDT work can be done in RAN2 with small changes. EVEX may require coordination with other WGs regarding RAN involvement.,

Observation 1: MDT effort may be smaller compared to EVEX for data collection over different RRC states

Granularity of data collection

MDT data collection is done for all configured parameters like RSRP, UE location etc., and it is not possible if one item is measured and collected at different rate than others e.g. UE location is reported more frequently. It is not clear if EVEX can support such a behaviour.

Observation 2: if any parameter needs to be collected with different frequency, such flexibility does not exist in MDT. FFS for EVEX

Based on above, we propose following table for comparison:

| Feature | EVEX | MDT |
| --- | --- | --- |
| RAN involvement in data collection | No, data collection is done via using user plane path. Need enhancements to involve RAN. | Yes, data collection is done via using RRC |
| Data collection in RRC states | User plane path is available in connected mode only. FFS for SDT | Data collection possible in all RRC states. However, data collection continuity during RRC state change is not supported yet |
| User consent for measurement collection | Yes | Yes |
| Record interval per measurement quantity | TBD | Not supported yet |
| Non standardised data collection | Yes Ideal for UE sided model and handling of non-standardised measurements | No Ideal for UE sided and NW sided models |
| Transport mechanism | User plane | Control plane |

We think its bit early to populate the data collection table based on agreements so far and above comparison table should be used as a reference.

Proposal 2: Use above comparison table as a starting point to populate the data collection table for EVEX and MDT.

# Conclusion

We propose RAN2 to discuss and agree on the following observations and proposal:

Proposal 1: RAN2 to discuss if a Standardised UE sided AI/ML model should have a standardised data collection method.

Observation 1: MDT effort may be smaller compared to EVEX for data collection over different RRC states

Observation 2: if any parameter needs to be collected with different frequency, such flexibility does not exist in MDT. FFS for EVEX

Proposal 2: Use above comparison table as a starting point to populate the data collection table for EVEX and MDT.

# References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, Dec. 2021

[2] AI Enlightens Wireless Communication: A Transformer Backbone for CSI Feedback accessed from: https://arxiv.org/pdf/2206.07949.pdf

[3] R2-2301427: Discussion on AI/ML methods, Qualcomm

[4] SP-230394: Alignment of activities on UE data collection reporting and event exposure, SA