3GPP TSG-RAN WG2 #122	R2-2305530
Incheon, Korea, May 2023

Agenda Item:	7.3.4

Source:	OPPO

Title:	Discussion on cell selection/reselection

Document for:	Discussion, Decision

Introduction

This paper will discuss cell selection/reselection, which is one objective of NES WID.

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

Discussion

In RAN2#121, RAN2 concludes the following, which in our view implies no backward compatibility issue exists for the legacy UE to access the NES cell.

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

While, per the previous discussions, companies also discuss barring the legacy UE if the NES has impacted such UE’s performance. In R2-2304353, there is a view that the UE’s performance would be degraded by some NES techniques and such UE can be switched to other cells accordingly. Some view also exists in RAN1 that the reduced antenna port due to the spatial-domain NES techniques would lead to the degradation of legacy UE’s performance. Besides, it would be possible for the operator to deploy a new specific cell to implement network energy saving. With this in mind, it values and seems reasonable to consider the mechanism to prevent legacy UEs from an NES cell.

TR 38.864 reflects RAN2’s conclusion on cell selection/reselection.

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

On the potential solutions above, we would like to provide our analysis.

In legacy, the intra-/inter frequency excluded cell list is contained via SIB3/4, which implies that such excluded cell list cannot be used for cell selection. Meanwhile, cellBarred or cell reservation field is normally provided via MIB/SIB1, thus this option is applicable for both cell selection and reselection. To simplify the spec, a unified mechanism is preferred to prevent the legacy UEs from selecting/reselecting an unsuitable cell, i.e. RAN2 is suggested to use the cellBarred or cell reservation fields in MIB/SIB to block the legacy UEs from accessing an NES cell.

As NES is a new feature of NR R18, the most straightforward way is to reuse cellBarred in MIB to bar the legacy UEs. From the network flexibility perspective, a new barring indication (e.g. cellBarredNES) can be introduced in SIB1, to control the NES-capable UEs accessing a certain NES cell. Accordingly, accessing a certain NES cell can be implemented as below,

- If cellBarred in MIB is indicated as barred, the legacy UEs are barred from this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is indicated in SIB1 as not barred, the NES-capable UE can access this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is indicated in SIB1 as barred, the NES-capable UE cannot access this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is not present in SIB1, the NES-capable UE cannot access this cell.

Besides, we prefer to have the unified control of NES-capable UEs accessing an NES cell since blocking the NES-capable UE from a certain NES cell actually reflects an appeal of the network itself (e.g. energy saving or load balance).

If needed, cellBarred in MIB is used to block the legacy UE from accessing an NES cell.

RAN2 introduces a new barring indication in SIB1 to control the NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

RAN2 confirms a unified control of all NES-capable UEs accessing a certain NES cell, i.e. all NES-capable UEs are uniformly blocked or allowed to access a cell.

Conclusion

We have the following proposals:

Proposal 1	If needed, cellBarred in MIB is used to block the legacy UE from accessing an NES cell.

Proposal 2	RAN2 introduces a new barring indication in SIB1 to control the NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

Proposal 3	RAN2 confirms a unified control of all NES-capable UEs accessing a certain NES cell, i.e. all NES-capable UEs are uniformly blocked or allowed to access a cell.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR