3GPP TSG-RAN WG2 #122	R2-2305533
Incheon, Korea, May 2023

Agenda Item:	7.5.4.1

Source:	OPPO

Title:	Discussion on BSR enhancement for XR

Document for:	Discussion, Decision

Introduction

This paper will discuss the below objectives of capacity enhancement.

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Discussion

Delay status reporting

In RAN2#120, RAN2 agrees to report delay information which is associated with data volume.

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

In reality, one UE can be served with multiple services simultaneously but only the XR services have the requirements of delay status reporting. To reduce the unnecessary overhead of delay status reporting, the gNB can simply configure which LCH/LCG to report the delay status information based on the QoS requirement and the traffic characteristics.

The gNB configures which LCH/LCG is allowed to report the delay status.

The delay status information would be represented as the remaining time, which can help the gNB know the tolerant time to transmit the packet. The remaining time would be reported in the unit of the PDU set, since QoS-related characteristics (e.g. PSDB) and the required AS operation are designed per PDU set basis.

The delay status information is represented as the remaining time, which is reported in the unit of the PDU set.

If the delay status is agreed to be reported per the PDU set basis, the value of the remaining time would be the difference value between the configured PSDB and the buffering time of the PDU set. The UE would derive/calculate such a value based on the boundary of the associated PUSCH which contains the reported remaining time.

The delay status information is the difference value between the associated PSDB and the buffering time of the PDU set.

As reporting the delay status is a new requirement of R18 and the delay status has not been contained in any existing MAC CE, there should be a new MAC CE to contain the accurate remaining time associated with individual PDU sets. Otherwise, timely scheduling is hard to implement and the benefit of introducing the latency report will be weakened. As how much data volume is buffered in the UE depends on the UL delay budget, scheduling strategies and radio link quality, one LCH/LCG may sometimes buffer the data volume of (less than) one PDU set and in other cases, one LCH/LCG may buffer the data volume of more than one PDU set among which some PDU set has just arrived but other PDU set(s) has been waiting for a long time. Thus, the new MAC CE would contain one or multiple delay status information of the associated LCH/LCG. In our view, either of the following can be used in delay status reporting.

- Alt 1: A new MAC CE is introduced to contain only one remaining time value. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

- Alt 2: A new MAC CE is introduced to contain more than one remaining time value. Each remaining time value is associated with an individual PDU set which is buffered in the associated LCH/LCG.

Each remaining time has its own associated data volume buffered. To better assist the gNB scheduling, the new MAC CE would also contain the data volume information, in order to indicate how much data buffered for the specific remaining time.

Introduce a new MAC CE to contain the remaining time of the associated LCH/LCG.

RAN2 considers either of the following to report the delay status information.

- Alt 1: The new MAC CE for delay status reporting contains only one remaining time value. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

- Alt 2: The new MAC CE for delay status reporting contains more than one remaining time value. Each remaining time value is associated with an individual PDU set which is buffered in the associated LCH/LCG.

On the triggering event of the delay status reporting, we understand the below criteria could be considered, to make sure the gNB knows the (urgent) delay status information as soon as possible.

- Alt a: The delay status reporting is triggered when a fresh PDU set is available.

- Alt b: The delay status reporting is triggered when the remaining time a PDU set approaches the threshold.

RAN2 considers the following triggering criteria for the delay status reporting.

- Alt a: The delay status reporting is triggered when a fresh PDU set is available.

- Alt b: The delay status reporting is triggered when the remaining time a PDU set approaches the threshold.

New BSR

In the current system, the BS information buffered in the uplink is reported by BSR MAC CE. The Index field in the current BSR format is used to represent the BS value, which is a quantization indication of the BS of an LCG in the UE’s buffer. The Index field indicates a range of buffer size, i.e. the actual buffer size is between the pre-defined maximum and minimum values. The larger the Index is, the larger the quantization error is introduced. The quantization error is tolerant in legacy but cannot be neglected for R18 XR since the XR service has a low latency requirement as well as large and various packet sizes.

In the previous meeting, RAN2 agrees to introduce new BSR table(s) for the XR service and has achieved the principle below.

- 1. 	As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- 3.	Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- 8.	New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- 6. 	Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- 2a. 	Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- 2b. 	Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

In our view, the fixed BSR table(s) is sufficient and can be pre-defined in the MAC spec, since the XR service we discussed in R18 has a common/known packet size range. The fixed BSR table(s) would avoid the UE/network complexity to generate or update the BSR table(s) dynamically.

For the XR service, the new BS table should be fixed and pre-defined.

In legacy, the BSR table covers the BS range from 0 to infinite. The 8-bit BS table would indicate the amount of available data from 0 to (large than) 81M bytes, while the 5-bit BS table would indicate the amount of available data from 0 to (large than) 150K bytes. In short, the legacy BS table is sufficient to indicate any amount of available data but with the drawback of large quantization error for the larger BS value. As the XR service normally matches the large BS value as well as RAN2 concludes to design the new BSR table(s) based on the XR-specific use cases and requirements, we prefer to introduce the new BS table with the BS range narrower than the legacy BS table. Meanwhile, the new BS table(s) only covers the large BS value range in the legacy 8-bit BS table.

The max and min values of the new BS table(s) would be designed by the packet range of the XR services, which can be derived based on the data rate and the frame rate. As captured in TR 38.835 and simulated by RAN1, the XR frame rates are 15, 30, 45, 60, 72, 90 and 120fps as well as the UL XR data rates are 10 and 20 Mbps with Gaussian distribution, thus, the estimated number of the code points is roughly about 42. While the actual code points needed for the new BS table should be much more than 42. It is because 1) There may exist more than one packet buffered in the LCG due to PDB/PSDB, and 2) In some cases, the uplink grant allocated could not carry all data due to the gNB scheduling strategy. It is true that we can have the ideal BS table for the XR service if we introduce multiple new BS tables with large code points. But it inevitably has a big spec impact and reporting overhead as well as complicates the UE/gNB implementation/behaviour. Thus, we understand introducing a new BS table is a good starting point. The new BS table would be 8-bit or even more to achieve the target quantization level.

The new BS table(s) has a BS range narrower than the legacy BS table, i.e. only covers the large BS value range in the legacy BS table.

As a starting point, RAN2 considers introducing a new 8-bit BS table.

Assuming new BS table(s) has a different BS range from the legacy BS tables, the amount of the available data sometimes would fall into the range of new BS table(s), while in other cases it would fall into the range of the legacy BS tables. If so, the UE would select a proper BS table to encode the BS value based on the data status. In one implementation, if the data volume buffered is larger than the min of the new BS table, the UE selects the new BS table for encoding, otherwise, the UE selects the legacy BS table. The BS table selected for this time BS encoding should be reported by the UE to the gNB for the BS decoding. A new MAC CE associated with a new eLCID(one-octet) should be introduced to contain the BS information as well as a new field indicating the selected BS table.

A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new field indicating the selected BS table.

Conclusion

We have the following proposals:

Proposal 1	The gNB configures which LCH/LCG is allowed to report the delay status.

Proposal 2	The delay status information is represented as the remaining time, which is reported in the unit of the PDU set.

Proposal 3	The delay status information is the difference value between the associated PSDB and the buffering time of the PDU set.

Proposal 4	Introduce a new MAC CE to contain the remaining time of the associated LCH/LCG.

Proposal 5	RAN2 considers either of the following to report the delay status information.

-	Alt 1: The new MAC CE for delay status reporting contains only one remaining time value. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

-	Alt 2: The new MAC CE for delay status reporting contains more than one remaining time value. Each remaining time value is associated with an individual PDU set which is buffered in the associated LCH/LCG.

Proposal 6	RAN2 considers the following triggering criteria for the delay status reporting.

-	Alt a: The delay status reporting is triggered when a fresh PDU set is available.

-	Alt b: The delay status reporting is triggered when the remaining time a PDU set approaches the threshold.

Proposal 7	For the XR service, the new BS table should be fixed and pre-defined.

Proposal 8	The new BS table(s) has a BS range narrower than the legacy BS table, i.e. only covers the large BS value range in the legacy BS table.

Proposal 9	As a starting point, RAN2 considers introducing a new 8-bit BS table.

Proposal 10	A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new field indicating the selected BS table.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] TS 23.501 v18.1.0