3GPP TSG-RAN WG2 Meeting #122	       R2-2305536

Incheon, Korea, May 22-26, 2023

Agenda Item	:	7.5.2

Title	:	On XR awareness

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

In RAN#99, the Rel-18 XR WI objectives have been revised as follow [1]:

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In this Tdoc, we share our views on the XR traffic UE assistance information.

# XR Traffic UE Assistance Information

In RAN2#121, the following agreement has been made regarding the UL jitter information and the signalling of some UL traffic information.

UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

FFS on whether EoDB signalling is needed.

The UE can indicate the periodicity for each traffic flow, including UL AR traffic and UL pose/control information. This allows the network to configure the CG configurations periodicities for each traffic flow. The UE can also update the network about any changes in periodicities via RRC. The information about the UL traffic periodicities can be provided by the 5GC through the AF which can get the information from the UE application layer. However, this can take time and could impact the UL traffic mainly at the start.

Proposal 1: The UE to signal the periodicities of the different XR UL traffics.

To allocate the right amount of UL resources, the UE can also signal information about the sizes of data bursts and pose/control packets. Since data bursts have variable sizes, the UE can report statistics about the sizes to help the network configure the resources. For example, the UE could be configured with a sliding window that it can use to measure data burst statistics such as average size and variance.

The size of the pose/control information packets is fixed and can be reported once at the start of the transmission

Proposal 2: The UE to signal statistics about Data Burst sizes.

To allocate UL resources at the right time, the UE can also signal information about the arrival time for traffic with jitter, such as UL AR traffic. The UE can also report statistics about the arrival time. For example, the UE could be configured with a sliding window that it can use to measure arrival time statistics such as average arrival time and variance. This allows the network to adjust the start of resources, taking into consideration the delay budget. The UE can also report the maximum and the minimum arrival time, but in order to avoid outliers, the UE can report (5%,95%) percentiles or (10% ,90%) percentiles of the jitter distribution.

Proposal 3: The UE to report jitter statistics based on a configured sliding window.

Proposal 4: The UE to report the average jitter, and (x%, y%) percentiles of the distribution of the jitter values in the configured window.

There is no need to consider UL arrival time or jitter for pose/control information.

The UE can also indicate to the gNB the end of each data burst. The gNB doesn’t have awareness about when the data burst will end because of the variable size of the data bursts. The gNB can use this information to recycle some of the overallocated CG resources or to prepare to allocate more dynamic resources if the CG resources are not enough to accommodate the current data burst.

Padding BSR with BSR=0 can indicate EoDB. However, not all BSR values of 0 can be interpreted as EoDB. The buffer might be empty due to various reasons, like jitter or delays in receiving some PDU-Sets from application to the UE modem. This can create an ambiguity and could mislead the gNB if the gNB is using the EoDB information to enable power saving schemes, such as an early termination of the On-Duration time or triggering PDCCH skipping.

Observation 1: Not all BSR values of 0 can be interpreted as EoDB

Proposal 5: The UE to signal the end of each UL data burst (EoDB).

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: The UE to signal the periodicities of the different XR UL traffics.

Proposal 2: The UE to signal statistics about Data Burst sizes.

Proposal 3: The UE to report jitter statistics based on a configured sliding window.

Proposal 4: The UE to report the average jitter, and (x%, y%) percentiles of the distribution of the jitter values in the configured window.

Proposal 5: The UE to signal the end of each UL data burst.

# 4		References

- RP-230786	‘Updated WID on XR Enhancements for NR’　 Nokia, Qualcomm