3GPP TSG-RAN WG2 Meeting #122	      R2-2305538

Incheon, Korea, May 22-26, 2023

Agenda Item	:	7.5.4.3

Title	:	On Configured Grant enhancements for XR

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

A new Rel-18 Work Item for XR enhancements in NR has been approved in RAN#98-e [1]. Multiple enhancements related to the system capacity are to be specified in RAN1 and RAN2 including [1]:

-	 Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

-	 Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE

In this Tdoc, we propose some techniques to specify the enhancements above and we share our views on the retransmission-less CG.

# Multiple CG PUSCH transmission occasions

The support of multiple PUSCH occasions in a CG period is beneficial to reduce latency for XR and improve performance.

Rel-16 NR-U specification includes the use of CG with multiple CG PUSCH occasions within a period. The Rel-16 NR-U feature should be considered the baseline for this XR enhancement in Rel-18.

The current parameters cg-nrofPUSCH-InSlot and cg-nrofSlots of the RRC parameter structure   ConfiguredGrantConfig defined in 3GPP TS 38.331 could be re-utilized to configure multiple PUSCH transmission occasions per CG period on licensed spectrum for XR, as these parameters are currently only supported for unlicensed spectrum operation when the RRC parameter cg-RetransmissionTimer is configured. Hence, the current restriction that the network can only configure these fields if cg-RetransmissionTimer is configured can be relaxed for the XR traffic.

As captured in TS 38.214 clause 6.1.2.3, the higher layer parameter cg-nrofSlots determines the number of consecutive slots allocated within a given CG period. The cg-nrofPUSCH-InSlot parameter configures the number of consecutive PUSCH allocations within each slot. The first allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions. Subsequent allocations have the same length, PUSCH mapping type and are appended without gaps. The same combination of start symbol, length and PUSCH mapping type repeats over the consecutively allocated slots.

The Rel-16 NR-U scheme described above can be extended to XR as is, but further improvements can be made to make it more suitable for the XR traffic.

- The  Rel-16 NR-U multiple CG PUSCH transmission occasions scheme can be extended to XR with some enhancements.

The specified NR-U scheme relies on a retransmission timer (cg-RetransmissionTimer) for autonomous retransmissions. If the UE does not receive an acknowledgment for the previous CG retransmission after this timer expires, it considers the transmission to have been unsuccessful and automatically initiates another transmission.

For XR and instead of the NR-U autonomous retransmission scheme (used mainly because the gNB can have difficulty acquiring a COT to send a DCI scheduling the retransmission), the configuredGrantTimer used with the CG legacy scheme could be applied for each HARQ process associated with each PUSCH occasion of the multiple CG PUSCH transmission occasions.  If the CG timer expires and the UE does not receive an UL DCI scheduling a retransmission then the UE can consider the corresponding HARQ transmission as successful and can consider the HARQ process released and could be used for another transmission.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

In the legacy Configured Grant, the UE does not receive dynamic scheduling for PUSCH transmission. Therefore, the UE is required to derive the HARQ Process Number associated with each PUSCH transmission using the following formula specified in 38.321:

HPI = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes

Where:

CURRENT_symbol = (SFN x SlotsPerFrame x SymbolsPerSlot) + (SlotNumber x SymbolsPerSlot) + SymbolNumber

Where SymbolsPerSlot is 14 for normal CP and 12 for ECP. The SFN is the system Frame Number. SlotNumber is the slot number within which the PUSCH starts. The SymbolNumber is the symbol number at which the PUSCH starts.

The gNB configures the UE with the number of HARQ processes and the periodicity. This is done using the ConfiguredGrantConfig information. The gNB will also dynamically schedule the UE with the HARQ retransmissions for configured grants. In that case, the gNB signals the HARQ process number associated with the requested retransmission in the scheduling DCI.

The existing formula has a limitation and cannot be applied as is for the case of multiple PUSCH occasions in the CG period as all PUSCH occasions in the same CG period will get the same HARQ Process Number.

We propose below to adjust the legacy CG formula to take the multiple PUSCH occasions in the CG into consideration.

HPI = [N *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

where the N defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period. The index I can be in the range from 0 to N.

- Ajust the HARQ Process ID formula to take the multiple CG PUSCH occasions into consideration.

- Adopt the following formula for the HARQ Process ID determination:

- HPI = [N *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

- where the N defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period.

# Retransmission-less CG

In RAN2#121bis-e, the following agreement has been made regarding the support of retransmission-less CG:

There is support to adopt NTN solution for the retransmission-less CG.

If adopted, RAN2 aims to only consider option1 or option 2:

- Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

- Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG.

FFS how the solution ensures consistent HARQ operation.

In the two proposed options in the agreement above, option 1 suggests to disable the HARQ RTT timer for CG while option 2 extends the change to DG as well in addition to CG. Also, regarding the granularity, Option 1 is applied per CG configuration, while option 2 is applied per HARQ process.

We support Option 1 and we think that the enhancements discussed are mainly for XR Pose/Control information which will be relying on the CG configuration. Hence, there is no need to extend this enhancement to DG. Also, the network should be given the flexibility to still re-schedule a retransmission using DG during the DRX On-Duration for Pose/Control information when needed as the UE is monitoring PDCCH anyway during the DRX On-Duration.

- For retransmission-less CG, adopt option 1:

- Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

# Conclusion

In this contribution, we have presented several proposals for the configured enhancements to support the XR traffic:

- The  Rel-16 NR-U multiple CG PUSCH transmission occasions scheme can be extended to XR with some enhancements.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

- Ajust the HARQ Process ID formula to take the multiple CG PUSCH occasions into consideration.

- Adopt the following formula for the HARQ Process ID determination:

- HPI = [N *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

- where the N defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period

- For retransmission-less CG, adopt option 1:

- Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

# References

- RP-223502, “New WID on XR Enhancements for NR” Nokia, Qualcomm, 3GPP TSG RAN Meeting #98-e, Dec. 2022

- 3GPP TR 38.838 V17.0.0 (2021-12) Study on XR (Extended Reality) Evaluations for NR (Release 17)