3GPP TSG-RAN WG2 Meeting #122	R2-2305540

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.4.2.1

Source:	Huawei, HiSilicon

Title:	RACH-less LTM and LTM procedure

Document for: Discussion

Introduction

In the last meeting, we have achieved the following agreements about early TA acquisition, RACH-less and LTM procedure and there are still several open issues left. In this contribution, we discuss the remaining open issues on the LTM procedure consolidation.

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). - R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. - With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - Value 0, - Value indicating that the UE shall apply the TA of one source cell. - R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. - In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. - In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE - In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) - For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. - For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. - Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - LTM supervisor timer is RRC layer timer. - At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. - While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

Discussion

RACH-less LTM

2.1.1 UL grant and beam alignment

In RACH-less procedure, the UE needs to send a UL message to let the target cell/DU know that this UE is coming. So a UL grant should be provided to the UE, either as a configured grant or a dynamic grant. In addition, beam alignment between the UE and the target cell/DU should be completed for delivering this UL message successfully.

Configured grant & beam alignment

The configured grant can be provided to the UE in the candidate cell pre-configuration phase. After the UE receives LTM MAC CE, the UE uses the CG resources for UL transmission to the target cell. Since a beam indication can be included in the LTM MAC CE for RACH-less LTM, the UE knows to use which beam to send a UL message. In intra-DU case, the target cell knows from which beam direction it should receive the UL message. However, in inter-DU case, the target cell/DU does not know from which beam direction. One way is that the source DU notifies the beam to the target DU, however, this F1AP signalling increases the interruption time. Generally, the F1AP coordination immediately before/after sending LTM MAC CE should be avoided in RACH-less LTM, in order to have less interruption.

Another way is that the CG occasions are associated with beams, which is similar with Rel-17 CG-SDT configuration. The UE selects a CG occasion associated with the indicated beam in the LTM MAC CE, and the target cell/DU receives data from the CG occasion with the associated beam. It is unnecessary to associate each fine beam direction with a CG occasion. E.g., some beams with near directions can be associated with one CG occasion, which helps to access to the target cell quickly.

Proposal 1a: If the configured grant solution is used for (inter-DU) RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell), similar to the CG-SDT configuration for beam association.

Dynamic grant & beam alignment

As for dynamic grant, in inter-DU case, the dynamic grant is provided by the target cell/DU PDCCH scheduling, this requires the source DU to notify the target DU that the UE is going to switch to the target cell. In addition, a beam direction is informed to the target DU as well. This F1 notification causes additional delay (maybe ~10ms in some deployment scenarios by source DU to CU via 5ms plus CU to target DU via another 5ms). Then the target DU needs wait for a PDCCH occasion (depends on search space configuration) to send a PDCCH.

Proposal 1b: If the dynamic grant solution is used for (inter-DU) RACH-less LTM, for the first UL data transmission to the target cell:

- the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- upon cell switch decision, the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

2.1.2 Early TA acquisition procedure

In RACH-less procedure, the UE needs a valid TA to send the first UL message. There are two supported methods to provide the TA in PDCCH-ordered early RACH procedure: without RAR method, and with RAR from the serving cell. In both two options, the source cell/DU will get the calculated TA from the candidate cell/DU. Therefore, the source cell/DU knows the value and the validity of candidate cell TA. The source cell/DU needs to know whether a candidate cell TA is still valid or not, because the source cell/DU needs to determine whether it can initiate a RACH-less cell switch and then determine whether it needs to include a beam indication in the LTM MAC CE. In addition, by knowing the validity of a candidate cell TA, the source cell/DU knows whether it needs to trigger another PDCCH order.

Observation 1: The source cell/DU has to know whether the TA is valid or not (e.g., by maintaining NW side timer), in order to know whether to include the beam indication in the LTM MAC CE and to know whether there is a need to trigger (another) PDCCH order early RACH.

Since the network side anyway needs to know the validity of candidate cell TA, in LTM MAC CE, the network can indicate a valid TA to the UE or indicate whether a TA is still valid or not. Therefore, the UE does not need to maintain a TA timer for candidate cells, which simplifies UE’s implementation. Upon receiving the TA information indicated in LTM MAC CE, the UE can apply the TA value and start the TA timer for the target cell upon cell switch.

Observation 2: The UE applies the TA value indicated in the LTM MAC CE and start the corresponding TA timer for the target cell upon cell switch as current spec (no need start any TA timer for candidate cell upon early TA acquisition).

In subsequent LTM case, if it is intra-DU case, the DU has the TA value for a previous cell and other candidate cells if early RACH was performed for the candidate cells. And the DU knows whether those TAs are still valid or not, e.g., by maintaining a network side timer. So if we consider the ping-pong case when the UE switches back to a previous cell within a short period of time, the DU can indicate that TA to the UE and a RACH-less cell switch is performed. If it is inter-DU case, another PDCCH order could be triggered for early TA acquisition. Or we can allow the source DU to notify the TA values to the target DU, then the target DU can switch the UE back to a previous cell by RACH-less as well.

Observation 3: For subsequent LTM: in intra-DU case, the network always knows whether UE has the valid TA for candidate cells; in inter-DU case, the network can trigger another PDCCH order for early RACH at the new DU. FFS if RAN3 supports the F1AP signalling to forward the TA value/timer information from one DU to another DU (no RAN2 impact is expected).

Proposal 2: For PDCCH ordered early TA acquisition, there is no need for UE to maintain the TA timer for candidate cells (i.e. it is NW implementation to determine the TA validity).

For scenario of PDCCH ordered early TA acquisition with RAR from serving cell, the MAC procedure is quite similar to the legacy PDCCH ordered RACH to serving SCell, where the preamble is to SCell and RAR is received from PCell. In this early TA acquisition case, UE transmits preamble to a candidate cell and RAR is received from PCell. Therefore, current spec can be largely reused.

Proposal 3: For the RAR from serving cell option in PDCCH ordered early TA acquisition, reuse the similar MAC layer procedure as the “PDCCH ordered RACH to serving SCell” in the current specification, by transmitting the preamble to the candidate cell instead of serving SCell.

Regarding the RAR from the serving cell method, if it is a real RAR MAC PDU scheduled by PDCCH addressed by RA-RNTI, RNTI collision issue may occur. Because the RA-RNTI is calculated by the candidate cell configuration, and this RA-RNTI value is possible to be the same with a source cell UE’s C-RNTI. This collision makes some UEs receive wrong message.

Observation 4: In the RAR from the serving cell method, it is better to avoid using RA-RNTI, since it may collide with C-RNTI in the serving cell.

Since the UE which performs early RACH is already in RRC connected mode and it has C-RNTI, we can use PDCCH addressed by C-RNTI to send this so-called “RAR” to provide TA values to the UE. In this case, this so-called “RAR” can use a format which is not a real RAR MAC PDU, e.g., it can be a MAC CE. In Rel-16 two-step RACH, when the UE is in RRC connected mode, the MsgB (RAR) can be a PDCCH addressed by C-RNTI. This PDCCH schedules an Absolute Timing Advance Command MAC CE. The Rel-18 early RACH is similar with the above scenario, because the UE is already in RRC connected mode and the UE expects to receive a TA. Therefore, we propose the “RAR” in the early RACH method is new MAC CE, with exact same format as legacy Absolute Timing Advance Command MAC CE, scheduled by PDCCH addressed by C-RNTI.

Observation 5: Similar to the 2-Step RACH, the MsgB (RAR) can be the Absolute Timing Advance Command MAC CE, scheduled by PDCCH addressed by C-RNTI.

Proposal 4: In the PDCCH ordered early TA acquisition with “RAR” from the serving cell method, RAN2 considers this “RAR” as a new MAC CE (including TA value) addressed by C-RNTI, rather than using the MAC RAR PDU addressed by RA-RNTI.

2.1.3 TA validity indication in the LTM MAC CE

In the PDCCH ordered RACH without RAR method, there is no doubt that the explicit TA value is provided in the LTM MAC CE.

Observation 6: Explicit TA value will be included in the LTM MAC CE in PDCCH ordered early TA acquisition without RAR scenario.

However, there are cases when the UE has the TA value before receiving LTM MAC CE, e.g., in the early RACH with RAR case and CA cell swap case. Regarding the early RACH with RAR case, as we discussed in previous sections, there is no need for UE to maintain the TA timer for candidate cells. So we prefer to let the network to indicate the validity of the TA in LTM MAC CE and this indication can be also regarded as a RACH-less or RACH-based cell switch indication. Since we try to make a common design for different cases, we prefer to use the same principle for CA cell swap case, i.e., the network indicates the TA validity (RACH-based or RACH-less cell switch) in LTM MAC CE.

Proposal 5a: In the CA cell swap scenario (i.e. when the target PCell is a source SCell) and in the PDCCH ordered early TA acquisition with RAR scenario, the LTM cell switch MAC CE indicates whether the TA is valid or not.

As for the detailed content of the LTM MAC CE, different options can be discussed in RAN2.

Proposal 5b: Discuss following detailed options for “CA cell swap” scenario and “PDCCH ordered early RACH with RAR” scenario:

-    Option 1: the LTM cell switch MAC CE indicates (with 1 bit) whether the TA is still valid (the UE to determine to use TA value of current serving cell/TAG or to use TA value provided from RAR).

-    Option 2: the LTM cell switch MAC CE explicitly provides the TA value to perform RACH-less cell switch.

2.1.4 RACH-less LTM determination at UE

Since TA information is one necessary component in the LTM MAC CE for RACH-less, it can be used as a RACH-less or RACH-based cell switch indicator.

Proposal 6: The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value or indicates that the TA is valid.

2.1.5 Coordination on the early RACH resources among DUs

In RAN2 #121bis-e meeting, RAN2 agreed that the CFRA resources for early TA acquisition can be shared among multiple UEs, see the extracted agreements below:

- From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

Since the CFRA resources may be shared by multiple UEs, it is possible that two UEs in the same cell transmit the same preamble to the same candidate cell, but in different ROs. This scenario is shown in Figure 1 below. In this case, the candidate cell will calculate two TA values without knowing which TA is for which UE. In inter-DU case, the candidate DU provides two TA values to the source DU via the CU, and the source DU needs to know which TA is for which UE. Therefore, additional information is needed along with the TA value. Since the source DU will allocate different RACH resources to different UEs, either different preamble index or different RO, the additional information along with the TA value can be the preamble index and the RO information, so that the source DU know which TA is for which UE.

Figure 1. scenario 1: two UEs in the same source cell send preambles to the same candidate cell

When candidate DU (e.g., DU2) manages multiple candidate cells (e.g., cell2 and cell3) for UEs in cell1 in DU1, each candidate cell provides its own CFRA resources for early RACH to UEs in cell1. It is possible that UE1 uses preamble index #1 in RO #1 of cell2, and the UE2 uses preamble index #1 in RO #1 of cell3. The DU2 will successfully calculate two TAs because cell2 and cell3 have different RACH configurations. This scenario is shown in the Figure 2 below.

Figure 2. scenario 2: two UEs in the same source cell send preambles to the different candidate cells

In this case, preamble index and RO information is not enough for the source DU to distinguish which TA is for which UE. Additional information related to candidate cell ID is needed.

Another scenario for inter-DU case is: a candidate cell allocates CFRA resources for early RACH to multiple source cells, which is illustrated in Figure 3 below. When UE1 and UE2 send preambles to cell3/DU3, and the cell3/DU3 calculates two TAs and send them to the CU, then how does the CU know which TA is for which source cell/DU. The CFRA resources of cell3 should be different for DU1 and DU2, otherwise collision may occur when UE1 and UE2 use the same preamble index. Once the DU3 receive a preamble in cell3, it knows this preamble was allocated to which source DU, then it can identify the source DU. Then the DU3 will notify the CU to forward the TA to which source DU. The detailed F1 signalling is up to RAN3.

Figure 3. scenario 3: two UEs in the different source cell send preambles to the same candidate cell

Proposal 7a: RAN2 assumes that the early RACH configuration/resource of candidate cells is source DU specific, so that candidate DU can identify the source DU triggering the early RACH.

Proposal 7b: The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g., preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE.

When we consider subsequent LTM, each candidate cell could become the source cell later. So the UE needs to be provided with paired early RACH resources, e.g., CFRA resource #1 {source cells in DU1, candidate cell3}, CFRA resource #2 {source cells in DU2, candidate cell3}, etc. In this case, the UE can get valid CFRA resources for early RACH in subsequent LTM.

Proposal 7c: For subsequent LTM purpose, the early RACH configuration/resources configured to a UE should be: the list of RACH configurations, with each entry corresponding to one pair of {source cell(s) of one DU, candidate cell(s)}. FFS on the ASN.1 details.

LTM completion

In the last meeting, we have agreed that the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data for RACH-less LTM. However, companies have different proposals on how the UE determines the NW acknowledge of its first UL data. Based on the email discussion [1] in the last meeting, there are 3 options on the table:

- Option 1: RLC ACK of RRCReconfigurationComplete message

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution Identify MAC CE

It should be noticed that RACH-less procedure is also discussed and supported in the Rel-18 NR NTN enhancements, and option 3 (i.e. UE Contention Resolution Identity MAC CE) was agreed to be the confirmation of RACH-less HO completion at least for pre-allocated grant in the last meeting.

Agreements online: - At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

To simplify the UE/network implementation and reduce specification efforts, it is expected to unify the methods of RACH-less procedure as much as possible among different topics. Therefore, option 3 should be also supported to be the confirmation of the first UL data in RACH-less LTM cell switch procedure, reusing the method of Rel-18 NR NTN RACH-less and LTE RACH-less.

Moreover, option 1 (i.e. RLC ACK of RRCReconfigurationComplete message) may cause T304 timer expire, since RLC ACK is not a “real-time” feedback and RLC status report is only initiated/triggered upon conditions (e.g. t-Reassembly expiring) are met. For option 2 (i.e. C-RNTI addressed PDCCH), there may be the case UE without any new data to transmit/schedule, so option 2 may cause some delay also. Although the network can always schedule UE for transmitting padding when there is no more new data, it may be clear to specify a certain MAC CE as the confirmation of the first UL data of RACH-less procedure.

Proposal 8: For LTM completion, follow the NTN conclusion for UE to determine the successful reception of its first UL data by the network, i.e. use option 3.

- Option 1: RLC ACK of RRCReconfigurationComplete message

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution identify MAC CE

RAN3 LS on inter-DU

In the last meeting, we discussed the following 2 approaches included in RAN3 LS in the email discussion [1].

Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

It seems that most companies agreed the approach 1 as the baseline and the current RAN2 assumption. Several companies clarified that approach 2 is beneficial to obtain dynamic information included in LTM MAC CE command, e.g. CFRA resources, BWP ID and UL grant of target cell. It was also mentioned that approach 2 can be allowed by the network implementation together with the approach 1.

Considering the dynamic information above (e.g. CFRA resources, BWP ID and UL grant of target cell) are not agreed yet in RAN2 to be included in LTM MAC CE command, the necessity of approach 2 is undetermined. Assuming any dynamic information included in LTM MAC CE command is agreed to be provided by target gNB-DU to the serving gNB-DU before triggering LTM cell switch command, 3 rounds of F1AP coordination may be needed before LTM execution. The first round is performed in the LTM preparation phase, to coordinate the configurations of candidate cells which are included in RRC pre-configuration message. The second round is for early TA acquisition if early UL sync is triggered by the network and RAR is configured, to transmit the TA value of candidate cell from candidate gNB-DU to the serving gNB-DU. The third round is just before the triggering of LTM MAC CE command, to obtain the dynamic information of target cell, which may introduce extra LTM delay to trigger LTM MAC CE command. From our perspective, it is better to avoid the complex F1AP coordination either in the network implementation and RAN3 specifications as much as possible. RAN2 should determine whether there indeed exists dynamic information requires F1AP coordination just before triggering LTM MAC CE command first, and then determine whether approach 2 is needed, before providing reply LS to RAN3.

Observation 7: Approach 2 requires three rounds of F1AP coordination: 1st round for the pre-configuration, 2nd round for the TA value provision and 3rd round for dynamic information (if any).

Proposal 9: Consider approach 1 in the RAN3 LS as the baseline and discuss whether there is any information to be provided in the LTM command MAC CE which requires approach 2, before reply to RAN3.

T304 timer

In the last meeting, we have agreed to control the success/failure of LTM procedure by a RRC supervisor timer and related behaviors were also agreed.

- Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - LTM supervisor timer is RRC layer timer.

One open issue is whether the RRC supervisor timer is the legacy T304 timer or a newly introduced timer (e.g. T304-like timer). As both RACH-based LTM and RACH-less LTM are supported, some companies mentioned whether a separate/shorter timer is needed for RACH-less LTM because of the shorter interruption time. In LTE RACH-less, RACH-based LTM and RACH-less LTM are both supported and only one T304 timer is used. So it is suggested to reuse the legacy T304 timer for both RACH-based and RACH-less LTM, like the way of LTE RACH-less.

Proposal 10: Reuse the T304 timer for LTM (for both RACH-based and RACH-less based cell switch).

Scenario

It can been seen that there are several open issues left in the last meeting, which are all related to SCG LTM procedure.

- In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) - Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - LTM supervisor timer is RRC layer timer. - At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

RAN2 should focus on the issues of MCG LTM first, since MCG change is more critical to traffic continuity and UE experience. To simply the discussion, RAN2 should confirm to only support PSCell change without MN involvement, i.e. the candidate configuration RRCReconfiguration message is one SN message in NR-DC LTM. We can reuse the conclusions for MCG LTM to SCG LTM as much as possible, to unify the methods of whole LTM procedure. RAN2 can have the discussion specific to SCG LTM after we get processes on MCG LTM, if time allowed.

Proposal 11: Deprioritize discussions specific to SCG LTM.

Conclusion

This contribution makes the following proposals:

Observation 1: The source cell/DU has to know whether the TA is valid or not (e.g., by maintaining NW side timer), in order to know whether to include the beam indication in the LTM MAC CE and to know whether there is a need to trigger (another) PDCCH order early RACH.

Observation 2: The UE applies the TA value indicated in the LTM MAC CE and start the corresponding TA timer for the target cell upon cell switch as current spec (no need start any TA timer for candidate cell upon early TA acquisition).

Observation 3: For subsequent LTM: in intra-DU case, the network always knows whether UE has the valid TA for candidate cells; in inter-DU case, the network can trigger another PDCCH order for early RACH at the new DU. FFS if RAN3 supports the F1AP signalling to forward the TA value/timer information from one DU to another DU (no RAN2 impact is expected).

Observation 4: In the RAR from the serving cell method, it is better to avoid using RA-RNTI, since it may collide with C-RNTI in the serving cell.

Observation 5: Similar to the 2-Step RACH, the MsgB (RAR) can be the Absolute Timing Advance Command MAC CE, scheduled by PDCCH addressed by C-RNTI.

Observation 6: Explicit TA value will be included in the LTM MAC CE in PDCCH ordered early TA acquisition without RAR scenario.

Observation 7: Approach 2 requires three rounds of F1AP coordination: 1st round for the pre-configuration, 2nd round for the TA value provision and 3rd round for dynamic information (if any).

RACH-less: UL grant/beam

Proposal 1a: If the configured grant solution is used for (inter-DU) RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell), similar to the CG-SDT configuration for beam association.

Proposal 1b: If the dynamic grant solution is used for (inter-DU) RACH-less LTM, for the first UL data transmission to the target cell:

- the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- upon cell switch decision, the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

Early TA acquisition procedure

Proposal 2: For PDCCH ordered early TA acquisition, there is no need for UE to maintain the TA timer for candidate cells (i.e. it is NW implementation to determine the TA validity).

Proposal 3: For the RAR from serving cell option in PDCCH ordered early TA acquisition, reuse the similar MAC layer procedure as the “PDCCH ordered RACH to serving SCell” in the current specification, by transmitting the preamble to the candidate cell instead of serving SCell.

Proposal 4: In the PDCCH ordered early TA acquisition with “RAR” from the serving cell method, RAN2 considers this “RAR” as a new MAC CE (including TA value) addressed by C-RNTI, rather than using the MAC RAR PDU addressed by RA-RNTI.

TA validity indication in the LTM MAC CE

Proposal 5a: In the CA cell swap scenario (i.e. when the target PCell is a source SCell) and in the PDCCH ordered early TA acquisition with RAR scenario, the LTM cell switch MAC CE indicates whether the TA is valid or not.

Proposal 5b: Discuss following detailed options for “CA cell swap” scenario and “PDCCH ordered early RACH with RAR” scenario:

-    Option 1: the LTM cell switch MAC CE indicates (with 1 bit) whether the TA is still valid (the UE to determine to use the TA value of current serving cell/TAG or to use the TA value provided from RAR).

-    Option 2: the LTM cell switch MAC CE explicitly provides the TA value to perform RACH-less cell switch.

RACH-less LTM determination at UE

Proposal 6: The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value or indicates that the TA is valid.

Coordination on the early RACH resources among DUs

Proposal 7a: RAN2 assumes that the early RACH configuration/resource of candidate cells is source DU specific, so that candidate DU can identify the source DU triggering the early RACH.

Proposal 7b: The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g., preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE.

Proposal 7c: For subsequent LTM purpose, the early RACH configuration/resources configured to a UE should be: the list of RACH configurations, with each entry corresponding to one pair of {source cell(s) of one DU, candidate cell(s)}. FFS on the ASN.1 details.

LTM completion

Proposal 8: For LTM completion, follow the NTN conclusion for UE to determine the successful reception of its first UL data by the network, i.e. use option 3.

- Option 1: RLC ACK of RRCReconfigurationComplete message

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution identify MAC CE

RAN3 LS on inter-DU

Proposal 9: Consider approach 1 as the baseline and discuss whether there is any information to be provided to the UE in the LTM command MAC CE which requires approach, before reply to RAN3.

T304 timer

Proposal 10: Reuse the T304 timer for LTM (for both RACH-based and RACH-less based cell switch).

Scenario

Proposal 11: Deprioritize the discussion which is specific to SCG LTM.

Reference

- R2-2304214 Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei)  Huawei, HiSilicon