3GPP TSG-RAN WG2 Meeting #122 	R2-2305563

Incheon, 22nd – 26th May, 2023

Agenda Item:	7.2.3

Source:	Spreadtrum Communications

Title:	Discussion on RAT-dependent integrity

Document for:	Discussion and Decision

Introduction

In RAN2 #120, #121 and #121-bis meeting, the following agreements are reached by RAN2:

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

Agreements:

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signalling from Rel-17.

Agreement:

LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

Agreement:

LS to RAN1 to include the question of whether RAN1 identify a need for a DNU flag for measurements.

Agreement:

For RAT-dependent integrity, the PL calculation is performed by the entity which also performs the position calculation for a location process.

Agreements:

For UE-based integrity, the integrity parameters of error sources for RAT-dependent integrity are included in assistance data.

LPP Request/Provide Assistance Data are reused for retrieving the integrity parameters to the UE from the LMF.  The request is per positioning method (as in legacy operation) and the provided integrity parameters are as appropriate for the selected positioning method.

Use of posSIBs for integrity parameters is not excluded.

Working assumption:

For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Agreement:

Capture the stage 2 impact for RAT-dependent integrity in section 7 of 38.305.  Initial running CR to be seen at next meeting, using R2-2302504 and R2-2303682 as baseline.

In this contribution, we discuss the question of the methodologies and procedures of RAT-dependent positioning and express our opinions.

Discussion

In RAN2 #120 [1] meeting, the following agreement is achieved:

Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Signalling details and relation to error sources can be determined in normative work. FFS which positioning methods are affected based on the progress in RAN1.

In Rel-17 GNSS integrity, Providing Assistance Data without the Integrity Service Alert IE or Real Time Integrity IEs is interpreted as a DNU=FALSE condition. And we can see that DNU flag in A-GNSS positioning integrity is shown in two parts. GNSS-Integrity-ServiceAlert is provided in GNSS-CommonAssistData IE and indicates the ionosphere and troposphere corrections can be used for integrity or not. GNSS-RealTimeIntegrity is provided in GNSS-GenericAssistData IE, which is associated with specific GNSS constellation and indicated the specific satellites can be used for integrity or not. This is specified in TS 37.355.

#### GNSS-Integrity-ServiceAlert The IE GNSS-Integrity-ServiceAlert is used by the location server to indicate whether the corresponding assistance data can be used for integrity related applications. -- ASN1START GNSS-Integrity-ServiceAlert-r17 ::= SEQUENCE { ionosphereDoNotUse-r17 BOOLEAN, troposphereDoNotUse-r17 BOOLEAN, ... } -- ASN1STOP

NOTE 1: If GNSS integrity assistance data are provided (i.e., any of GNSS-Integrity-ServiceParameters, GNSS-Integrity-ServiceAlert, ORBIT-IntegrityParameters, SSR-IntegrityOrbitBounds, CLOCK-IntegrityParameters, SSR-IntegrityClockBounds, SSR-IntegrityCodeBiasBounds, SSR-IntegrityPhaseBiasBounds, STEC-IntegrityParameters, STEC-IntegrityErrorBounds, SSR-GriddedCorrectionIntegrityParameters, TropoDelayIntegrityErrorBounds) the following interpretation of the IE GNSS-RealTimeIntegrity applies: - Absence of the IE GNSS-RealTimeIntegrity indicates DNU=FALSE according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40] for all GNSS satellites for which integrity assistance data are provided. - Presence of the IE GNSS-RealTimeIntegrity for a GNSS satellite and signal combination indicates DNU=TRUE for this GNSS satellite and signal combination according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40].

And for RAT-dependent integrity, DNU can be used in DL assistance data, which means that the concerned assistance data cannot be used for integrity calculation but may be used for positioning. And in SI phase, RAN 1 has agreed that for UE-based integrity, two types’ error sources including TRP location, inter-TRP synchronization will be provided to UE via assistance data from LMF. In order to indicate to UE which assistance data information cannot be used for integrity calculation, a DNU flag can be introduced. As for the configuration, DNU flags can be configured per frequency layer, per TRP, per resource set or per resource. Considering configuration overhead and flexibility, DNU flag should be configured with each TRP for the error sources in the assistance data.

Proposal 1: RAN2 to configure DNU flags per TRP per positioning method in assistance data.

In study phase, RAN 1 has identified error sources for timing-based and angle-based positioning methods that affect the integrity of RAT-dependent positioning, which we summarize in table 1 and analyses the specifications impact.

Table 1: Analysis of RAT-dependent error sources and impacted specifications

| mode | method | Error sources | Category of error sources | Spec (Procedure) Impact |
| --- | --- | --- | --- | --- |
| LMF-based | DL-TDOA | RSTD measurement | Timing related measurement in UE | LPP ProvideLocationInformation IE |
|  |  | inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | TRP-related information in TRP | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | UL-TDOA | RTOA measurement | Timing related measurement | NRPPa |
|  |  | inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | TRP-related information | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | Multi-RTT | UE Rx-Tx time difference measurement | Timing related measurement | LPP ProvideLocationInformation IE |
|  |  | gNB Rx-Tx time difference measurement | Timing related measurement | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  | UL-AoA | Angle of arrival measurement | Angle related measurement | NRPPa |
|  |  | TRP location | TRP-related information | NRPPa |
|  |  | ARP location (e.g., ARPLocationInformation in TS 38.455) | TRP-related information | NRPPa |
|  | DL-AoD | DL PRS RSRP/RSRPP measurement | measurement | LPP ProvideLocationInformation IE |
|  |  | TRP location | TRP-related information | NRPPa |
| UE-based | DL-TDOA | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) | Assistance data | LPP provideAssistanceData IE |
|  |  | Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355) | Assistance data | LPP provideAssistanceData IE |
|  | DL-AoD | TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) | Assistance data | LPP provideAssistanceData IE |

Thus, in order to obtain positioning integrity calculation estimation, error sources related information needs to be delivered between UE, network and LMF as shown in the table 1, which will modify the LPP and NRPPa protocols.

In addition to error sources related information transmission, UE also need to send capability information to LMF on integrity for UE-based and LMF-based integrity. In Rel-17, we introduced GNSS positioning integrity for UE. And for the RAT-dependent positioning integrity, we can follow the Rel-17 to define the positioning integrity capabilities for each positioning method, which includes error source receiving, error source reporting and positioning integrity result reporting.

Proposal 2: To support RAT-dependent positioning integrity, UE sends positioning integrity capabilities for each RAT-dependent positioning method to LMF, which includes error source receiving, error source reporting and positioning integrity result reporting.

Agreements:

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

In RAN2 #121-bis agreements, determining TRP related error sources is left to deployment and implementation. For UE-based integrity, in order to calculate the integrity, the LMF needs to send the TRP-location, inter-TRP synchronization error distribution related information to the UE through the LPP provideAssistanceData IE.

Proposal 3: For UE-based positioning integrity mode, the following signalling procedure are supported:

- LPP provideAssistanceData signalling to deliver the error of related assistance data from LMF to UE, which at least includes:

  - TRP-location error source statistics information

  - Inter-TRP synchronization error source statistics information

In last RAN2 meeting, the following agreements are achieved.

For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signalling from Rel-17.

For UE-based integrity, the PL and/or its corresponding TIR are provided to LMF as legacy. And for obtain the PL and TIR, UE can transmit a request to request RAT-dependent integrity results by reusing the the TargetIntegrityRisk in commonIEsRequestLocationInformation.

Proposal 4: For UE-based integrity, reuse the TargetIntegrityRisk in commonIEsRequestLocationInformation to request RAT-dependent integrity results.

For UE-based integrity, as last RAN2 meeting agreement agreed, the PL and/or TIR are provided to LMF, which is mode 1 report mode.

For mode 2, the integrity computing entity calculates the PL. Then, the integrity computing entity further compares the calculated PL with the given AL to determine if the positioning system is still available to offer trustable position estimation. Thus, the integrity computing entity may only have to report a binary flag (0 and 1) to indicate whether the positioning system is available or not. Thus, the AL needs to transfer to UE to comparing the relationship between the calculated PL and AL.

And RAN2 has discussed many issues of reporting mode 2 in GNSS integrity, such as whether should include an indicator to tell the UE either reporting mode 1 or reporting mode 2? Whether allows the UE to raise the flag indicating whether there is an integrity risk? And considering mode 2 can be useful in simple APP implementation where the application requires only information on whether integrity performance is met with respect to the KPIs. Thus, in Rel-18, in order to support the positioning integrity in various scenarios, mode 2 integrity result reporting scan be considered for RAT-dependent positioning.

Proposal 5: Mode 2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 positioning integrity techniques. And the proposals are given as follows:

Proposal 1: RAN2 to configure DNU flags per TRP per positioning method in assistance data.

Proposal 2: To support RAT-dependent positioning integrity, UE sends positioning integrity capabilities for each RAT-dependent positioning method to LMF, which includes error source receiving, error source reporting and positioning integrity result reporting.

Proposal 3: For UE-based positioning integrity mode, the following signalling procedure are supported:

- LPP provideAssistanceData signalling to deliver the error of related assistance data from LMF to UE, which at least includes:

  - TRP-location error source statistics information

  - Inter-TRP synchronization error source statistics information

Proposal 4: For UE-based integrity, reuse the TargetIntegrityRisk in commonIEsRequestLocationInformation to request RAT-dependent integrity results.

Proposal 5: Mode 2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

References

[1] Report of 3GPP TSG RAN WG2 meeting #120, 2022-11.