3GPP TSG-RAN WG2 Meeting #122											   R2-2305575

Incheon, Korea, May 22-26, 2023

Source: 	Xiaomi

Title:  	Discussion on RACH-less LTM

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

# Introduction

According to the recent RAN1/2/3 agreements as given in the Annex, the RACH-less solution is targeting at the early TA acquisition for the candidate cell before the cell switch. In this contribution, we provide our understandings on the RAN2 specification impacts by consolidating the RACH-less procedures in RAN1/2/3.

# Discussion

## Early TA acquisition

The solutions for acquiring the TA before cell switch include the follows:

- Alt 1: PDCCH ordered-RACH without RAR

- Alt 2: PDCCH ordered-RACH with RAR and the RAR is received from the serving cell

For Alt 1, we think that the legacy RACH procedure can be reused. According to the current MAC procedure as quoted below, when the UE is only allowed to transmit PUSCH to the target cell when the TAT for the target cell is running.

38.321: ## 5.2 Maintenance of Uplink Time Alignment … The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer is not running.

Observation 1: The legacy MAC procedure requires the UE to have a running TAT so as to transmit PUSCH to a serving cell.

To align with the legacy TA maintenance procedure, we think that the UE needs to use the TAT for validate the TA of the candidate cell, so that the UE can send its first PUSCH to the target cell when the TAT for the target cell is running after the reception of the cell switch MAC CE. For Alt 1, the TA is received via the RACH RAR, as the legacy RACH. Then the TA is not always required in the cell switch MAC CE for Alt 1. For Alt 2, the TA of the candidate cell can be received via the cell switch MAC CE, according to the RAN1 agreement.

Proposal 1: For both RACH with RAR and RACH without RAR, the UE starts the TAT for the candidate cell when the TA for the candidate cell is received.

As the legacy TA maintenance procedure, if the early acquired TA for Alt 1 is not valid (i.e. TAT for the candidate cell is not running), the UE shall not trigger RACH-less LTM since the PUSCH is not allowed to be transmitted to the target cell.

Proposal 2: If the TAT for the target cell is not running, the UE does not trigger RACH-less LTM.

For Alt 2, RAN1 agreed that the power ramping for PRACH transmission is “determined from PDCCH order”, and whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order. We think that RAN2 specification for the power ramping procedure of the RACH without RAR should be updated based on the latest RAN1 agreement.

Proposal 3: For RACH without RAR, PREAMBLE_POWER_RAMPING_COUNTER is set to “0” when PDCCH order indicates a PRACH initial transmission.

Proposal 4: For RACH without RAR, PREAMBLE_POWER_RAMPING_COUNTER increases by 1 when PDCCH order indicates a PRACH retransmission.

As each preamble transmission (including both initial transmission and retransmission) of the RACH without RAR is triggered by PDCCH order, then there is no need to count the number of preamble transmission, as no RACH failure will be triggered by the UE.

Proposal 5: For RACH without RAR, PREAMBLE_TRANSMISSION_COUNTER is not needed.

## Uplink grant and target confirmation for RACH-less

When the TA is acquired by the UE before the cell switch, the UE can trigger UL PUSCH transmission (i.e. data transmission) directly to the target cell. To trigger UL PUSCH transmission, the gNB needs to provide the UL grant for the target cell. Then we could have the following solutions:

- Dynamic grant (e.g. provided via cell switch MAC CE)

- Configured grant type 1 configured via RRC

- Configured grant type 2 (e.g. configured via RRC and activated via cell switch MAC CE)

According to the LTE RACH-less solution, “Configured grant type 1” is supported to avoid unnecessary activation. Since Configured grant type 1 is also supported for the CG-SDT procedure which is a RACH-less solution for SDT, we think that RAN2 can reuse the same CG type-1 configuration as SDT.

Observation 2: The CG-SDT procedure is a RACH-less solution for SDT.

Observation 3: The CG type-1 configuration for SDT can be reused for the provisioning of uplink grant for RACH-less.

Regarding dynamic grant provided via cell switch MAC CE, we think that this solution may have problem on the UL retransmission, and need more discussion on the timing of the dynamic grant in RAN1 and RAN3. RAN2 can wait for the inputs from other working groups. For CG type-2, as the uplink grant is also provided in the cell switch MAC CE. It would have the same issues on the transmission timing as the dynamic grant solution. Thus, we think that CG type-1 as used for SDT can be considered as the baseline for RACH-less LTM, and other solutions can be FFS and wait for the more inputs from RAN1/RAN3.

Proposal 6: The CG type-1 configuration as used for SDT is reused for providing the initial uplink grant for RACH-less LTM.

Regarding the confirmation from the network side, we think that the LTE solution can be reused. The contention resolution MAC CE is used to terminate the initial access procedure to the target cell (i.e. to stop t304).

Proposal 7: The reception of UE Contention Resolution Identity MAC CE terminates the RACH-less LTM procedure to the target cell (i.e. stops t304).

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: The legacy MAC procedure requires the UE to have a running TAT so as to transmit PUSCH to a serving cell.

Observation 2: The CG-SDT procedure is a RACH-less solution for SDT.

Observation 3: The CG type-1 configuration for SDT can be reused for the provisioning of uplink grant for RACH-less.

Proposal 1: For both RACH with RAR and RACH without RAR, the UE starts the TAT for the candidate cell when the TA for the candidate cell is received.

Proposal 2: If the TAT for the target cell is not running, the UE does not trigger RACH-less LTM.

Proposal 3: For RACH without RAR, PREAMBLE_POWER_RAMPING_COUNTER is set to “0” when PDCCH order indicates a PRACH initial transmission.

Proposal 4: For RACH without RAR, PREAMBLE_POWER_RAMPING_COUNTER increases by 1 when PDCCH order indicates a PRACH retransmission.

Proposal 5: For RACH without RAR, PREAMBLE_TRANSMISSION_COUNTER is not needed.

Proposal 6: The CG type-1 configuration as used for SDT is reused for providing the initial uplink grant for RACH-less LTM.

Proposal 7: The reception of UE Contention Resolution Identity MAC CE terminates the RACH-less LTM procedure to the target cell (i.e. stops t304).

# Reference

[1] R1-2304276, LSin from RAN1, “LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM”.

[2] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

# Annex

## RAN1 agreements for RACH-less LTM

The RAN1#111 meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.  The PDCCH order is only triggered by source cell  FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.  Note: any other RACH-based solutions are for discussion separately Agreement (Made in RAN1#110b-e) Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2) Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported · Introduce indication of candidate cell and/or RO of candidate cell in DCI · configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order · FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study • Alt 1: RAR is needed • Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command • Alt 3: whether RAR is needed can be configured Agreement · TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. · same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

The RAN1#112 meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. § The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated § If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order § If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell u if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR § FFS: signaling for configuration/indication of whether RAR needs to be received § UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM § Send LS to RAN2 and RAN3 to check the feasibility about this agreement § Note: Definition of candidate cells is up to RAN2 Agreement • For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. • Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) • Alt 2: UE autonomous Re-transmission of PRACH is allowed, · The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. • The maximum number of TA values memorized by UE is a UE capability • FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured • Whether gap between the DCI and PRACH longer than timeline defined in spec is needed • Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. • Corresponding UE capability is to be introduced to support UE-based TA measurement • For a UE reports support of this capability, configuration of UE-based TA measurement is supported • FFS: other impacts on RAN1 spec Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. § The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

The RAN1#112bis meeting agreements regarding “Timing advance management to reduce latency” are quoted as follows:

- Time gap between a PDCCH order and the corresponding PRACH transmission RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. RAN1 believes that this will require that the time gap is increased at least for the following scenario - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell RAN1 relies on RAN4: - to verify the need for the above additional latency and, if so, the corresponding value is needed. - to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH - to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed Potential RAN1 spec update will be based on RAN4’s feedback. - RAN1 #112bis-e agreements Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable) Agreement - RRC parameter ss-PBCH-BlockPower for candidate cells is included in the LTM configuration. - UE needs the parameter to (at least) perform RACH towards candidate cells - Note: how to capture this parameter and RRC structure are up to RAN2 Agreement For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax. Agreement When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. Agreement When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues Agreement For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability Agreement For PDCCH-order based PRACH for candidate cell study the following issues: - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission Agreement For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism) - power ramping-up value is configured - else, the power should be determined by open-loop power control

## RAN2 agreements for RACH-less LTM

- RAN2#121bis-e meeting agreements:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - - Value 0, - - Value indicating that the UE shall apply the TA of one source cell.

## RAN3 agreements for RACH-less LTM

RAN3#119bis-e meeting agreements:

RAN3 would like to provide the following feedback for RAN1’s question: RAN1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B. From RAN3 perspective, all options are feasible. The potential impacts of the following options on specifications are being taken into consideration by RAN3. - Case 1: with RAR and the RAR is received from the candidate cell - RAN3 will not pursue any standardization work in Rel-18 considering other groups output of this option. - Case 2: with RAR and the RAR is received from the serving cell - RAN3 will further discuss how to specify how the information required for the RAR transmission is signalled from the candidate DU to the source DU through the CU. - Case 3: without RAR - For the RACH-less solution, RAN3 will further discuss how to specify how the TA value(s) is (are) transmitted from the candidate DU to the source DU through the CU.