3GPP TSG-RAN WG2 Meeting #122											   R2-2305582

Incheon, Korea, May 22-26, 2023

Source: 	Xiaomi

Title:  	Remaining issues for TDM solutions

Agenda Item:	7.10.3

Document for:	Discussion and Decision

# Introduction

According to the email discussion for drafting the running RRC CR for IDC, several issues were raised for the TDM solutions (i.e. TDM assistance information and autonomous denial). In this contribution, we provide some clarifications on the remaining issues for TDM solution.

# Discussion

## Autonomous denial

R2-2304331 38.331 running CR for introduction of IDC Xiaomi draftCR Rel-18 38.331 17.4.0 NR_IDC_enh-Core 1> if the CellGroupConfig contains the autonomousDenialParameters: 2> consider itself to be allowed to deny any transmission in a particular UL slot if during the number of slots indicated by autonomousDenialValidity, preceding and including this particular slot, it autonomously denied fewer UL slots than indicated by autonomousDenialSlots within the same cell group; Editor’s Note: FFS whether the UE sums up the denied UL slots together across all CC(s) in the CG and how the UE calculates the number of denied slots when different CCs have different slot length. Editor’s Note: FFS whether the NR autonomous denial configuration and the LTE autonomous denial configuration can be configured simultaneous for EN-DC. Editor’s note: FFS on the UE behaviour when receiving autonomousDenialValidity.. As RAN2 agreed that the autonomous denial configuration is per CG, some companies consider that it is not very clear how the UE counts the UL slots across all CCs in the same CG, while considering the case that different CCs could have different slot length. It seems that if we do not consider the numerology impacts on counting the dropped UL slots, the UE would have to count all UL slot in all CCs. It is also not very clear whether we need to clarify the UE behaivour when the UE sums up the denied UL slots together across all CC(s) in the CG. The rapporter considers that if companies agree with the common UE behaviour as “the UE sums up the denied UL slots together across all CC(s) in the CG”, we could add a NOTE in the specification to avoid some ambiguities. #### Question 3: Do you think that the UE sums up the denied UL slots together across all CC(s) in the CG?

Figure 1: Slot number cross CCs within a CG

According to our understanding, the UE and the gNB needs to have a common understanding on how to count the number of dropped UL slots and the number of slots indicated by autonomousDenialValidity.

Firstly, we think that the UE could drop the UL slot in any CC, since the interference could be caused by the uplink transmission of any CC. It is also possible that the UE drop two UL slots simultaneously in two CCs, as two CCs could also cause different interferences to different frequencies.

Observation 1: The UE could drop the UL slot in any CC, since the interference could be caused by the uplink transmission of any CC.

Observation 2: The UE could drop two UL slots simultaneously in two CCs, as two CCs could cause different interferences to different frequencies.

Secondly, each dropped UL slot should be counted when there are two simultaneous dropped UL slots in two CCs, as this is to have the aligned number of dropped UL slots between the UE and the gNB.

Observation 3: Each dropped UL slot should be counted when there are more than one simultaneous dropped UL slots in multiple CCs, so as to align the number of dropped UL slots between the UE and the gNB.

Proposal 1: Each dropped UL slot is counted once.

Regarding the number of slots indicated by autonomousDenialValidity, the UE needs to count all UL slots across all CCs, if the UE is allowed to drop any UL slot in any CC. Otherwise the gNB is not able to control the overall number of slots which can be dropped across all CCs. As the example shown in Figure 1, the number of slots counted for autonomousDenialValidity is (8+4+2 =14). As such, the UE can drop slot#0 of CC3, slot#1 of CC2 and slot#1 of CC1, when autonomousDenialValidity is set to 14 and autonomousDenialSlots is set to 4.

Observation 4: To control the overall number of slots which can be dropped across all CCs, the UE should count all UL slots across all CCs for the number of slots indicated by autonomousDenialValidity.

Proposal 2: All UL slots across all CCs are counted for the number of slots indicated by autonomousDenialValidity.

As some companies also consider that the UE behaviours for Proposal 1 and Proposal 2 would be already reflected by the current running CR. We consider that the running CR can be updated by capturing a NOTE so avoid misunderstandings.

Text Proposal 1: 1> if the CellGroupConfig contains the autonomousDenialParameters: 2> consider itself to be allowed to deny any transmission in a particular UL slot if during the number of slots indicated by autonomousDenialValidity, preceding and including this particular slot, it autonomously denied fewer UL slots than indicated by autonomousDenialSlots within the same cell group; Note X: Each dropped UL slot is counted once, and the number of slot indicated by autonomousDenialValidity includes all UL slots across all serving cells within the same cell group.

Proposal 3: RAN2 is kindly requested to update the running RRC CR by using Text Proposal 1.

## Reporting of TDM assistance information

According to the LTE specification as quoted below, as the TDM assistance information is only allowed for MCG, the timing referenc for IDC TDM assistanc information uses MCG. 36.331: 2> include Time Domain Multiplexing (TDM) based assistance information, unless idc-HardwareSharingIndication is configured and the UE has no Time Doman Multiplexing based assistance information that could be used to resolve the IDC problems: 3> if the UE has DRX related assistance information that could be used to resolve the IDC problems: 4> include drx-CycleLength, drx-Offset and drx-ActiveTime; 3> else (the UE has desired subframe reservation patterns related assistance information that could be used to resolve the IDC problems): 4> include idc-SubframePatternList; 3> use the MCG as timing reference if TDM based assistance information regarding the SCG is included;. Since the NR TDM assistance information is per CG, the rapporteur considers that we can reuse the LTE solution to use the CG for reporting TDM as the timing reference. The following FFS(s) are quoted from the endorsed CRs given in the reference section. R2-2304331 38.331 running CR for introduction of IDC Xiaomi draftCR Rel-18 38.331 17.4.0 NR_IDC_enh-Core 3> include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems; 3> if idc-TDM-AssistanceConfig is provided from MCG: 4> use the MCG as timing reference; 3> else: 4> use the SCG as timing reference; Editor’s Note: FFS whether the time reference for idc-TDM-AssistanceConfig is needed.. #### Question 7: Do you think that the time reference for idc-TDM-AssistanceConfig is needed, as LTE?

RAN2#121bis-e meeting agreements: - Per CG pattern is supported for MR-DC. SN can configure the UE to report the TDM assistance information directly to SN, either through SRB 1 (if SRB3 is not configured) or SRB 3. - For NR-DC, per CG idc-AssistanceConfigTDM-r18 is introduced to indicate whether TDM assistant information needs to be reported.

According to the discussion procedure as quoted above, the UE needs to report IDC-TDM-Assistance-r18 per CG. Firstly, we think that the timing reference for the IDC-TDM-Assistance-r18 needs to be aligned between the UE and the gNB. Otherwise, the TDM solution may not cover the interference period correctly. In order to have an aligned understandings between the UE and the gNB and also to follow the previous RAN2 agreement, we think that the UE should use the timing reference from the node which provides the idc-TDM-AssistanceConfig. If the UE uses the MCG timing reference also for the SCG, then the SN needs to recalculate the TDM pattern by translating the MCG timing to the SCG timing. From our understanding, this would require some coordination between the MN and the SN, as the SN needs to know the timing of the SFN number and the slot number of the MCG. For EN-DC, we think that more complex coordination is required, as the NR SN would be required to understand the timing of the LTE MCG.

Observation 5: Using the per CG timing for IDC-TDM-Assistance-r18 is aligned with the RAN2 agreements.

Observation 6: Using the MCG timing for IDC-TDM-Assistance-r18 reported to SN would require inter-node coordination, which would cause even more complex coordination for EN-DC.

According to the RAN2 agreement, the SN can also provide the idc-TDM-AssistanceConfig via either SRB1 (of MCG) or SRB3, we think that the UE can know whether the idc-TDM-AssistanceConfig is provided by MN or SN based on the RRC configuration since the idc-TDM-AssistanceConfig provided via SRB1 from SN is included in mrdc-SecondaryCellGroupConfig.

Observation 7: According to the current RRC specification, the UE knows which node (i.e. either MN or SN) provides the idc-TDM-AssistanceConfig.

Thus, we think that the current running RRC CR could be updated to clarify that the timing for idc-TDM-Assistance follows the timing of CG where the idc-TDM-AssistanceConfig is provided.

Proposal 4: The timing for idc-TDM-Assistance follows the timing of CG for which the idc-TDM-AssistanceConfig is provided.

If companies think that this is obvious from the specification as the UE should not use the timing of another CG for TDM reporting, we consider that a NOTE would be helpful for the UE implementation so that we can avoid some error implementation for DC case. An example of TP could be as follows:

Text Proposal 2: 3> include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems; Note X: The idc-TDM-Assistance uses the MCG as timing reference if the idc-TDM-AssistanceConfig is provided for MCG. Otherwise the idc-TDM-Assistance uses the SCG as timing reference.

Proposal 5: RAN2 is kindly requested to update the running RRC CR by using Text Proposal 2.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: The UE could drop the UL slot in any CC, since the interference could be caused by the uplink transmission of any CC.

Observation 2: The UE could drop two UL slots simultaneously in two CCs, as two CCs could cause different interferences to different frequencies.

Observation 3: Each dropped UL slot should be counted when there are more than one simultaneous dropped UL slots in multiple CCs, so as to align the number of dropped UL slots between the UE and the gNB.

Observation 4: To control the overall number of slots which can be dropped across all CCs, the UE should count all UL slots across all CCs for the number of slots indicated by autonomousDenialValidity.

Observation 5: Using the per CG timing for IDC-TDM-Assistance-r18 is aligned with the RAN2 agreements.

Observation 6: Using the MCG timing for IDC-TDM-Assistance-r18 reported to SN would require inter-node coordination, which would cause even more complex coordination for EN-DC.

Observation 7: According to the current RRC specification, the UE knows which node (i.e. either MN or SN) provides the idc-TDM-AssistanceConfig.

Proposal 1: Each dropped UL slot is counted once.

Proposal 2: All UL slots across all CCs are counted for the number of slots indicated by autonomousDenialValidity.

Proposal 3: RAN2 is kindly requested to update the running RRC CR by using Text Proposal 1.

Proposal 4: The timing for idc-TDM-Assistance follows the timing of CG for which the idc-TDM-AssistanceConfig is provided.

Proposal 5: RAN2 is kindly requested to update the running RRC CR by using Text Proposal 2.

# Reference

[1] [Post121][655][IDC] Discussion on Leftover issues for IDC (xiaomi)