3GPP TSG RAN WG2 #122          								   R2-2305588

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.20.2

Source: 	NEC

Title:	Discussion on Two TAs for Multi-TRP

Document for:	Discussion

- Introduction

In RAN2#121 meeting, RAN2 received an LS [1] from RAN1, requesting RAN2 to consider the agreements/conclusions in RAN1 as below.

In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two TAs. The following has been agreed/concluded in RAN1: Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

This contribution provides our considerations on multi-DCI based multi-TRP operation with two TAGs from RAN2 perspective.

- Discussion

In RAN2#121bis-e meeting, RAN2 discussed the LS from RAN1 and formed an LS to RAN1 on issues of 2 TAs [2]. It lists the following questions:

Question 1 on TAG grouping RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs. Q1a: For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs? Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Question 2 on operation Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

In addition to the above questions, we still have a question about the definition of PTAG. If SpCell has 2 TAGs, from RAN2 perspective, the question is how to define the PTAG for this case. The following solutions can be considered to define the PTAG for this case:

- Solution 1: Only one TAG of SpCell is treated as PTAG. Thus, if the TAT of PTAG is timeout, legacy UE behaviours as in TS 38.321[3] can be adopted;

- Solution 2: Two TAGs of SpCell are treated as two PTAGs. Then extra specific effort is required to manage the TAT and the corresponding resources when TATs expire, for example how to manage the resources of one cell, one TAG or all cells when the TAT of one PTAG expires or when TATs of both PTAGs expire.

RAN2 to update the concept of PTAG and related procedures to manage the resources of one cell or one TRP or all cells when TAT expires.

- Conclusion

We have the following proposal:

Proposal 1:	RAN2 to update the concept of PTAG and related procedures to manage the resources of one cell or one TRP or all cells when TAT expires.

- References

- R2-2300021 LS on Multi-DCI Multi-TRP with two TAs

- R2-2304342 LS on 2TA for multi-DCI multi-TRP

[3] TS 38.321 V17.5.0