3GPP TSG-RAN WG2 Meeting #122	R2-2305590

Incheon, South Korea, 22– 26 May 2023	R2-2302791

Agenda item:	7.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	U2U relay (re)selection and Local ID assignment

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#121 and RAN2#121bis U2U relay was discussed, leaving some open issues on ID(s) to be used in the SRAP header. This T-doc discusses these open issues among others and proposes relevant issues for RAN2 to discuss during RAN2#122.

# 2	Discussion

## 2.1	U2U relay (re)selection

RAN2#121 and RAN2#121bis respectively agreed that:

Agreement:

Each Remote UE can trigger Relay reselection based at least on current hop quality.

Agreement:

Each remote UE (source or destination) can trigger relay selection based on the direct link quality.  FFS interaction between discovery and selection.

In SL U2U relays, a U2U Relay UE may need to discover a target End UE proactively or upon a U2U relay discovery solicitation from a source End UE. The discovery of the target End UE by the U2U Relay UE may be based on either Model A or Model B. Thus, if the U2U Relay UE knows which of Model A or Model B the target End UE is using, the Relay UE may be able to perform the discovery of the target End UE in a fast and efficient way by applying the same discovery model as the target End UE is using. If the discovery model is not known by the U2U Relay UE, the U2U Relay UE may need to perform the discovery in both Model A and Model B to minimize the latency of the discovery. The prior knowledge of the discovery model of the target End UE may be provided to the U2U Relay UE by the source End UE at least in case the source End UE and the target End UE in need of U2U relay discovery and (re)selection are coordinated beforehand over an existing direct or indirect SL connection, corresponding to a direct-to-indirect path switch or an indirect-to-indirect path switch. For examples, the source End UE may request the target End UE to use Model A or Model B or both or the target End UE may indicate to the source End UE which discovery model it is using before the U2U relay discovery is initiated for the needed U2U relay (re)selection. Then the source End UE may indicate the discovery model of the target End UE to U2U Relay UE candidates along with its U2U relay discovery solicitation for example.

Observation 1: if the U2U Relay UE knows the discovery model (Model A or Model B) used by the target End UE, the Relay UE can perform the discovery of the target End UE in a fast and efficient way by applying the same discovery model as the target End UE.

Proposal 1: The source End UE may indicate to the U2U Relay UE candidates the discovery model (Model A or Model B) used by the target End UE along with its U2U relay discovery solicitation.

It has been agreed that either the source End UE or the target End UE may be allowed to trigger U2U relay selection in case the U2U relay selection is for a direct-to-indirect path switch. To avoid both the UEs triggering U2U relay selection in parallel as well as to speed up the U2U relay selection, one of the UEs may provide a list of U2U relay UE candidates to the other UE using PC5-RRC signaling over the existing direct path and let the other UE to make the U2U relay selection decision.

Proposal 2: As when triggering the relay selection, any of the UEs may provide a list of U2U Relay UE candidates for the other UE to perform the U2U relay selection for a direct-to-indirect path switch.

For U2U relay reselection for reestablishment of an indirect path between the source End UE and the target End UE, either the source End UE, the target End UE or the current U2U Relay UE may trigger a U2U relay reselection for an indirect-to-indirect path switch, as agreed in RAN2. However, to minimize the service interruption in case the current U2U Relay UE is not able to continue providing relay services to the source End UE and the target End UE for some reason other than PC5 RLF, it is desirable that the current U2U Relay UE continues serving the source End UE and the target End UE for a certain time period for the source End UE and the target End UE to perform the U2U relay reselection. Therefore, the PC5 release from the current U2U Relay UE to either one of the source End UE or the target End UE, as agreed in RAN2, may indicate a time instant when the current U2U Relay UE stops serving the source End UE and the target End UE in term of PC5 slot or system frame number for examples.

Observation 2: To minimise the service interruption, upon triggering the relay reselection or connection reestablishment, the current U2U Relay UE should continue acting as relay for the source End UE and the target End UE for a certain time period while the source and target End UEs perform the U2U relay reselection.

Proposal 3: The trigger from the current U2U Relay UE for U2U relay reselection may include a condition on when the current U2U Relay UE stops serving as a U2U relay between the source End UE and the target End UE.

For U2U relay reselection, either the source End UE or the target End UE may be allowed to make the U2U relay selection decision. The source End UE and the target End UE may also coordinate for performing U2U relay discovery and reselection for an indirect-to-indirect path switch, similarly to that described above for U2U relay discovery and selection for a direct-to-indirect path switch.

Proposal 4: One of the UEs (either the source End UE or the target End UE) may provide a list of U2U Relay UE candidates for the other UE to perform the U2U relay reselection for an indirect-to-indirect path switch.

In addition to support for SL U2U relay selection and reselection for direct-to-indirect and indirect-to-indirect path switches, at least trigger(s) for indirect-to-direct path switch related to SL U2U relay should also be considered and supported, even when service continuity with U2U relay is not supported in Rel-18.

The source End UE and the target End UE need to be enabled to probe one another over SL for direct reachability while communicating over the indirect path. One option with minimized standardization impact is to configure the UEs to transmit some SL message such as SL discovery announcement periodically while communicating over the indirect path.

Observation 3: Enabling the source and target End UEs to probe each other over SL for direct reachability will ensure that relay connections are maintained only when needed.

Proposal 5: RAN2 considers specifying triggers at least for indirect-to-direct path switch related to U2U relay.

## 2.2	Local ID assignment for SRAP in L2 U2U relay

RAN2#120 agreed that:

Agreement:

RAN2 confirms the user plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-1 and control plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-2 of TR 38.836 [2].

Agreement:

RAN2 confirms Remote UE E2E Radio Bearer ID should be included in the adaptation layer in first and second PC5 hop.

Agreement:

RAN2 confirms Remote UE determines the egress RLC channel based on the mapping from the E2E bearer ID to egress RLC channel, for a particular target Remote UE.

Agreements:

FFS if multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the destination remote UE is needed in the first hop (Tx remote UE to relay), at least in case multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the source remote UE is needed in the second hop (relay to Rx remote UE).

FFS if the IDs are different (e.g., source and destination UE IDs) or common (e.g., a local ID for the pair).

FFS whether both UE IDs are included in the header or the relay UE does a mapping.

RAN2#120bis agreed that:

Agreements:

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

In L2 U2U relay, a source End UE may use the same serving U2U Relay UE to connect to different target End UEs or different source End UEs may use the same serving U2U Relay UE to connect to the same target End UE. In the former case, for the direction from the source End UE to the target End UE, multiplexing of data from the same source End UE to different target End UEs at the source End UE on SRAP is possible. For the direction from the target End UE to the source End UE, multiplexing of data to the same source End UE from different target End UEs at the serving U2U Relay UE on SRAP is possible. In the latter case, similar multiplexing of data to/from the same target End UE is possible. These cases may be expected in communications between different UE members of a UE group using a star topology via the same U2U relay UE, as shown in Figure 1 for example.

Figure 1: End UE1, UE2 and UE3 communicating with each other via U2U Relay UE

For the Rel-18 scope that is limited to support of U2U relay for unicast using a single U2U relay UE between a source End UE and a target End UE, ProSe L2 IDs of target End UEs or source End UEs may be used for multiplexing of data in the same RLC channel in the cases described above. That is, L2 ID of individual target End UE may be included in the SRAP header on the first hop between the source End UE and the U2U relay UE and L2 ID of individual source End UE may be included in the SRAP header on the second hop between the U2U relay UE and the target End UE. Thus, different IDs are used on different hops. There is no need to include L2 IDs of both the source and target End UEs in the SRAP header, as the L2 ID that is left out from the SRAP header is already provided in lower layer as either SRC or DST ID for SL transmissions to/from the U2U relay UE.

Though Rel-18 U2U relay is limited to support for E2E unicast connection between the source and target End UEs via a single U2U relay UE, the Rel-18 WI description in RP-221262 has the following note for L2 U2U relay:

Note 1A: This work should take into account the forward compatibility for supporting more than one hop in a later release.

For support of multi-hop U2U relay, using a single L2 ID of either the source or target End UE in the SRAP header is no longer enough for data mapping on hop between U2U relay UEs and in this case L2 IDs of both the source and target End UEs may need to be included in the SRAP header.

Using L2 IDs of source and target End UEs for the SRAP header avoids the need for ID assignment but has high protocol overhead. To reduce this protocol overhead significantly as well as consider the forward compatibility noted above, assigning a short ID locally unique for a hop, denoted as Local ID, for addressing (replacing the need for using) both the source and target End UEs or, that is, the corresponding E2E connection between them, in SRAP is preferable. Thus, the Local ID can be assigned hop-by-hop and can be the same or different on different hops. The assigned Local ID is included in the SRAP header specific to each hop in U2U relay and mapped on the corresponding E2E connection identified by L2 IDs of the source and target End UEs. The same 8-bit length as of UE ID assigned to the Remote UE and included in the SRAP header in Rel-17 L2 U2N relay is sufficient and future-proof for the Local ID.

Proposal 6: The Local ID identifies a pair of the source and target End UEs on SRAP level over each hop in the L2 U2U relay connection.

Proposal 7: The Local ID is unique per hop and specific to each hop.

Proposal 8: RAN2 adopts a single 8-bit Local ID that is included in the SRAP header.

In general, hop-by-hop distributed control is preferable for U2U relay, especially when considering support for multi-hop U2U relay. In this regard, the assignment of the Local ID may also be hop-by-hop, initiated by either the source End UE or the target End UE, e.g., the UE that makes the U2U relay (re)selection decision initiates the Local ID assignment. Thus, either the source End UE or the target End UE may initiate the Local ID assignment by assigning the Local ID for the first hop and then the U2U relay UE may assign the Local ID for the second hop. These Local IDs need not be identical for different hops, but all are mapped on the same pair of the source and target End UEs or, that is, the same E2E connection.

Proposal 9: The assignment of the Local ID is hop-by-hop.

Proposal 10: The assignment of the local ID is initiated by either the source End UE or the target End UE whichever performs the U2U relay (re)selection.

# 3	Conclusion

This paper had the following observations and proposals:

Observation 1: if the U2U Relay UE knows the discovery model (Model A or Model B) used by the target End UE, the Relay UE can perform the discovery of the target End UE in a fast and efficient way by applying the same discovery model as the target End UE.

Proposal 1: The source End UE may indicate to the U2U Relay UE candidates the discovery model (Model A or Model B) used by the target End UE along with its U2U relay discovery solicitation.

Proposal 2: As when triggering the relay selection, any of the UEs may provide a list of U2U Relay UE candidates for the other UE to perform the U2U relay selection for a direct-to-indirect path switch.

Observation 2: To minimise the service interruption, upon triggering the relay reselection or connection reestablishment, the current U2U Relay UE should continue acting as relay for the source End UE and the target End UE for a certain time period while the source and target End UEs perform the U2U relay reselection.

Proposal 3: The trigger from the current U2U Relay UE for U2U relay reselection may include a condition on when the current U2U Relay UE stops serving as a U2U relay between the source End UE and the target End UE.

Proposal 4: One of the UEs (either the source End UE or the target End UE) may provide a list of U2U Relay UE candidates for the other UE to perform the U2U relay reselection for an indirect-to-indirect path switch.

Observation 3: Enabling the source and target End UEs to probe each other over SL for direct reachability will ensure that relay connections are maintained only when needed.

Proposal 5: RAN2 considers specifying triggers at least for indirect-to-direct path switch related to U2U relay.

Proposal 6: The Local ID identifies a pair of the source and target End UEs on SRAP level over each hop in the L2 U2U relay connection.

Proposal 7: The Local ID is unique per hop and specific to each hop.

Proposal 8: RAN2 adopts a single 8-bit Local ID that is included in the SRAP header.

Proposal 9: The assignment of the Local ID is hop-by-hop.

Proposal 10: The assignment of the local ID is initiated by either the source End UE or the target End UE whichever performs the U2U relay (re)selection.