3GPP TSG RAN WG2#122	R2-2305597

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.7.4.1.1

Source:	CMCC

Title:	Discussion on NTN-TN cell reselection enhancements

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions about NTN-TN cell reselection enhancements and some agreements were reached as following [1]:

Agreements:

1.	For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

Agreements via email – from offline 106:

- Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused

- Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent.

Agreements online:

- The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead

- The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info

Working assumption:

- We do not introduce new triggers making the UE reacquire the TN coverage information from SI

Agreements via email – from offline 107:

- On a frequency band number shared by TN and NTN (e.g., n1), if NTN-specific assistance information is NOT provided for a neighbour cell configured in SIB3/SIB4, UE assumes this is a TN neighbour cell. This understanding is also applicable for Rel-17 and it does not need any spec update

In this contribution, we would like to provide some further analysis about TN coverage information aspects and UE storage overhead issue for NTN-TN reselection enhancements.

Discussion

  - 2.1 Additional information in dedicated signaling for TN coverage

In last RAN2 meeting, we have discussed the TN coverage information provision solution among option 1, 2 and 6, and agreed to broadcast center location coordinates with radius of a area with a group TN cells instead of a single one. Further, some companies indicates that SIB may provide just a coarse information considering the signaling overhead, while more details shall be given using dedicated signaling. However, the TN coverage information is just used to help UE initiate TN neighbor cell measurement, coarse information is sufficient. And the introduction intention of this information is for idle/inactive state, no dedicated signaling is available. Therefore, no additional information in dedicated signaling is needed.

Proposal 1: Additional information in dedicated signaling for TN coverage is not needed.

  - 2.2 Whether reuse Rel-17 referenceLocation and distanceThresh

R17 referenceLocation and distanceThresh description are as following [3]:

- –	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

- –  Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

–   SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

⁝

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

⁝

And we agreed that area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. Therefore, to be simple, Rel-17 referenceLocation and distanceThresh could be reused.

Proposal 2: Rel-17 referenceLocation and distanceThresh could be reused.

  - 2.3 TN coverage list size and which SIB to use

Regarding the TN coverage list size, companies provided comments in email discussion [3], while there were no solid value. And we agreed to postpone this issue until more is known on the format of this information and how is it sent. From our perspective, to help push the progress, we could discuss typical NTN deployment scenario first due to that in scenario where NTN is deployed, TN coverage may generally be rare. And different satellite type has different coverage, which may also has impact on the TN coverage list size.

And for which SIB to use, this issue is related to the TN coverage list size mentioned above, and TN coverage list size issue should go for first. Then, as we known, TN coverage information is introduced for NTN-TN reselection. There are still remaining room in SIB19, maybe we could consider SIB19 firstly. If the TN coverage information size is larger than the remaining room in SIB19 after evaluation, we could consider a new SIB.

Proposal 3: To help push the progress about TN coverage list size, we could discuss typical NTN deployments and/or different satellite type cases firstly.

Proposal 4: Once TN coverage list size is determined, we could consider SIB19 firstly. If the TN coverage information size is larger than the remaining room in SIB19 after evaluation, we could consider a new SIB.

  - 2.4 UE storage overhead

For the UE storage overhead, with the proper assistance information about TN neighbour cells, UE could be able to avoid some TN neighbour cells measurements. And UE just need to focus on limited information. Therefore, there may not be large amounts of data at UE side without storage problem.

Proposal 5: With the proper assistance information about TN neighbour cells, there may not be storage problem at UE side.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on NTN-TN cell reselection enhancements and have the following proposals:

Proposal 1: Additional information in dedicated signaling for TN coverage is not needed.

Proposal 2: Rel-17 referenceLocation and distanceThresh could be reused.

Proposal 3: To help push the progress about TN coverage list size, we could discuss typical NTN deployments and/or different satellite type cases firstly.

Proposal 4: Once TN coverage list size is determined, we could consider SIB19 firstly. If the TN coverage information size is larger than the remaining room in SIB19 after evaluation, we could consider a new SIB.

Proposal 5: With the proper assistance information about TN neighbour cells, there may not be storage problem at UE side.

Reference

- RAN2-121bis - NR-NTN-IoT-NTN (Sergio) 2023-04-26_EOM

- 3GPP TS 38.331 V17.4.0 NR; Radio Resource Control (RRC) protocol specification

- R2-2304246 Report from [AT121bis-e][106][NR NTN Enh] Signaling of TN coverage (Nokia)_summary