3GPP TSG RAN WG2#122	R2-2305599

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.7.4.2

Source:	CMCC

Title:	Discussion on handover enhancements for NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions about handover enhancements and some agreements were achieved as follows [1]:

Agreements:

- In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility

- For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command

Agreements via email – from offline 109:

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- RAN2 confirms the general UE procedure for NTN RACH-less HO

1.	receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

2.	start timer T304 for the target cell (RRC)

3.	perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

4.	start time alignment timer (MAC)

5.	monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

6.	send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

7.	consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

8.	stop timer T304 for the target cell. (RRC)

FFS whether to release UL grant if pre-allocated after RACH-less HO completion

FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

4.	The pre-allocated grant is provided as type-1 CG

5.	Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

1. whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

2. to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

3. power control for initial UL transmission

Agreements online:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

Agreements:

- In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

- Remove the part in brackets “as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18” in the LS to RAN1. The action to RAN1 will also ask for feedback for the hard satellite switch (not only the soft satellite switch case), e.g. action to RAN1 is to see if there are any major technical issues (as in the agreement).

Then in this contribution, we would like to provide further analysis on group HO, RACH-less HO and common (C)HO configuration for handover enhancements.

Discussion

- 2.1 Group handover

Regarding group HO, there are few discussion in previous meetings and whether to support it is still not clear. Considering the handover signaling overhead in NTN system where there may be plenty of UEs need to hand over to a new cell in a short period of time due to the large coverage of satellites as shown in Figure 1 and 2, group HO is also an interesting candidate solution. Specifically, group HO could help to reduce the number of UEs that perform handover simultaneously, which is important for HO signaling reduction and resource(e.g. RACH resources) shortage alleviation.

Fig.1 UEs transition in moving cell

Fig.2 UEs transition in quasi-earth fixed cell

And UEs could be divided in the same group according to given rules could use the configurations and timing information to handover to the corresponding target cell. And there are many kinds of UE division principles, for example, distance-based solution(distance between UEs and cell(serving cell/target cell) reference are same or belong to same range), time-based solution(remaining serving time is same or belong to same range, time-based solution may be more suitable to moving cell in which remaining serving time is per UE. ) , etc.

Furthermore in a same group, the measurement results of some UE can be used to determine whether all UEs in the group need to handover to decrease measurements reporting. Therefore, considering the different characteristics between quasi-earth fixed cell case and earth moving cell case, if PCI unchanged solution just support for quasi-earth fixed cell case, then group handover could be discussed for earth moving cell.

Observation 1: Group HO can bring obvious benefits for HO signaling reduction, resource(e.g. RACH resources) shortage alleviation and measurements reporting decrease, etc.

Observation 2: UEs could be divided in the same group according to given rules(e.g. distance-based solution, time-based solution).

Proposal 1: RAN2 could discuss group HO for earth moving cell case if PCI unchanged just support for quasi-earth fixed cell case.

- 2.2 RACH-less HO combined with PCI unchanged

In last meeting, there were some discussions on RACH-less HO, and confirmed the general UE procedure almost same as LTE RACH-less HO. And in this contribution, we would like to focus on the FFS issue about RACH-less HO combined with PCI unchanged. We have already reached the agreement that in quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18). Then if combining RACH-less with the PCI unchanged solution, the L3 mobility which brings more overhead is not needed and step 2-8 of RACH-less HO procedure may not be mandatory either.

Proposal 2: Kindly suggest RAN2 consider to combine RACH-less with the PCI unchanged solution, the L3 mobility which brings more overhead is not needed and step 2-8 of RACH-less HO procedure may not be mandatory either.

## 2.3 Common (C)HO configuration

There are some discussions on broadcast common (C)HO configuration in RAN2 #121 meeting, and RAN2 suggest that as a starting point, t304 and spCellConfigCommon within ReconfigurationWithSync can be delivered to UEs in common signaling, while some companies still have concerns about the actual gain. And in last RAN2 meeting, during the offline [2], the slight majority shown the gain of signaling overhead reduction and prefer to provide common (C)HO configuration, while opponents still have concern about the complexity and gain. Then there are no agreement and the issue is postponed to this meeting with suggestion to focus on the quasi-Earth Fixed Cell case.

From our perspective, provide common (C)HO configuration has some benefits due to handover signaling overhead reduction(e.g. some message size reduction). However, considering the large coverage of a NTN cell, different coverage parts may require slightly different common signaling, and just with common signaling by broadcast may not be proper for all the UEs in the cell.Therefore, we could consider a compromise solution, for example, to combine the group HO and common (C)HO configuration, and provide common (C)HO configuration in a same group by groupcast signaling. Further, earth moving cell may face more serious signaling overhead considering the high speed movement satellite with non-steerable beams, which may also need to discuss signaling overhead reduction, and the combination of group HO and common (C)HO configuration is of benefit for this case. Thence we could consider not to exclude earth moving cell case.

Proposal 3: It is proposed to combine the group HO and common (C)HO configuration, and provide common (C)HO configuration in a same group by groupcast signaling.

Proposal 4: Kindly suggest RAN2 not to exclude earth moving cell case.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on group HO, RACH-less HO and common (C)HO configuration for handover enhancements and have the following observations and proposals:

Observation 1: Group HO can bring obvious benefits for HO signaling reduction, resource(e.g. RACH resources) shortage alleviation and measurements reporting decrease, etc.

Observation 2: UEs could be divided in the same group according to given rules(e.g. distance-based solution, time-based solution).

Proposal 1: RAN2 could discuss group HO for earth moving cell case if PCI unchanged just support for quasi-earth fixed cell case.

Proposal 2: Kindly suggest RAN2 consider to combine RACH-less with the PCI unchanged solution, the L3 mobility which brings more overhead is not needed and step 2-8 of RACH-less HO procedure may not be mandatory either.

Proposal 3: It is proposed to combine the group HO and common (C)HO configuration, and provide common (C)HO configuration in a same group by groupcast signaling.

Proposal 4: Kindly suggest RAN2 not to exclude earth moving cell case.

Reference

- RAN2-121bis - NR-NTN-IoT-NTN (Sergio) 2023-04-26_EOM

- R2-2304248 Report of [AT121bis-e][108][NR NTN Enh] Common (C)HO configuration (Ericsson)