3GPP TSG RAN WG2#122	R2-2305603

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.8.5

Source:	CMCC

Title:	UAV identification broadcast

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, UAV identification broadcast was discussed and some agreements were reached as following[1]:

Agreements:

- DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements.

- LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported

- NR PC5 mode-1 is not supported

- For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason

- RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators

- As a baseline, we will use the existing V2X QoS framework.  FFS whether different resource pools are needed for UAV services

- No further enhancement on PC5 range for A2X broadcast services will be pursued in this release

- We will not investigate interference for BRID and DAA broadcast

- Send an LS to SA2 to:

- inform them as a result of RAN Plenary decision to re-use BRID RAN2 will only support PC5 broadcast for deconfliction in RAN in release 18.

- ask SA2 whether BRID and DAA broadcast over LTE and NR PC5 requires new QoS requirements and parameters not supported within the ranges supported for V2X

In this contribution, we will provide some further analysis on the open issues for UAV identification broadcast.

Discussion

Resource pool configuration

In last RAN2 meeting, we have discussed QoS framework and agreed to use existing V2X framework as baseline. And we sent an LS to SA2 to ask whether BRID and DAA broadcast over LTE and NR PC5 require new QoS requirements and parameters not supported within the ranges supported for V2X. Therefore, we could wait for SA2 reply, if new QoS requirements are needed for UAV, then separate resource pool configuration should be needed. Otherwise, to differentiate between V2X and U2V, some enhancements may also be needed considering the high altitude characteristic of UAV system. For example, utilize measurement reporting, flight path information, height, and beam information, etc. to help improve PC5-based BRID solution.

Proposal 1: It is proposed to wait for SA2 reply firstly, if new QoS requirements are needed for UAV, then separate resource pool configuration should be needed.

Interference avoid between V2X and UAV

As the latest WID scope and discussion progress, regarding UAV identification broadcast, it has decided to study PC5-based enhancements if needed. Considering that the scope is not intended to introduce new band(s), there may be some potential interference problem. And unlike V2X communication, UAVs would face even more serious coexistence problems. Therefore, some further interference avoidance or reduction enhancements may be needed based on PC5-based BRID(Broadcast remote ID) solution.

Proposal 2: It is proposed that some further interference avoidance or reduction enhancements may be needed based on PC5-based BRID (Broadcast remote ID) solution.

Remote ID content

Then, we can find detail information about remote ID in FAA(Federal Aviation Administration) website [2], for the usage of remote ID, Remote ID helps the FAA, law enforcement, and other federal agencies find the control station when a drone appears to be flying in an unsafe manner or where it is not allowed to fly. Remote ID also lays the foundation of the safety and security groundwork needed for more complex drone operations. And we also could find the information to be broadcasted:

Fig.1 Illustration of remote identification in [2]

thence, regarding the specific information included in UAV identification, the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Proposal 3: It is proposed that the the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on UAV identification broadcast for NR UAV and have the following proposals:

Proposal 1: It is proposed to wait for SA2 reply firstly, if new QoS requirements are needed for UAV, then separate resource pool configuration should be needed.

Proposal 2: It is proposed that some further interference avoidance or reduction enhancements may be needed based on PC5-based BRID (Broadcast remote ID) solution.

Proposal 3: It is proposed that the the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Reference

- R2-2304203_R2_121bis-e_Notes_R18 UAV NES R15-17 UP_SDT (Diana)_04-26

- https://www.faa.gov/uas/getting_started/remote_id