3GPP TSG RAN WG2 #122 Incheon, KR	R2-2305604

May 22nd-26th, 2023

Agenda item:	7.5.4.1

Source:	CMCC, Huawei, HiSilicon, China Unicom

Title:	Consideration on Piecewise Linear BS Table

Document for:	Decision, Discussion

1	Introduction

In RAN2 121bis-e[1], consensus has been reached on BSR solution for XR, and there are still some remaining  FFSs left . One of them we would like to discuss in this contribution, which is shown in the following:

(25/30) At least linear distribution is used for generating code points in new BSR table(s).  FFS (13/25) whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported.

In this contribution, we will provide our analysis and simulation results on BS table for XR service.

2	Discussion

2.1 Overview

In the last meeting, the distribution about the new BSR table(s) was discussed. Most of the companies thought that the distribution could be exponential, linear or truncated Gaussian distribution.

Considering the simulation assumption of RAN1 and other surveys and studies, although there are some traffics of pose information, video traffics still occupy a large proportion in the UL XR traffics, which issues desire to be addressed as well. Terminology such as Data burst and PDU sets from SA2 are defined based on video frames. Hence the size of video frame obeys the Truncated Gaussian distribution, and the introduction of Gaussian distribution for the new BSR table is beneficial to improve the capacity of XR service.

Observation 1: Although there are some traffics of pose information, video traffics still occupy a large proportion in the UL XR traffics, which issues desire to be addressed as well.

In addition, some studies [2] outside 3GPP have shown that the data of VR service obeys student's t-distribution, which is kind of a special Gaussian distribution. Therefore, it is necessary to investigate the performance of Gaussian BS table in terms of capacity improvements and quantization error reduction for XR services.

On the other hand, since the CDF of Gaussian distribution cannot be expressed by simple algebraic expression (requires the use of erf function, i.e., Gauss error function):

Φx= -∞x1σ2π∙e-(x-μ)22σ2dx= 121+erfx-μσ2

In the following section, relevant simulation results and observations on this issue are provided.

2.2 Verification and simulation

Considering that there does not exists an off-the-shelf XR performance test platform, we used WLAN instead of cellular networks to provide internet access for the head mounted device. For the XR application, we chose Ace combat 7: Sky Unknown and Project: Wingman, both of which can run in VR mode and player is allowed tobe interacted with 3DoF, which is a typical VR application at present.

We set the graph setting to 1080p@60fps to match the common use cases. We then collect the steamed packets and use that data as the basis for our simulation data to improve the confidence of the simulation. Please be kindly noted that the steamed video frame has been compressed by NVENC.

Simulation results based on the captured packets is shown in the following figure where both therival interval and size of the frames obey the truncated Gaussian distribution, and the video frame size distribution is shown in the following figure as well:

Observing  the CDF of video frame buffer size, we can see that CDF is not linear. Therefore, using single linear distribution introduces quantization errors. However, as mentioned above, concern rises from some companies when configuring the original CDF may cause excessive signaling overhead and increase the implementation complexity of both NW and UE.

As a compromising approach, we think using piecewise linear distribution as the quantization function of BS table can reduce the complexity of Gaussian distribution while maintain a low-level quantization error.

We use short BS table firstly for our simulation, since short BS table fits more for our steamed packets. As shown in the figure below, several common BS Tables are drawn together with actual buffer size. The trend of the CDF is obvious that piecewise linear distribution has the lowest quantization error (the closer to the actual buffer size, the lower to the quantization error):

In order to verify the reliability of this conclusion, we then performed some simulations and counted the quantization errors and obtained the following results:

Per the simulation result, Linear has about 10 dB gain compared to Legacy (exponential), while Piecewise linear(Gaussian) has about 3 dB gain compared to Linear, so it can be proved that Piecewise linear is a better choice than Linear in terms of reducing the quantization error in our traffic assumption.

The used BS Tables are shown in the following figures:

In RAN2 121bis-e, companies has agreed to introduce new BS table with narrow range. Due to we do not have any agreement for the specific value for the min/max value of the narrow range, we set range of short BSR to [0, 250KB], and the long BS table to [0, 77MB] as baseline, considering legacy BS table goes to +∞.

To better exploit the performance of BS table with narrow range, we conduct further simulation with narrow range, the results are as follows,

The simulation results are similar, piecewise linear still has 3dB less quantization error than linear distribution. As can be observed from the buffer table curve, the linear and piece wise linear distribution has the same narrow range.

Then we use long BS table for our simulation, the results changes a bit here. As explained, we narrow the legacy BS table range, and the short BS table has 32 indexs with its buffer size ranges [0, 250KiB] in our simulation, while long BS table has 256 indexs with its buffer size ranges [0, 77MiB]. Therefore we expect the quantization error of linear distribution to increase substantially due to increased quantization step.

And the simulation proves our conjecture as follows:

The reason is that the buffer size recorded and simulated is in a narrow range (less than 200KiB) compared to that of long BS table (up to 77MiB), and linear distribution which has a constant quantization step over the whole range of long BS table range will have a larger quantization error as result.

For better compatible with RAN1, we use the traffic assumption in [3], as follows,

| Category | Direction | Data Rate (Mbps) | Periodicity (ms) | PDB (ms) | Reliability | Jitter (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| VR | UL | a 100-byte packet once | 4 | 10 | 99% | No jitter |
|  | DL | 30 | 16.67 | 10 | 99% | [-4,4] |
| Cloud Gaming | UL | a 100-byte packet once | 4 | 10 | 99% | No jitter |
|  | DL | 30 | 16.67 | 15 | 99% | [-4,4] |
| AR | UL | 10 | 16.67 | 30 | 99% | [-4,4] |
|  | DL | 30 | 16.67 | 10 | 99% | [-4,4] |

The average frame size here is about 170KiB, and is similar with our assumption that the frame size is about150KiB at 1080@60fps with NVENC. Considering that there can be multiple AR flows simultaneously, we re-conduct the simulation with 3,10 and 30 AR flows, the results are as follows,

Still, piecewise linear distribution has the least quantization error.

Observation 2: Piecewise linear distribution has the least quantization error compared to exponential and linear distribution.

Proposal: RAN2 to agree that piecewise linear distribution can be used for new BS table(s) for XR traffic.

3	Summary

This contribution discusses potential capacity enhancement for XR, the following are the observation that may be taken into consideration.

Observation 1: Although there are some traffics of pose information, video traffics still occupy a large proportion in the UL XR traffics, which issues desire to be addressed as well.

Observatiuon 2: Piecewise linear distribution has the least quantization error compared to exponential and linear distribution.

Based on the above observation, we propose that:

Proposal: RAN2 to agree that piecewise linear distribution can be used for new BS table(s) for XR traffic.

References

- R2-2304202, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

- M. Lecci, M. Drago, A. Zanella and M. Zorzi, "An Open Framework for Analyzing and Modeling XR Network Traffic," in IEEE Access, vol. 9, pp. 129782-129795, 2021, doi: 10.1109/ACCESS.2021.3113162.

- TR 38.838, 	Study on XR (Extended Reality) evaluations for NR, RAN 1