3GPP TSG RAN WG2 #122 Incheon, KR	R2-2305605

May 22nd-26th, 2023

Agenda item:	7.5.4.3

Source:	CMCC

Title:	Consideration on Retransmission less CG for XR

Document for:	Decision, Discussion

1	Introduction

In RAN2 121bis-e[1], consensus has been reached on the introduction of Retranssmission less CG for XR, , which are shown in the following:

There is support to adopt NTN solution for the retransmission-less CG.

If adopted, RAN2 aims to only consider option1 or option 2:

- Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

- Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG.

FFS how the solution ensures consistent HARQ operation.

In this contribution, a discussion on retransmission less CG will be conducted for XR capacity improvements and power saving enhancement.

2	Discussion

2.1 Retransmission-less CG

In the last meeting, companies agree to introduce retransmission-less CGs for XR for potential energy saving gain compared to regular CG. The removal of drx-RetranssmissonTimerUL has raised concerns about the reliability of retransmitting less CG. Before discussing specific enhancements, it is necessary to study in advance how to make the HARQ Mode B introduced for NTN adaptable to XR, since it is clear than XR service should be performed in TN environment.

In R17 NTN, due to the need to communicate with the satellite, RTT time in NTN environment is much longer than TN. At that time, if we continue to use legacy HARQ mechanism, it will cause more HARQ processes to be occupied due to the long RTT, which leads to new transmission has no available HARQ process, i.e., "HARQ process stalling" issue. To solve this problem, NTN introduces a new HARQ Mode, i.e., Mode B, as follows:

uplinkHARQ-mode Used to set the HARQ mode per HARQ process ID, see TS 38.321 [3]. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. A bit set to one identifies a HARQ process with HARQmodeA and a bit set to zero identifies a HARQ process with HARQ modeB. This field applies for SRBs and DRBs.

In TS 38.321,

1> if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers: 2> if this Serving Cell is configured with uplinkHARQ-Mode: 3> if the corresponding HARQ process is configured as HARQModeA: 4> set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value; 4> if drx-LastTransmissionUL is configured: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 4> else: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 2> else: 3> if drx-LastTransmissionUL is configured: 4> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 3> else: 4> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 2> stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

It can be seen that when a HARQ rocess is configured as HARQModeB, UE shall not start drx-HARQ-RTT-Timer-NTN after an uplink transmission, as well as drx-RetransmissionTimerUL. However, if it is configured as HARQModeA, the drx-HARQ-RTT-Timer-NTN will be extended with regard to the UE-gNB RTT time and drx-RetransmissionTimerUL will be started as legacy. Therefore, if HARQModeA is configured, legacy retransmission mechanism based on PUSCH decoding can be applied, while in HARQModeB, due to the removal of drx-RetransmissionTimerUL, retransmission based on PUSCH decoding cannot be applied.

For retransmission-less CG, the first issue is that HARQModeB and HARQModeA are introduced for NTN, and XR is a service provided based on TN, so many UE may not support HARQModeB. And it requires an explicit IE to indicate that UE supports HARQModeB. Luckily, it has already been done in NTN.

uplink-Harq-ModeB-r17 ENUMERATED {supported} OPTIONAL

With uplink-Harq-ModeB-r17 and uplinkHARQ-Mode already defined, NTN's support for HARQModeB is relatively complete. XR service in the TN environment can also reuse the NTN mechanism without too many modifications.

Observation 1: uplink-Harq-ModeB-r17 and uplinkHARQ-mode can be reused for retransmission-less CG for XR in TN.

Since uplinkHARQ-mode is configured per HARQ process, both CG and DG can implement retransmission-less transmission using HARQ process configured with HARQModeB for potential power saving gain. Although CG is supposed to be more suitable for frequent and periodic transmission of uplink pose information, it is not necessary to exclude DG for scheduling pose and motion information. Retransmission-less DG should also be supported in case the reporting rate of interaction device is not fixed. The per HARQ process ID HARQ mode configuration can better solve this problem.

The other reason is that the allocation and occupation of HARQ process ID is more a problem of NW implementation. Configuring HARQ mode per HARQ process can make it possible for both DG and CG fulfill the requirement of more power saving gain by using retransmission-less transmission, and also the requirement of flexibility in the compromise of reliability and power saving by selective using HARQ process configured HARQModeA and HARQModeB, e.g., for high reliability requirements (operating UAV/drones), CG/DG with HARQModeA can be configured and for low power consumption requirements, CG/DG with HARQModeB can be configured.

Observation 2: Reusing uplink-Harq-ModeB-r17 and uplinkHARQ-mode can support retransmission-less transmission for both CG and DG and offer a flexible compromise between power consumption and reliability.

Therefore, we think configuring retransmission-less CG per CG configuration is not necessary.

Proposal 1: RAN2 to support retransmission-less CG by reusing uplinkHARQ-mode and uplink-Harq-ModeB in TN.

As to reliability assurance, which we think is different from NTN, XR tends to have higher reliability requirement. The introduction of retransmission-less CG is for frequent and periodic pose and motion information. Research [2] shows that latency leads to visually induced motion sickness (VIMS). The removal of drx-HARQ-RTT-TimerUL can result in latency increase due to late retransmission or even no retransmission and further make user feels discomfort. Therefore, we think it is necessary to discuss whether there exist need to introduce mechanisms ensuring transmission reliability and to avoid larger latency caused by less or no retransmission.

Proposal 2: RAN2 to introduce reliability assurance mechanism for retransmission-less CG, e.g., blind retransmission, using LowSE MCS table.

2.2 Multi-PUSCHs CG

Since RAN1 carried out plenty of the work for CG enhancement, e.g., new UCI for unused TOs indication and allocation of HARQ process ID. The discussion in RAN2 should focus on high layer details (e.g., signaling, configuration, assistance information, etc.). Due to the introduction of multi-PUSCHs CG in RAN1, the NW should indicate explicitly UE the number of TOs (transmission occasion) and the interval of TOs within one CG period in CG configuration based on assistance information and resource occupation.

Proposal 3: NW (RRC) can configure the number of transmission occasions (TOs) within one CG period in multi-PUSCHs CG configuration.

Besides, in multi-PUSCHs CG configuration, the interval of TOs can be even (single valued) or uneven (multivalued). If interval of TOs is single valued, all TOs are considered to be uniformly distributed within CG period. If not, the interval between TOs is uneven.

It is generally believed that jitter obeys the Poisson distribution in time domain, that is, the probability of small jitter is high, so it is actually not a wise choice to use a uniform interval between TOs.

Observation 3: Uneven interval between TOs is more appreciated since jitter obeys Poisson distribution other than Uniform distribution.

We believe that the other reason for using uneven TOs is that it can cover a longer period of time with fewer TOs and makes it better for adopting jitter. And it can better adapt jitter and reduce signaling overhead.

This consideration comes with that too many TOs can affect the scheduler's resource allocation strategy, e.g., radio resources may not be used by other UEs due to potential TO transmissions, and too many TOs may also cause the signaling overhead of the indicated unused CG TOs to become significant.

Observation 4: Too many multi-PUSCHs CG TOs can cause overhead and difficulty in indicating unused CG PUSCH TOs.

Proposal 4: NW (RRC) can configure the interval between TO within one CG period, and the intervals can be uneven. FFS configure all intervals or the step of interval (scale factor).

In addition, we noticed a RAN1 agreement which has impact on RAN2,

Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the indicated “unused” CG PUSCH TO(s), if any, by the UCI in a CG PUSCH for a CG configuration - can be consecutive or non-consecutive CG PUSCH TO(s) in time domain [in one CG period] - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. Note: FFSs and further details in corresponding agreement in RAN1#112 for the selected option are remained for further discussion Noe: Above corresponds to Option 2 (w.r.t. agreement in RAN1#112)

We notice that if unused CG can be non- consecutive, which means more than one TO can be utilized by UE within one CG period.

For better adopt the jitter of data burst in data volume, we think it is not necessary to restrict UE to only be able to allocated one TO within CG period.

Proposal 5: UE can be allocated multiple TOs within one CG period for better adaptation the jitter caused by jitter in data volume for data burst.

Considering the requirement to send video frames in uplink in XR services, it is also necessary to consider the jitter and non-integer interval of video frame arrival, as well as the variation of video frame size. In particular, data burst may need to transmit more data in one CG period (i.e., one TO is not enough to bear all the data), and we believe it is necessary to evaluate the performance of using multiple TOs within CG period or using single TO and the rest data is transferred by DG.

Using multiple TOs within CG period, UE need not to send BSR or other indicator for UL grant and does not have to wait for the NW to send a UL grant, which obviously has the advantage of less delay, but this may have some impact on RAN1 (e.g., HARQ process ID assignment and retransmission).

As for using single TO and DG, where a BSR or other indicator is necessary to indicate UE still has data to send, we do not observe any major impact on current specification, except that the BSR trigger mechanism may need to update. But this approach may cause an increase in latency and the use of additional PDCCH resources, especially compared to the multiple TOs within CG period approach.

Observation 5: When the TBS of data burst exceeds that of single TO within one CG period, using multiple TOs within one CG period can reduce overall latency but may have impacts on RAN1, while using single TO+DG has no specification impact but could increase latency.

Proposal 6: RAN2 is kindly asked to discuss how to transfer data burst in uplink when its TBS exceeds single TO in Multi-PUSCHs CG:

Opt1: using multiple TOs within one CG period firstly, if all TOs are still not enough, use DG.

Opt2: using single TO+DG. FFS whether to send UTO indication for less latency.

2.3 Indication of unused CG TO(s)

The other most mentioned issue is the dynamic indication of unused CG PUSCH TO(UTO). In RAN1#112bis-e, related agreements are as follows [3],

Agreement - Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Agreement The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement). Agreement - With respect to PHY two-level priority, for a configured grant PUSCH configuration, the “UTO-UCI” has the same priority level as the configured grant PUSCH. - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

We can see that RAN1 is working on UTO-UCI for dynamic indication of unused CG TO and indication of multiple unused CG TOs within one CG period is already agreed by RAN1.

However, there exists an issue that PHY is not aware of the size of data burst and cannot recognize which TOs will not be used. Such information should be provided from MAC.

Proposal 7: MAC entity is responsible for recognize unused CG TO(s) and signaling PHY.

Another issue we concerns is the availability of recycled UTO. Since the CG periodicity is close the video frame arrival interval (16.67ms as a baseline), the interval of TOs within one CG period is can be less than 3ms when there are more than 5 TOs.

With such short of time, whether the UTO can be recycled for new resource allocation is questionable considering k2 and both the processing time of gNB and sending UTO-UCI. Therefore, we think that UTO-UCI should be sent as soon as possible (e.g., no need to wait for used CG TO) for recycling.

If the time left is not enough for UTO recycling, UTO-UCI can be omitted since UTO cannot be available for other UE either.

Proposal 8: UTO-UCI should be sent as soon as possible for UTO recycling, while if there left little time for NWrecycling, UE can omit to send UTO-UCI.

On the other hand, from the perspective of power saving and complexity reduction, if UE use TO(s) to transfer data, the indication of UTO(s) can also be multiplexed into the MAC PDU (e.g., new MAC CE) to avoid sending additional UTO-UCI signaling in PUCCH.

Proposal 9: RAN2 is kindly asked to discuss whether to allow UTO indication be multiplexed in MAC PDU, if exists.

3	Summary

This contribution discusses potential capacity enhancement for XR, the following are the observation that may be taken into consideration.

Retransmission-less CG aspect:

Observation 1: uplink-Harq-ModeB-r17 and uplinkHARQ-mode can be reused for retransmission-less CG for XR in TN.

Observation 2: Reusing uplink-Harq-ModeB-r17 and uplinkHARQ-mode can support retransmission-less transmission for both CG and DG and offer a flexible compromise between power consumption and reliability.

Proposal 1: RAN2 to support retransmission-less CG by reusing uplinkHARQ-mode and uplink-Harq-ModeB in TN.

Proposal 2: RAN2 to introduce reliability assurance mechanism for retransmission-less CG, e.g., blind retransmission, using LowSE MCS table.

Multiple-TOs aspect:

Proposal 3: NW (RRC) can configure the number of transmission occasions (TOs) within one CG period in multi-PUSCHs CG configuration.

Observation 3: Uneven interval between TOs is more appreciated since jitter obeys Poisson distribution other than Uniform distribution.

Observation 4: Too many multi-PUSCHs CG TOs can cause overhead and difficulty in indicating unused CG PUSCH TOs.

Proposal 4: NW (RRC) can configure the interval between TO within one CG period, and the intervals can be uneven. FFS configure all intervals or the step of interval (scale factor).

Proposal 5: UE can be allocated multiple TOs within one CG period for better adaptation the jitter caused by jitter in data volume for data burst.

Observation 5: When the TBS of data burst exceeds that of single TO within one CG period, using multiple TOs within one CG period can reduce overall latency but may have impacts on RAN1, while using single TO+DG has no specification impact but could increase latency.

Proposal 6: RAN2 is kindly asked to discuss how to transfer data burst in uplink when its TBS exceeds single TO in Multi-PUSCHs CG:

Opt1: using multiple TOs within one CG period firstly, if all TOs are still not enough, use DG.

Opt2: using single TO+DG. FFS whether to send UTO indication for less latency.

Indication of UTO aspect:

Proposal 7: MAC entity is responsible for recognize unused CG TO(s) and signaling PHY.

Proposal 8: UTO-UCI should be sent as soon as possible for UTO recycling, while if there left little time for NW recycling, UE can omit to send UTO-UCI.

Proposal 9: RAN2 is kindly asked to discuss whether to allow UTO indication be multiplexed in MAC PDU, if exists.

References

- R2-2304202, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

- M. Lecci, M. Drago, A. Zanella and M. Zorzi, "An Open Framework for Analyzing and Modeling XR Network Traffic," in IEEE Access, vol. 9, pp. 129782-129795, 2021, doi: 10.1109/ACCESS.2021.3113162.

- R1-2304045	Moderator Summary#2 - XR Specific Capacity Improvements	Moderator (Ericsson)