3GPP TSG RAN WG2 #122 Incheon, KR	R2-2305607

May 22nd-26th, 2023

Agenda item: 	7.14.4

Source: 	CMCC

Title: 	Consideration on QoE measurement for NR-DC

Document for: 	Decision, Discussion

1	Introduction

In RAN2 121bis-e[1], consensus has been reached on NR-DC, and several specific measures were agreed upon and leaving some FFSs. These FFSs include the following:

As a baseline, Rel-17 pause/resume procedure is reused to pause/resume reporting of one or multiple QoE measurement configurations in a UE for NR-DC. Details are FFS, e.g. whether paused QoE reports can be reported to SN (if SN is not overload).

The network can optionally explicitly indicate the SRB for the QoE reporting if both SRB4 and SRB5 are configured. FFS on the granularity, e.g. per QoE config or otherwise.

MN- or SN-associated QoE reports can use either SRB4 or SRB5 if only one of SRB4 or SRB5 is configured for the UE. FFS whether network configuration is needed

For NR-DC, if SRB5 is not configured (FFS on the SCG deactivation case), UE can transmit the SN-associated QoE reports via SRB4. FFS whether there are some ambiguities how MN knows where to forward this.

FFS pending RAN3 decisions:  Whether RVQoE reports and encapsulated QoE reports are reported together to the same node (MN or SN) in NR-DC.

For SRB switching in NR-DC scenario, FFS on the explicit indication and implicit indication, e.g. signaling impacts, details on UE/NW behaviours.

In this contribution, a comprehensive discussion on QMC in NR-DC will be conducted.

2	Discussion

2.1 QoE configuration in NR-DC

Since the QoE measurement collection job that the UE can perform is limited. In R17, maxNrofAppLayerMeas-r17 is introduced to indicate the maximum number of simultaneous QMCs supported by the terminal, which is currently 16. Therefore, in the NR-DC scenario due to the presence of both MCG and SCG, it is necessary to require MN and SN Confirm that the number of configured QMCs does not exceed this maximum

On the other hand, RAN2 should also discuss whether to increase the maximum number of simultaneous QMCs for NR-DC, since both MCG and SCG can perform QMC jobs.

Proposal 1: MN and SN should be able to coordinate and ensure the max number of simultaneously QoE measurement collection will not exceed maxNrofAppLayerMeas-r17. FFS whether to introduce new parameter or extend maxNrofAppLayerMeas-r17.

As for the QoE report leg issue, we believe that whether the indication of the QoE report leg is explicit or implicit actually depends on whether SRB4 and SRB5 will be configured in the MN/SN at the same time. If only SRB4 is configured in MN and only SRB5 is configured in SN, UE can be implicitly informed of the change of QoE report leg by the addition and release of SRBs. However, if it is not possible that only SRB4 or SRB5 is configured at the same time, then a display indication should be introduced.

In general we think that explict configuration indicator is better, so that any potential risks can be avioded when SRB4 or SRB5 configuration and/or release encounters implantation issues.

We noticed that RAN3’s also discusses this issue and they decide to use explict leg change indication[2].

The network can explicitly instruct a UE in NR-DC to switch the reporting leg. Proposal 4-2: Discuss whether the reporting leg switch is per QoE reference. Nok: Do we need 4-2? It has been covered by 5-1. E///: UE may be configured from different nodes The leg switching command can be sent to the UE by the node that configured that specific QoE configuration. FFS how to handle the maintenance of QoE configuration after SN release, after mobility for an NR-DC UE and after the change from NR-DC to single connectivity. The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node. The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

Hence, an explict indiation is the baseline for changing QoE reporting leg in NR-DC.

Proposal 2: RAN2 agree to introduce an explict indicator for QoE reporting leg switching. If not, RAN2 has to ensure that SRB4 and SRB5 will not be configured simultaneously.

On the other hand, if both SRB4 and SRB5 are not configured, this will result in QoE reports not being sent to the MCE. So we think it is necessary to discuss what should be done at this point.

Proposal 3: RAN2 is kindly asked to discuss NW and UE behavior if neither SRB4 or SRB5 is configured for QoE in NR-DC.

2.3 QoE pause/resume in NR-DC

Since the Paused QoE mechanism is introduced in R17 to solve the problem of RAN overload. In NR-DC scenario, since both MN and SN will exist, it is reasonable to report Paused QoE through SN, especially when MN overload and SN is not yet overload, and vice versa, which can help to mitigate the RAN overload as the problem. For that consideraion, we believe that MN and SN do not need to suspend or resume QoE reporting at the same time.

Proposal 4:  MN and SN are not necessarily to pause or resume QoE report simultaneously in NR-DC.

According to the previous conclusions of RAN3, the SN can send the QoE report directly to MCE, so there is no problem of re-forwarding the QoE reports to the MN.

If QoE reports are received by the SN, SN can forward the QoE reports to MCE directly.

Proposal 5:  If MN is overloaded while SN is not, UE can send QoE reports to SN according to NW’s indication, SN can send the QoE reports to MCE directly, and vice versa.

Proposal 6:  RAN2 to discuss whether needs an explicit indicator for MN/SN reporting or reuses pause Report in AppLayerMeasConfig for MN/SN overload.

3	Summary

This contribution discusses potential capacity enhancement for MBS QoE, the following are the proposals that may be taken into consideration.

QoE configuration in NR-DC aspect:

Proposal 1: MN and SN should be able to coordinate and ensure the max number of simultaneously QoE measurement collection will not exceed maxNrofAppLayerMeas-r17. FFS whether to introduce new parameter or extend maxNrofAppLayerMeas-r17.

Proposal 2: RAN2 agree to introduce an explict indicator for QoE reporting leg switching. If not, RAN2 has to ensure that SRB4 and SRB5 will not be configured simultaneously.

Proposal 3: RAN2 is kindly asked to discuss NW and UE behavior if neither SRB4 or SRB5 is configured for QoE in NR-DC.

QoE pause/resume in NR-DC aspect:

Proposal 5: If MN is overloaded while SN is not, UE can send QoE reports to SN according to NW’s indication, SN can send the QoE reports to MCE directly, and vice versa.

Proposal 6: RAN2 to discuss whether reuse the explicit indicator for MN/SN QoE reporting leg switching or reuses pauseReporting-r17 in AppLayerMeasConfig for MN/SN overload.

References

- R2-2304202, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

- RAN3_119bis-e_agenda_20230426_EOM1