3GPP TSG-RAN WG2 Meeting #122    	                             R2-2305608

Incheon, Korea, 22 - 26 May, 2023

Agenda item:	7.4.3

Source:	CMCC

Title:	Discussion on NR-DC with selective activation of cell groups

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

# 1	Introduction

In previous meeting, RAN2 discussed the scenarios, baseline procedure, delta configuration and security issue for NR-DC with selective activation of the cell groups. The following agreements in last meeting [1] were achieved and some issues were still FFS:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format. For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC Assume for now that there is only one reference configuration. The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported. For SN initiated procedure, execution conditions based on events A3/A5 are supported. UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

This contribution will further discuss the open issues and potential enhancements on NR-DC with selective activation of the cell groups.

- 2 Discussion

  - Terminology

In RAN2#120 meeting, RAN2 discussed whether a name is needed for NR-DC with selective activation of cell groups, and some terminologies were proposed, e.g. SACG (Selective Activation of Cell Groups), SAPC(Selective Activation of PSCell Change), subsequent CPA/CPC, etc. and there was no consensus on this issue.

We prefer the terminology of SACG (Selective Activation of Cell Groups), since it can be well aligned with the objective of WID[2] as following:

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

Proposal 1: RAN2 to use “SACG” as term for NR-DC with selective activation of the cell groups.

  - Condition to release SACG configuration

In RAN2#121 meeting, RAN2 agreed that the CPC/CPA configurations should be released after PCell change, at least for inter MN (by explicit indication from network) and FFS for other cases.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

As specified in TS 37.340 [3], in legacy MN/SN initiated conditional SN change procedure, MN sends to the UE a RRCReconfiguration message which possibly includes an MCG configuration. When PCell is changed, the corresponding MCG configuration will be changed. And the UE will stop evaluating the execution condition for legacy CPC once PCell change is triggered regardless of inter-MN or intra-MN PCell change.

10.5.2 MR-DC with 5GC <omitted> MN initiated conditional SN Change <omitted> - The MN sends to the UE an RRCReconfiguration message including the CPC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 2 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements. <omitted> 10.6 PSCell change <omitted> A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change or PCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported. <omitted>

For R18 selective activation of cell group, we think the similar mechanism to R17 can be reused, i.e. the stored SACG configuration should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. In addition, the explicit indication from network is not needed.

Proposal 2: The stored SACG configuration should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. The explicit indication from network is not needed.

In addition to PCell change, other condition/indication for releasing the stored SACG configuration should be discussed. In last meeting, RAN2 agreed to release CPC candidate explicitly by RRC reconfiguration procedure.

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

We understand that the legacy method can be reused to release CPC candidate, i.e. condReconfigToRemoveList IE in RRCReconfiguration as specified in TS 38.331 [4].

Proposal 3: Reuse the condReconfigToRemoveList IE in RRCReconfiguration to explicitly release the candidate SACG configuration.

In addition, the SACG procedure means that network resources need to be reserved for a longer time, which will increase the network overhead and reduce the system capacity. The validity duration of SACG configuration should be aligned between the UE and network, for example, timer-based, execution number-based, area-based or based on the network indication, etc. In RAN2#119-e meeting, the issue on how many subsequent conditional changes are targeted was discussed, we think there is a trade-off between the performance enhancement from subsequent cell groups change, network capacity and UE’s extra overhead to keep the configuration.

Anyway, the network is normally responsible for the performance of the entire network, and thereby it is reasonable to let it be controlled by network, and the network can configure different timer value or number based on its own policy for different UEs. The UE may send some assistance information to network for validity duration configuration, such as how many candidate SCGs can be supported by the UE. If the above condition is configured by network, the UE shall release the stored SACG configuration when the condition is fulfilled.

Proposal 4: It can be possible to release the stored CPA/CPC configuration when the pre-configured condition to release the stored configuration is fulfilled, e.g. timer-based, execution number-based.

Proposal 5: The UE assistance information (e.g., how many candidate SCGs can be supported by the UE) can be introduced for configuring the condition to release the stored CPA/CPC configuration.

  - Reference configuration

In last meeting, RAN2 agreed that the reference configuration is always explicitly signaled during the discussion for LTM. And for the reference configuration for SCG Selective Activation, aim at following the similar design as LTM.

Whether the Reference configuration is a complete configuration or not is up to the network implementation. Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current). The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

Proposal 6: The reference configuration is always explicitly signaled for selective activation of cell groups.

Based on the above agreements, reference configuration + candidate configuration has to be a complete configuration. In RAN2#121, it was agreed that the reference configuration can be empty in LTM discussion. Thus, we understand that if the reference configuration is empty, the candidate configuration should be full configuration.

Proposal 7: If the reference configuration is empty, the candidate configuration is full configuration.

  - Stored CPA/CPC configuration

In last meeting, RAN2 assumed that CPA configuration can be used for CPC but with different triggering conditions. In our understanding, the CPA configuration can be also used for subsequent CPA. For example, the SN will be released if there is no or few data/signaling transmission between UE and SN, so the UE will enter to SA mode, and it is expected to perform CPA procedure to go back to DC mode when high data rate arrives. And another case is that when the SCG failure happens, the CPC is not applicable since there is no serving PSCell and CPA procedure is required. Hence, if the CPA configuration is kept after the first execution of CPA, the UE can perform subsequent CPA when the condition of candidate PSCell addition is satisfied.

Observation 1: UE could return to SA due to the release of SCG or SCG failure, and the CPA configuration could be used for the UE that returns to SA.

Proposal 8: The stored CPA configuration can be used for subsequent CPA when UE returns to SA mode.

Other issue is whether the stored CPC configuration can be used for subsequent CPA. Even if the delta configuration is applied, the UE can form the complete/full configuration by applying the delta configuration on top of the reference configuration as we proposed in P6. Thus, we think when the UE returns to SA mode, the UE can use the stored CPC configuration for subsequent CPA if the execution condition of CPA for the same candidate PSCell is configured.

Proposal 9: The stored CPC configuration can be used for subsequent CPA with different triggering conditions.

  - RRC signaling structure

In last meeting, RAN2 assumed that there is only one reference configuration and is provided to the UE explicitly. During the discussion of RAN2#119bis-e meeting, the nested configuration is proposed and FFS. In our understanding, the RRC signaling structure for SACG (including reference configuration and candidate PSCell configuration) may be the following options:

Option 1: All candidate SCG configurations are in parallel in RRCReconfiguration message. An example is as follows:

{

Reference configuration

Configuration for Candidate PSCell 1

Configuration for Candidate PSCell 2

...

Configuration for Candidate PSCell N

}

Option 2: Candidate SCG Configurations can be included in the candidate SCG configuration (i.e. nested configuration). An example is as follows:

{

Reference configuration

Configuration for Candidate PSCell 1

{

Configuration for Candidate PSCell 2

{configuration for candidate PSCell A {...}, configuration for candidate PSCell B {...}}

Configuration for Candidate PSCell 3

{configuration for candidate PSCell C {...}, configuration for candidate PSCell D {...}}

}

Configuration for Candidate PSCell 4 {...}

...

}

For option 1, the signaling structure is simple and clear, and similar to legacy CPA/CPC. In addition, the signaling overhead is lower than option 2.

For option 2, there maybe a lot of configuration possibilities and will increase signaling overhead. On the other hand, as the level of nesting increases, so does the complexity. And network needs to know accurate UE trajectory for candidate PSCell configuration. In addition, if the UE is allowed to return the previous connected PSCell, the same PSCell configuration may be configured multiple times in the SACG configuration which will greatly increase the signaling overhead.

Considering the assumption with only one explicit reference configuration, the benefit of indicating the reference configuration implicitly for option 2 is lost. Thus, option 1 is preferred for selective activation cell of groups for simplicity.

Proposal 10: Nested configuration is not supported for SACG configuration, and all candidate SCG configurations are in parallel in RRCReconfiguration message.

  - Execution condition

In last meeting, RAN2 discussed whether candidate SN can generate execution conditions for subsequent CPC for MN-initiated and SN-initiated inter-SN SCG selective activation.

For MN-initiated case, we prefer to follow the R17 principle that the execution condition is generated by MN. If the candidate SN generates the execution condition, the issue is that a target SN does not know the existence of candidate PSCells belonging to other candidate SNs. To support this feature, the MN should transmit all candidate PSCells of candidate SNs to each candidate SN, which introduce additional signaling and complexity. Moreover, since the execution conditions are configured based on measConfig for MN, it is more suitable for MN to generate all execution conditions.

Proposal 11: For MN-initiated inter-SN SCG selective activation, source MN generates the execution conditions for all subsequent CPC.

For SN-initiated case, the legacy principle is that execution condition is generated by source SN. During the previous discussion, some companies think that the execution condition configured by source SN may become invalid when the UE moves to another candidate SN, since the execution condition generated by the source SN is based on the source SCG MeasConfig. And it is proposed that candidate SN can generate execution conditions for subsequent CPC. We think there are two ways to achieve it:

- Option 1: All candidate PSCells are transmitted to each candidate SN during the preparing stage, and candidate SN configures all possible execution conditions for each candidate PSCell.

- Option 2: The execution condition is updated/configured by new serving SN after each PSCell change.

Option 1 will introduce additional signaling overhead and complexity of configuration, especially when there are many candidate PSCells, candidate SN needs to configure all possible execution conditions for each candidate PSCell in the initial RRCReconfiguration message.

By comparison, the initial configuration for option 2 is simpler, but option 2 needs RRCReconfiguration after each PSCell change which is not well aligned with the objective for this sub-agenda.

Anyway, we think the network is not prevent to update SACG configuration (e.g. execution condition, candidate SCG configuration), so candidate SN generating/updating execution condition is acceptable, but it is not expected that the execution condition is updated after each PSCell change.

Proposal 12: For SN-initiated inter-SN SCG selective activation, candidate SN may generate/update the execution conditions for subsequent CPC, but it is not expected that the execution condition is updated after each PSCell change.

# 3	Conclusion

Here are the observations and proposals for NR-DC with selective activation cell of groups.

Terminology:

Proposal 1: RAN2 to use “SACG” as term for NR-DC with selective activation of the cell groups.

Condition to release SACG configuration:

Proposal 2: The stored SACG configuration should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. The explicit indication from network is not needed.

Proposal 3: Reuse the condReconfigToRemoveList IE in RRCReconfiguration to explicitly release the candidate SACG configuration.

Proposal 4: It can be possible to release the stored CPA/CPC configuration when the pre-configured condition to release the stored configuration is fulfilled, e.g. timer-based, execution number-based.

Proposal 5: The UE assistance information (e.g., how many candidate SCGs can be supported by the UE) can be introduced for configuring the condition to release the stored CPA/CPC configuration.

Reference configuration:

Proposal 6: The reference configuration is always explicitly signaled for selective activation of cell groups.

Proposal 7: If the reference configuration is empty, the candidate configuration is full configuration.

Stored CPA/CPC configuration:

Observation 1: UE could return to SA due to the release of SCG or SCG failure, and the CPA configuration could be used for the UE that returns to SA.

Proposal 8: The stored CPA configuration can be used for subsequent CPA when UE returns to SA mode.

Proposal 9: The stored CPC configuration can be used for subsequent CPA with different triggering conditions.

RRC signaling structure:

Proposal 10: Nested configuration is not supported for SACG configuration, and all candidate SCG configurations are in parallel in RRCReconfiguration message.

Execution condition:

Proposal 11: For MN-initiated inter-SN SCG selective activation, source MN generates the execution conditions for all subsequent CPC.

Proposal 12: For SN-initiated inter-SN SCG selective activation, candidate SN may generate/update the execution conditions for subsequent CPC, but it is not expected that the execution condition is updated after each PSCell change.

# 4	Reference

- Draft_R2_121bis_meeting_report_v2

- RP-222332, Revised WID on Further NR mobility enhancements

- TS 37.340 Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.331 NR; Radio Resource Control(RRC)