3GPP TSG-RAN WG2 Meeting #122       	                       R2-2305615

Incheon, Korea, 22 -26 May, 2023

Agenda item:	7.16.2.3

Source:	CMCC

Title:	Discussion on AI/ML model transfer/delivery

WID/SID:	FS_NR_AIML_air

Document for:	Discussion

- Introduction

In RAN2#121 meeting [1], RAN2 agreed to study the following potential solutions for AI/ML model transfer/delivery and use the pros/cons [2] as a starting point.

Agreed: - Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP). The table can serve as starting point for continued discussion (but contains some parts that seems non consensus, e.g. delta configuration).

|  | Pros | Cons |
| --- | --- | --- |
| Solution 1a | 6. The existing RRC signaling solutions can be reused as baseline, at least including delta signaling and segementation 9. Additional security and verification may not be necessary as the UE already established security before the transfer is initiated 11. gNB can take the control of the AIML model transfer itself, which can not be achieved by traditional UP based solution | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery 3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the gNB to send new/updated AI/ML to the UE |
| Solution 2a and 3a | 5. Service continuity on model transfer/delivery is easy to achieve compared with Solution 1a 6. Impacts on RAN2 may be limited (some companies think that LPP signalling is in RAN2 scope) | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 3. If NAS does the segmentation, it may introduce some overhead 4. (only valid for Solution 2a) CN is not a good option for later on model monitoring/activation/deactivation/fallback/update that requires less latency. The model transfer/delivery is transparent to gNB, it could be tricky to get gNB involved in the AI model LCM. It could be problematic when the network needs to be in control of what happening at the UE side and especially in two-sided models where one side of the model is intended to be located at the network side |
| Solution 1b | 1. The network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size) 2. Compared with CP-based solutions, this Solution 1b can reduces control plane overhead, reduces overhead at gNB for model delivery/transfer 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 5. Not compatible with current mobility procedure. Supporting model transfer during mobility is not so straightforward |
| Solution 2b and 3b | 1. The network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size) 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 2. CP signalling is needed to configure and initiate the model transfer from the CN 4. May be unable to support delta-model transfer/delivery based on current user plane framework |
| Solution 4 | 2. If 3GPP network can be aware of AI/ML model in this Solution 4, the network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size). How to synchronize 3GPP and server so that the network can take appropriate actions is not clear, and it may not be fully under 3GPP control | 2. There may be inter-operability issues, such as: a) Different implementations may lead to different model performances and a huge burden of model management (e.g., frequent model activation/deactivation) b) Massive offline coordination is needed or requires lots of coordinations among vendors, especially for the CSI compression use case 4. When network cannot control the model transfer/delivery, the transfer of large model may impact important and delay sensitive user data traffic |

In this contribution, we will discuss the potential issues on AI/ML model transfer/delivery.

- Discussion

In last meeting, RAN2 agreed the following relations between solutions and use cases.

Table: relations between solutions and use cases

| Solutions | Applicable use cases |
| --- | --- |
| Solution 1a, 1b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b. |
| Solution 2a, 2b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b. |
| Solution 3a, 3b | Positioning accuracy enhancement |
| Solution 4 | CSI feedback enhancement Beam management Positioning accuracy enhancement |

For CSI feedback enhancement and beam management use case, CP-based and UP-based solutions between UE and gNB/CN is proposed. However, for CN involved solution (i.e. solution 2a and 2b), we think the motivation is still unclear, the use cases (i.e. CSI feedback and beam management) is purely over air interface which only involves UE and gNB, so it is tricky for CN to collect all necessary data for model training and perform model transfer. In addition, it is also hard for CN to understand the physical parameters and determine which AI/ML model is applicable for the specific use case. On the other hand, solution 2a and 2b need more works in other WGs (e.g. SA2, CT1).

Observation 1: The motivation for CN to train the AI/ML model is still unclear and it is tricky for CN to collect all necessary data for model training for CSI feedback and beam management use cases.

Therefore, we propose to not consider CN involved solution (i.e. solution 2a and 2b) for CSI feedback enhancement and beam management in R18 SI.

Proposal 1: Solution 2a and 2b is not pursued for model transfer/delivery for CSI feedback enhancement and beam management in R18 SI.

In general, the UP-based solution can support the large size model transfer/delivery or transfer/delivery multiple models simultaneously, while the CP-based solution only is challenged due to the limited signaling size. However, for solution 1b, as many companies commented, existing user plane is not applicable since it terminates at UE and UPF. Some companies proposed that a new layer may be introduced to handle the AI model transfer functionality. This will change the basic protocol stack of NR which has significant impacts on overall structure and specs. We think the solution with basic protocol stack change is difficult to be implemented in 5G-A stage, and it is better to consider in early 6G stage. Of course, other solutions without basic protocol stack change can be further studied.

Proposal 2: For Solution 1b, the potential options with basic protocol stack change is not pursued in R18, and potential options without basic protocol stack change can be further studied for model transfer/delivery.

For solution 1a (i.e. RRC signaling between UE and gNB), the biggest challenge is to transfer a large size model, we think RAN2 can further discuss how to resolve this issue.

Proposal 3: For solution 1a, RAN2 can further discuss how to transfer a large size model via RRC signaling.

Considering the training entity and pros of UP-based solution, we understand another solution (solution 2c) may be applicable for model transfer/delivery, i.e. the model is trained at gNB and delivered to CN, then CN transfer/delivery the model to the UE via UP-based solution. With this solution, CN is not required to be aware of the physical parameters, while can support the large model transfer/delivery, gNB can also be naturally responsible for the life cycle management of AI/ML models. Although this solution has more delay compared with solution 2a and 2b, it is applicable to transfer the offline training model which is latency insensitive for all use cases. We think this potential solution can be further discussed.

Figure 1: Basic flow for solution 2c - model training at gNB, model transfer/delivery from CN to UE

Proposal 4: RAN2 take the solution 2c (i.e. the model is trained at gNB and delivered to CN, then CN transfer/delivery the model to the UE via UP-based solution) for further discussion.

For positioning accuracy enhancement, solution 3a and 3b is feasible for model transfer/delivery, since LMF has the ability to collect data from multiple gNBs and UEs for the model training. However, some companies pointed that solution 3b is only included in SA2 TR 23.700-71, and it should be specified by SA2. Thus, we prefer to leave it open and wait until SA2 finishes the normative work on the UP solution. We think RAN2 can firstly focus on solution 3a.

Proposal 5: For positioning accuracy enhancement, RAN2 focus on solution 3a firstly, and whether/how to support solution 3b can wait SA2’s progress on the UP solution.

For solution 4, it may be transparent to 3GPP and can be left to implementation. Although solution 4 has no spec impacts on model transfer/delivery, there may be some impacts on other LCM procedures, e.g. model identification, model control. The possible impacts can be further studied.

Proposal 6: RAN2 can further study the potential spec impacts on solution 4 for other LCM procedures.

# 3	Conclusion

Here are the observations and proposals for model transfer/delivery.

Observation 1: The motivation for CN to train the AI/ML model is still unclear and it is tricky for CN to collect all necessary data for model training for CSI feedback and beam management use cases.

Proposal 1: Solution 2a and 2b is not pursued for model transfer/delivery for CSI feedback enhancement and beam management in R18 SI.

Proposal 2: For Solution 1b, the potential options with basic protocol stack change is not pursued in R18, and potential options without basic protocol stack change can be further studied for model transfer/delivery.

Proposal 3: For solution 1a, RAN2 can further discuss how to transfer a large size model via RRC signaling.

Proposal 4: RAN2 take the solution 2c (i.e. the model is trained at gNB and delivered to CN, then CN transfer/delivery the model to the UE via UP-based solution) for further discussion.

Proposal 5: For positioning accuracy enhancement, RAN2 focus on solution 3a firstly, and whether/how to support solution 3b can wait SA2’s progress on the UP solution.

Proposal 6: RAN2 can further study the potential spec impacts on solution 4 for other LCM procedures.

# 4	Reference

- R2-230XXXX, Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece

- R2-2302268, Report of Offline 027 model transfer delivery (Huawei)