# 3GPP TSG-WG2 Meeting #122              	   R2-2305618

# Korea, Incheon, May 22nd - 26th, 2023

# Source: 			CMCC

# Title:  Discussion on U2U relay

# Agenda item:	7.9.2

# Document for: 	Discussion

Introduction

- Support of UE-to-UE relay is essential for the sidelink coverage extension without relying on the use of uplink and downlink. In R18 SL relay WID[1], the objective for U2U relay has been listed as follow,

- Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4].

  - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98

    - Relay discovery and (re)selection [RAN2, RAN4]

    - Signalling support for Relay and Source UE authorization if SA2 concludes it is needed [RAN3]

  - Layer-2 relay specific part

    - UE-to-UE relay adaptation layer design [RAN2]

    - Control plane procedures [RAN2]

    - QoS handling if needed, subject to SA2 progress [RAN2]

Note 1A: This work should take into account the forward compatibility for supporting more than one hop in a later release.

Note 1B: A Source UE is connected to only a single relay UE at a given time for a given destination UE.

- Meanwhile in the last #121-bis meeting, RAN2 has also made some agreements as follow:

Agreements:

For the integrated-discovery case, the relay UE forwards the discovery message for DCR message with integrated Discovery case only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

Both mode-1 and mode-2 resource allocation can be supported on both remote UE and relay UE in U2U relay case.  No impact to legacy resource allocation procedures is expected.

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

Each remote UE (source or destination) can trigger relay selection based on the direct link quality.  FFS interaction between discovery and selection.

Postpone replying to the SA2 LS on authorization.

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

WA: E2E bearer ID (i.e., configuration index in the list of SLRB configurations) is used as input for the L2 U2U relay ciphering and deciphering at PDCP.

LS to SA3 to confirm the feasibility of using the configuration index.

Based on above agreements and legacy issues, we continue to discuss some key issues and remaining FFS discussion about U2U discovery, (re)selection, adaptation layer, QoS flow and security as well.

Discussion

Discovery

SA2 has defined that (from 23.304) In Model B, Target remote UE can select to response or not to the discovery solicitation message that relay UE sent. Also SA2 has defined that source remote UE is responsible for relay UE selection in both Model A and Model B. In R17 U2N, RAN2 has defined relay selection is based on source UE implementation. Thus, in R18 U2U relay, we can have similar conclusion like R17 U2N: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

Proposal 1: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

Since source remote UE decides to select a proper relay UE, but the source remote UE does not know the second hop PC5 link condition between candidate relay UE and Target remote UE. Thus, from last meeting conclusion, target remote UE can decide whether transmitting discovery response message to the relay UE based on the PC5 RSRP threshold, which can help source remote UE to filter out unqualified relay UEs. When the source remote UE receives discovery response message from one or more relay UEs, that means the second hop PC5 link conditions between those candidate relay UEs and target UE are above the threshold and can be used for sidelink relay communication. Thus, from that point of view, target UE to transmit discovery response message according to PC5 RSRP has no separate impact on relay selection functionality.

Proposal 2: In Model B, target UE transmitting discovery response message according to PC5 RSRP has no separate impact on relay selection functionality

Relay (re)selection

For model A, we think neighbourlist is very helpful for relay (re)selection procedure, which can give lots of reference for remote UE consideration.

1) The remote UE can know all attachable ProSe UEs around the candidate relay UE.

2) Since the relay UE can know PC5 channel quality between itself and its around UEs. The relay UE can indicate PC5 channel quality in the neighbourlist for remote UE reference. Thus the remote UE can know the channel quality between candidate relay UE and the peer remote UE through the neighbourlist which is very useful for remote UE to select a suitable Relay UE among candidate relay UEs.

Proposal 3:  For model A, neighbourlist can be used for relay (re)selection procedure by having following two functions:

1) To let the remote UE know all attachable ProSe UEs around the candidate relay UE

2) To let the remote UE know the second hop channel quality between candidate relay UE and the peer remote UE

AS for whether to configure different thresholds for SL-RSRP and SD-RSRP, RAN2 has sent LS to RAN1 and RAN4 in 121 meeting. RAN1 has replied in R2-2304617 as: RAN1’s understanding is that comparison of SL-RSRP and SD-RSRP measurement cannot be used for the purposes of triggering a measurement report. Thus, since SL-RSRP and SD-RSRP can not be compared together, different thresholds for SD-RSRP and SL-RSRP are needed.

Proposal 4: Different thresholds for SD-RSRP and SL-RSRP are needed.

The adaptation layer

The most important issue we think is to reduce signalling overhead in SRAP UE ID allocation. To reduce signalling overhead, 8 bit local UE IDs can be reused just like what we did in R17 U2N relay.

Proposal 5: Local UE ID should be used in SRAP header.

As for which UE ID option in SRAP header design, we prefer containing both source and destination UE IDs in the header since it is an easier and simpler method to implement and benefit for multi hop compatibility. For the collision issue, if we use local ID allocation mechanism, it can not be avoided, we can further discuss how to reduce the impact.

- Proposal 6: Both source remote UE local ID and target remote UE local ID should be included in the adaptation layer header. FFS on how to reduce UE ID collision impact.

If local UE ID is adopted, then we need to decide which node to assign local UE ID. We think either source remote UE or relay UE is responsible for ID assignment. And don’t have a very strong opinion. Considering multi hop compatibility, source remote UE is slightly preferred for UE ID assignment.

- Proposal 7: Considering multi hop compatibility, source remote UE is slightly preferred for UE ID assignment.

QoS handling

- Although PC5-S link is responsible for other Sidelink and Sidelink relay communication, in U2U relay, since there are two separate PC5 links and without gNB involvement during data transmission. We need to consider different PC5 links condition to decide QoS split firstly regardless which hop to perform QoS split. For this reason, we prefer AS layer to perform L2 U2U QoS split.

- Proposal 8: AS layer (e.g. PC5-RRC message) is responsible for QoS split in L2 U2U relay.

- For the question of which UE to perform QoS split from last meeting offline emial discussion, both methods (Relay UE or Source remote UE take charge of QoS splitting) are feasible and all related to message interaction between source remote UE and relay UE:

- 1.For Option1, then relay UE needs to tell source remote UE about the second hop PC5 link quality.

- 2.For Option2, then remote UE needs to tell relay UE about the E2E QoS information.

- AS SA2 has decided relay UE is responsible for L3 U2U QoS split. Thus, we prefer to keep aligned with SA2 that using relay UE to perform QoS split in L2 U2U relay as well.

- Proposal 9: Relay UE is responsible for QoS split in L2 U2U relay.

Security

From last meeting post email discussion for L3 to SA3 about PDCP ciphering and deciphering, in the discussion, there is a divergence on how does Rx end remote UE PDCP layer get E2E bearer ID. One solution is that Rx UE PDCP layer get the E2E bearer ID from RRC signalling. The other solution is to get E2E bearer ID from lower SRAP layer header through E2E data transmission. Both solutions are feasible in theory. As per previous conclusion, both two hop PC5 links contain E2E bearer ID. Also considering it’s a huge signalling overhear for Rx end RRC layer to inform PDCP layer about E2E bearer ID (especially for N:N UEs connection scenario), even some connections need to set up SRB transmission. Thus, we think it’s a better and simpler way from Rx end UE PDCP layer to receive E2E bearer ID from SRAP layer through receiving Tx end UE data transmission.

- Proposal 10: Rx remote UE’s PDCP entity get E2E bearer ID from lower SRAP layer header through E2E data transmission.

Conclusions

According to the above discussion, the following proposals are given:

Discovery:

Proposal 1: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

Proposal 2: In Model B, target remote UE transmitting discovery response message according to PC5 RSRP has no separate impact on relay selection functionality

Relay (re)selection

Proposal 3:  For model A, neighbourlist can be used for relay (re)selection procedure by having following two functions:

1) To let the remote UE know all attachable ProSe UEs around the candidate relay UE

2) To let the remote UE know the second hop channel quality between candidate relay UE and the peer remote UE

Proposal 4: Different thresholds for SD-RSRP and SL-RSRP are needed.

Adaptation layer

Proposal 5: Local UE ID should be used in SRAP header.

- Proposal 6: Both source remote UE local ID and target remote UE local ID should be included in the adaptation layer header. FFS on how to reduce UE ID collision impact.

- Proposal 7: Considering multi hop compatibility, source remote UE is slightly preferred for UE ID assignment.

QoS handling

- Proposal 8: AS layer (e.g. PC5-RRC message) is responsible for QoS split in L2 U2U relay.

- Proposal 9: Relay UE is responsible for QoS split in L2 U2U relay.

Security

Proposal 10: Rx remote UE’s PDCP entity get E2E bearer ID from lower SRAP layer header through E2E data transmission.

References

- 3GPP RP-221262, WID for NR sidelink relay enhancements.

- 3GPP TR 38.836, Study on NR sidelink relay.