3GPP TSG-RAN WG2 Meeting #122                      	                                    R2-2305624

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.2.3

Source: 	CMCC

Title:  	Discussion on the RAT-dependent integrity issues

WID/SID:	         NR_pos_enh2

Document for:	Discussion and Decision

Introduction

In the RAN#121bis meeting, the RAT-dependent integrity issue have been discussed and the following items have been achieved:

Agreements:

LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

LS to RAN1 to include the question of whether RAN1 identify a need for a DNU flag for measurements.

For RAT-dependent integrity, the PL calculation is performed by the entity which also performs the position calculation for a location process.

For UE-based integrity, the integrity parameters of error sources for RAT-dependent integrity are included in assistance data.

LPP Request/Provide Assistance Data are reused for retrieving the integrity parameters to the UE from the LMF.  The request is per positioning method (as in legacy operation) and the provided integrity parameters are as appropriate for the selected positioning method.

Use of posSIBs for integrity parameters is not excluded.

For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Hence, in this contribution, we will discuss the issues on the integrity of RAT-dependent positioning methods.

Discussion

- 2.1 Integrity Signalling

Based on WID [1], both the UE-based and LMF-based integrity for RAT-dependent integrity cases have been agreed for further study.

2.2.1 Signalling for LMF-based Positioning Integrity Mode

Based on the investigation from RAN1, the error sources for LMF-based positioning integrity mode were listed in below table:

| Positioning Integrity Mode | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - RSTD measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - RTOA measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | - Angle of arrival measurement - TRP location - ARP location (e.g., ARPLocationInformation in TS 38.455 [17]) | - TRP location - DL-PRS RSRPP of the first path or RSRP |

From the table above, both the errors in assistant data and measurement should be sent to the LMF for the PL calculation in LMF-based positioning integrity mode.  Since the UE measurment error is only used in the PL calculation, which is preformed in LMF in LMF-based positioning integrity mode, UE could directly send its measurement error to LMF without the NG-RAN nodes notice. Therefore, the UE could send the measurement error generated from UE, e.g. RSTD measurement, UE Rx-Tx time difference measurement, to LMF via LPP ProvideLocationInformation message while NG-RAN could send the error in assistant data and measurement generated from NG-RAN node, e.g. TRP location, RTOA measurement, inter-TRP synchronization, gNB Rx-Tx time difference measurement to LMF via NRPPa messages. The procedure for LMF-based positioning integrity mode is listed in below Figure 1.

Figure 1. the procedure for LMF-based positioning integrity mode

Proposal 2: It is suggested that the UE could send the measurement error generated from UE, e.g. RSTD measurement, UE Rx-Tx time difference measurement, to LMF via LPP ProvideLocationInformation message while NG-RAN could send the error in assistant data and measurement generated from NG-RAN node, e.g. TRP location, RTOA measurement, inter-TRP synchronization, gNB Rx-Tx time difference measurement to LMF via NRPPa messages.

- 2.2 DNU Flag in Integrity Principle of Operation

As discussed in last RAN2 meeting, the DNU flag used in integrity principle of operation in GNSS integrity method [4] could be reused in RAT-dependent positioning integrity method. The DNU in RAT-dependent positioning integrity method is considered to indicate whether the assistant data is usable for positioning integrity calculation. In this case, the DNU flag should be provided at least for each positioning method. For the GNSS integrity, the DNU flags are used for both the GNSS satellite and its signals to indicate the stability of the GNSS satellite and its signals. For RAN integrity, the DNU flags may be provided to the assistant data, such TPRs and/or PRS resources to indicate the health of assistant data.

Proposal 3: the DNU flag used in integrity principle of operation should be provided at least for each positioning method. And the DNU flags may be provided to the assistant data, such TPRs and/or PRS resources to indicate the health of assistant data.

- Conclusion

In this contribution, the discussion on the RAT-dependent positioning integrity is provided.

Proposal 1: It is suggested that the UE could send the measurement error generated from UE, e.g. RSTD measurement, UE Rx-Tx time difference measurement, to LMF via LPP ProvideLocationInformation message while NG-RAN could send the error in assistant data and measurement generated from NG-RAN node, e.g. TRP location, RTOA measurement, inter-TRP synchronization, gNB Rx-Tx time difference measurement to LMF via NRPPa messages.

Proposal 2: the DNU flag used in integrity principle of operation should be provided at least for each positioning method. And the DNU flags may be provided to the assistant data, such TPRs and/or PRS resources to indicate the health of assistant data.

Reference

- [1] RP-223549 New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson

- [2] TR 38.859 V18.0.0, Study on expanded and improved NR positioning (Release 18), 2022-12

- [3] TS 37.355 V17.3.0, LTE Positioning Protocol (LPP) (Release 17), 2022-12

- [4] TS 38.305 V17.3.0, User Equipment (UE) positioning in NG-RAN (Release 17), 2022-12