3GPP TSG-RAN WG2 Meeting #122                      	                                    R2-2305626

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.5.3

Source:	CMCC

Title:	Discussion on the DRX enhancement

WID/SID:	             NR_XR_enh-Core

Document for:	Discussion

Introduction

In the RAN2#121bis meeting, the XR-specific power saving issue has been discussed as below:

RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

Therefore, we provide our views on the DRX enhancements for XR in this document.

Discussion

Comparison of the Non-integer DRX periodicity schemes

In the last RAN2 meeting, several companies proposed their non-integer DRX periodicity schemes.  It is agreed that the multiple active DRX configurations solution is not considered in this version.

The left schemes include as below:

- configure multiple DRX cycles which to be used in a round , e.g., three DRX cycles including 16ms, 17ms, 17ms to be used in a round cycle like 50ms, which can align with 60fps XR traffic.

- configure multiple DRX ON starting offsets, e.g., drx-SlotOffset in a DRX cycle and UE needs to start drx-onDurationTimer more than once in one DRX cycle to align with XR traffic.

- Support rational numbers for DRX cycles [1] & [2]

In this document, we list the comparisons based on these schemes.

| Non-integer DRX periodicity schemes | Pros and Cons |
| --- | --- |
| Non-uniform DRX periodicity in a round | Introduce the DRX periodicity round with multiple DRX cycles. Pros: No formula design was needed. The current RRC spec needs to support the configuration of the DRX periodicity round with multiple DRX cycles, which is flexible and simple. Cons: maybe need to extend the number of DRX cycles in one DRX periodicity round in future due to the introduction of the new frame rate for XR services. |
| Non-uniform DRX drx-StartOffset in one DRX cycle | Introduce the multiple DRX drx-StartOffset in one DRX cycle. Pros: No formula design was needed. The current RRC spec needs to support the configuration of multiple DRX drx-StartOffset in one DRX cycle, which is flexible and simple. Cons: maybe need to extend the number of drx-StartOffset in one DRX periodicity round in future due to the introduction of the new frame rate for XR services |
| Support rational numbers for DRX cycles [1] & [2] | Introduce the rational number for DRX cycles as dividend and divisor. Pros: may support any rational number. Cons: introduce the modulo operation or the modified modulo operation to deal with the rational number for DRX cycles in DRX formula. The workload for formula design may be complicated and take huge effect. |

For above schemes, the workload for formula design may be complicated and take huge effect; therefore it’s better to avoid formula enhancements for the periodicity mismatch issue. The schemes mentioned in this document configure the multiple values for existing DRX parameters and keep the DRX mechanism formula unchanged. We recommend using multiple values for a certain DRX parameter to solve the match between C-DRX periodicity and XR traffic cycle.

Proposal 1: Use multiple values for a certain DRX parameter to solve the match between C-DRX periodicity and XR traffic cycle. The following enhanced DRX schemes can be considered for alignment between CDRX and XR traffic and multiple XR data flow.

- Scheme #1: Configure multiple DRX cycles which used in a round;

- Scheme #2：Configure multiple drx-StartOffset values in one DRX cycle.

In the schemes listed in this document, the way of multiple DRX configurations, at least multiple values for a certain DRX parameter is good to solve the mismatch between C-DRX periodicity and XR traffic cycle, since the multiple XR flows in one XR application/traffic could also utilize the multiple DRX value to satisfy the different traffic periodicities.

Proposal 2: One DRX configuration template with multiple values for specific parameters provided can be used to achieve multiple DRX configurations for XR traffic.

For the switching of DRX configurations, we think if the appropriate values are configured to UE, UE can apply them in the proper time. Besides, since there can be jitter for the traffic arrival, DCI based indication for the DRX configuration adjustment is needed in this case, and it can also be used for UE to switching the DRX configuration.

Proposal 3: UE can switch the DRX configuration autonomous with appropriate configuration provided.

Conclusions

In this contribution, we analyse the open issues of XR DRX enhancements Following is our observations and proposals.

Proposals:

Proposal 1: Use multiple values for a certain DRX parameter to solve the match between C-DRX periodicity and XR traffic cycle. The following enhanced DRX schemes can be considered for alignment between CDRX and XR traffic and multiple XR data flow.

- Scheme #1: Configure multiple DRX cycles which used in a round;

- Scheme #2：Configure multiple drx-StartOffset values in one DRX cycle.

Proposal 2: One DRX configuration template with multiple values for specific parameters provided can be used to achieve multiple DRX configurations for XR traffic.

Proposal 3: UE can switch the DRX configuration autonomous with appropriate configuration provided.

References

- R2-2301372 C-DRX enhancements for XR, MediaTek Inc

- R2-2300188 DRX enhancements for XR, Qualcomm Incorporated