3GPP TSG-RAN WG2 Meeting #122	R2-2305628

Incheon, Korea, 22th – 26th May 2023

Agenda Item:	7.3.2

Source:	CMCC

Title:	Discussion on cell DTX/DRX

Document for:	Discussion

Introduction

In the RAN2#121bis-e meeting, cell DTX/DRX configuration and the behaviour of UE and gNB were discussed. Following are the agreements [1]:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

In this contribution, we provide our view on the open issues of SR transmission, Cell DTX/DRX parameters and alignment between Cell DTX/DRX and UE DRX.

Discussion

SR transmission

In last meeting, the gNB and UE behaviour during the non-active period of cell DTX/DRX were discussed, and there’s no consensus on whether SR can be transmitted during Cell DRX non-active period to support high priority traffic.

SR is used for requesting UL-SCH resources for new transmission for an RRC_CONNECTED UE. The MAC entity may be configured with zero, one, or more SR configurations, and each SR configuration corresponds to one or more logical channels, which may have different logical channel priorities. And it’s spontaneous to configure logical channels with same or similar priority to the same SR configuration. Based on the QoS requirements of the logical channel associated with the SR and its priority, gNB decides the scheduling priority and allocates suitable UL resources for the UE.

Observation 1: One SR configuration may be associated with high priority logical channel(s).

Observation 2: Based on the QoS requirements of the logical channel associated with the SR and its priority, gNB decides the scheduling priority and allocates suitable UL resources for the UE.

We don’t think within a cell supporting NES there’s no UE have traffic with high priority. Though the cell DTX/DRX applying may considered the traffic load status, the uplink traffic cannot be predicted in some case, and SR may be transmitted.

Observation 3: There could be uplink high priority traffic when cell DTX/DRX is applied.

The ideal case is that SR is transmitted during the cell DTX active period, but it should be noticed that the PUCCH resource for SR transmission is a periodical resource, and the maximum periodicity of SR resource is 80ms (taking SCS=15kHz as an example, the maximum periodicity is sl80, that is 80ms.) Though not all SR resources will be used, but to support high priority traffic and assure SR is in the Cell DTX active period, the maximum periodicity of Cell DTX can be configured is 80ms. However, the motivation of cell DTX/DRX is to provide more opportunity for the cell to “sleep”, and the maximum periodicity with 80ms is not suitable. In other words, to assure SR of high priority traffic falling into the Cell DRX active period will cause the deterioration of network energy saving gain.

Observation 4：Since the SR resource periodicity is limited, with approach of assuring SR of high priority traffic falling into the Cell DRX active period will cause the deterioration of network energy saving gain.

SchedulingRequestResourceConfig information element [2]

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-START

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                                                       OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                                                OPTIONAL    -- Need M

}

SchedulingRequestResourceConfigExt-v1610 ::=   SEQUENCE {

phy-PriorityIndex-r16                       ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need M

...

}

SchedulingRequestResourceConfigExt-v1700 ::=    SEQUENCE {

periodicityAndOffset-r17                        CHOICE {

sl1280                                          INTEGER (0..1279),

sl2560                                          INTEGER (0..2559),

sl5120                                          INTEGER (0..5119)

}                                                                                                       OPTIONAL   -- Need M

}

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-STOP

-- ASN1STOP

SchedulingRequestResourceConfig field descriptions

periodicityAndOffset SR periodicity and offset in number of symbols or slots (see TS 38.213 [13], clause 9.2.4) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl SCS = 480 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl SCS = 960 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl, 5120sl sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities 2sym, 7sym and sl1 the UE assumes an offset of 0 slots. If periodicityAndOffset-r17 is present, any previously configured periodicityAndOffset (without suffix) is released, and vice versa.

phy-PriorityIndex Indicates whether this scheduling request resource is high or low priority in PHY prioritization/multiplexing handling (see TS 38.213 [13], clause 9.2.4). Value p0 indicates low priority and value p1 indicates high priority.

resource ID of the PUCCH resource in which the UE shall send the scheduling request. The actual PUCCH-Resource is configured in PUCCH-Config of the same UL BWP and serving cell as this SchedulingRequestResourceConfig. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1 (other formats not supported) (see TS 38.213 [13], clause 9.2.4)

schedulingRequestID The ID of the SchedulingRequestConfig that uses this scheduling request resource.

One reason of not supporting SR transmission of SR during non-active period of Cell DTX is to maximize the network energy saving gain, but it should not be achieved by sacrificing the UE experience. Besides, other common signalling is transmitted/received by the gNB, such as SIB, paging and RACH. That means the cell is not really in “sleeping” mode and signalling with necessity can be transmitted during non-active period of Cell DTX/DRX.

Based on the discussion above, we propose that traffic with high priority can transmit SR during the non-active period of Cell DTX/DRX. In this way, gNB may have more flexibility to do the trade-off between high priority traffic assurance and network power saving gain.

Proposal 1: Traffic with high priority can transmit SR during the non-active period of Cell DTX/DRX.

Cell DTX/DRX parameters

In last meeting, RAN2 agreed that the Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration. During the cell DTX/DRX, the concept of cell active or non-active period is use, whether active period is only the on duration is not clear. In our understanding, the parameters for Cell DTX/DRX have similar meanings as UE C-DRX. For example, the on duration is a fixed period within a cell DTX/DRX periodicity in which the cell is in active period.

Proposal 2: On duration is a fixed period within a cell DTX/DRX periodicity in which the cell is in active period.

Besides, in UE C-DRX, the actual active time for a UE may be longer than on duration, due to the start of drx inactivity timer for a new UL/DL transmission or the start of drx retransmission timer for a retransmission. In other words, the on duration may be extended.

In RAN2#121 meeting, there’s an FFS about inactivity timer: FFS whether we have DTX UE specific inactivity timer [3]. Considering that there’s possibility that UE is scheduled with a new transmission or a retransmission in the last slot of the on duration of cell DTX/DRX, it’s better to extend the fixed length on duration to assure the completion of the new transmission or retransmission. In other words, inactivity timer can be introduced in this case. Whether it is only for new transmission or it can also be used for retransmission can be further discussed.

Proposal 3: Inactivity timer is introduced for extend the fixed length on duration for a better transmission assurance. Whether it is only used for a new transmission or a it can also be used for retransmission can be further discussed.

Alignment between cell DTX/DRX and UE DRX

When both cell DTX/DRX and UE DRX are configured, if they are not aligned, the power saving gain or UE experience will be reduced.

For example, as shown in Figure 1, different UEs have different or same DRX cycles cycle but different start offsets, then these UEs will be scheduled dispersedly in time. When cell DTX/DRX are configured, the active time of some UEs will fall out of on-duration time of gNB, then such UEs will not be scheduled, unless a longer on-duration timer of cell DTX/DRX is configured, while it may cause the power saving gain of cell DTX/DRX to be limited.

Figure 1

To maximize the energy saving gain, we need to try best to make the alignment of UE and networks’ on-duration time. To achieve this, one alternative is to reconfigure the UE DRX parameters, such as drx-LongCycleStartOffset, to align the cycle and start offset of UE DRX with cell DTX/DRX. However, this takes longer time for the RRC reconfiguration parameters to take effect. And once the traffic load increases, gNB has to reconfigure those parameters again to distribute the UE’s active time.

Another alternative is to use dynamic signaling. The alignment of UE DRX can be done along with L1 activation and deactivation of cell DTX/DRX. As shown in Figure 2, when gNB decides to start cell DTX/DRX, it sends a dynamic indication to inform UE about the activation of cell DTX/DRX, and along with the dynamic activation, all UEs will update the DRX parameters such as DRX cycle, drx-startoffset, or drx-slotoffset to a common start offset, common DRX cycle, common slot offset value configured by the network, or the common parameters share same value with cell DTX/DRX. As shown in Figure 2, all UE use the same offset and cycle as cell DTX/DRX, and the on duration timer can share the same value or different values.

By this dynamic way, gNB can turn on/off the cell DTX/DRX flexibly, and also align the active time of gNB and UEs, therefore, the power saving gain can be increased, and it can react to dynamic UE traffic varying. So, we make the following proposals.

Proposal 4: Alignment of cell DTX/DRX and UE DRX can be triggered dynamically.

Proposal 5: Activation of cell DTX/DRX and alignment of cell DTX/DRX and UE DRX can share the same L1 indication.

Figure 2 Dynamic alignment of UE DRX with cell DTX/DRX

Conclusions

In this contribution, we analyse the open issues related to cell DTX/DRX. Following are our observations and proposals.

Observations:

Observation 1: One SR configuration may be associated with high priority logical channel(s).

Observation 2: Based on the QoS requirements of the logical channel associated with the SR and its priority, gNB decides the scheduling priority and allocates suitable UL resources for the UE.

Observation 3: There could be uplink high priority traffic when cell DTX/DRX is applied.

Observation 4：Since the SR resource periodicity is limited, with approach of assuring SR of high priority traffic falling into the Cell DRX active period will cause the deterioration of network energy saving gain.

Proposals:

Proposal 1: Traffic with high priority can transmit SR during the non-active period of Cell DTX/DRX.

Proposal 2: On duration is a fixed period within a cell DTX/DRX periodicity in which the cell is in active period.

Proposal 3: Inactivity timer is introduced for extend the fixed length on duration for a better transmission assurance. Whether it is only used for a new transmission or a it can also be used for retransmission can be further discussed.

Proposal 4: Alignment of cell DTX/DRX and UE DRX can be triggered dynamically.

Proposal 5: Activation of cell DTX/DRX and alignment of cell DTX/DRX and UE DRX can share the same L1 indication.

References

- R2_121bis_meeing_report;

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.

- R2_121_meeting_report_v1;