3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2305629

Incheon, Korea, 22th – 26th May 2023

Agenda Item:	7.3.5

Source:	CMCC

Title:	Discussion on Connected mode mobility

Document for:	Discussion

Introduction

In RAN2#121bis meeting, connected mode mobility was discussed, following are the agreements achieved in last RAN2 meeting [1]:

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

In this contribution, we discuss the CHO enhancement for serving cell and target cell.

Discussion

Enhancements for source cell

NES mode definition

In the offline discussion of connected mode mobility, the definition of NES mode was discussed, but there’s no consensus. In our view, the source cell entering NES mode means the source cell is applying a NES technique discussed in the WI, or the cell is turning off. But with different NES technique, the CHO enhancements may have some difference.

Proposal 1: The source cell entering NES mode means the source cell is applying a NES technique or the source cell is turning off.

Procedure

Based on the current agreement, the basic procedure for CHO considering NES can be as following:

- CHO configuration including the execution condition and configurations in the candidate cells is configured to UE earlier than the source cell entering NES mode;

- UE starts evaluation of the CHO execution condition;

- UE accesses to a candidate cell.

However, it is not clear whether the UE’s access to candidate cell is due to the network indication, or it’s based on source cell’s indication. In our perspective, how UE makes the decision depends on which event is configured for the CHO considering source cell entering NES mode.

Observation 1: whether the handover is based on the execution condition or source cell indication is based on the events of execution condition.

In Rel-16, the CHO execution conditions (Event A3/A5) are all measurement-based events. In Rel-17, one extra measurement-based event (Event A4), one timer-based trigger condition (Event T1) and one location-based trigger condition (Event D1) were introduced in NTN. The time-based or location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5), but it is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event [2][3].

If only measurement-based event is configured to UE, as legacy procedure, UE will evaluate the execution condition immediately when it receives the CHO configuration. But considering the motivation is to handover UEs before the source cell entering NES mode, while only measurement-based event is configured to UE may lead to UE’s “late” handover, which means the execution condition is not satisfied when the source cell enters NES mode, and this does not align with the original motivation. In this case, the extra signalling besides RRC reconfiguration is needed to trigger UE’s handover, and the UE’s evaluation results may help UE to choose a suitable candidate cell as the target cell.

Observation 2: With only measurement-based event, the UE’s CHO may be late than the time than the serving cell entering NES mode, which have effect on the NES gain.

However, if timer-based trigger condition is configured to UE, since it needs to work with RRM measurement-based event, UE will evaluate the channel quality and perform the handover at the right time. In this case, with proper timer-based trigger condition, UE may perform the handover before the source cell enters NES mode. And in this case, extra signalling is not needed to trigger the CHO execution. However, as specified in [2], it is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event, which means UE may not evaluate the measurement-based events upon receiving the CHO configuration, especially when the CHO configuration is delivered to UE much earlier than the source cell entering NES mode.  It’s considerable to introduce extra signalling to trigger UE’s evaluation to reduce UE’s power consumption.

Proposal 2: In case only measurement-based event is configured, extra RRC signalling is needed to trigger CHO execution to avoid UE’s late handover when the source cell enters NES mode.

Proposal 3: In case timer-based trigger event is configured, no extra signalling is needed to trigger CHO execution when the source cell enters NES mode.

Proposal 4: In case timer-based trigger event is configured, extra signalling can be considered to trigger UE’s measurement-based events evaluation.

Enhancements for target cell

When it comes to NES, CHO enhancement is to offload the UEs in the cell entering the NES mode, but the service continuity should be assured as legacy handover procedure. Therefore, UE should avoid to handover to a candidate cell entering NES mode.

In legacy CHO, target cell/candidate cell may be chosen by the source cell based on UE’s measurement results and other factors, thus, the source cell can exclude some cells in NES mode in the candidate cell selection based on neighbour cell’s NES mode. But it should be noticed this way would introduce limitation for candidate cell selection, and fewer options are provided for UE’s selection. Therefore, indicating the candidate cells’ NES mode information is reasonable. But it should be noticed this way would introduce limitation for candidate cell selection, and fewer options are provided for UE’s selection. Therefore, indicating the candidate cells’ NES mode information is reasonable. And from our perspective, serving cell pre-selection of candidate cells and indication of candidate cells’ NES mode can be used together. What’s more both the two ways based on the Xn information exchange between the serving cell and the candidate cells.

Proposal 5: The source cell can exclude some cells in NES mode in candidate selection phase.

Proposal 6: Candidate cell’s NES mode or the applied NES technique can be indicated to UE for UE’s target selection.

Proposal 7: The NES mode information should be exchanged between cells.

Conclusions

In this contribution, we the definition of NES mode and the potential enhancements in source cell and target cell. Following are our observations and proposals:

Observations:

Observation 1: whether the handover is based on the execution condition or source cell indication is based on the events of execution condition.

Observation 2: With only measurement-based event, the UE’s CHO may be late than the time than the serving cell entering NES mode, which have effect on the NES gain.

Proposals:

Proposal 1: The source cell entering NES mode means the source cell is applying a NES technique or the source cell is turning off.

Proposal 2: In case only measurement-based event is configured, extra RRC signalling is needed to trigger CHO execution to avoid UE’s late handover when the source cell enters NES mode.

Proposal 3: In case timer-based trigger event is configured, no extra signalling is needed to trigger CHO execution when the source cell enters NES mode.

Proposal 4: In case timer-based trigger event is configured, extra signalling can be considered to trigger UE’s measurement-based events evaluation.

Proposal 5: The source cell can exclude some cells in NES mode in candidate selection phase.

Proposal 6: Candidate cell’s NES mode or the applied NES technique can be indicated to UE for UE’s target selection.

Proposal 7: The NES mode information should be exchanged between cells.

References

- R2_121bis_report

- 3GPP TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 17).

- 3GPP TS 38.331 NR; Radio Resource Control (RRC); Protocol specification.