3GPP TSG-RAN WG2 Meeting #122	R2-2305630

Incheon, Korea, 22th – 26th May 2023

Agenda Item:	7.4.4

Source:	CMCC

Title:	Discussion on CHO including target MCG and candidate SCGs for CPC/CPA

Document for:	Discussion

Introduction

According to the WID [1], CHO including target MCG and candidate SCG for CPC/CPA is one of the Rel-18 NR mobility objectives for standardization:

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

In the previous discussion, RAN2 has achieve consensus on the evaluation order and execution order of CHO and CPAC in this case：

RAN2#121bis meeting agreements: - For the CHO+CPC case: When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions. RAN2#121 meeting agreements： RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18 The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

However, there are still some open issues, such as signalling structure, procedure etc., in this contribution, we provide our view on these open issues.

Discussion

Scenario

According to the WID, both CPA and CPC are introduced in the CHO configuration to achieve a quick dual connectivity after handover to mitigate the throughput impact due to the poor radio link quality of the conditionally-configured PSCell when CHO and MR-DC is configured together. However, it’s not clear whether both CPA and CPC is reasonable in this scenario.

The difference for CPA and CPC is originated due to whether UE is served with dual connectivity. For Conditional PSCell addition, it is defined as that a PSCell addition procedure that is executed only when the PSCell addition condition(s) are met, which is based on the DC SN addition procedure. UE is served from one single connectivity to dual connectivity without PCell/MN change after a successful CPA/SN addition.

Similarly, conditional PSCell Change is a PSCell change procedure that is executed only when PSCell execution condition(s) are met, and it is based on the SN modification or SN change procedure. No matter which procedure is used, UE is served with the same PCell/MN before and after the procedure.

Observation 1: For Rel-16/17 CPA and CPC, the PCell/MN is not changed.

When it comes to R18 CHO including target MCG and candidate SCGs, the discussion should also consider whether dual connectivity is configured for the UE when CHO with target and MCG and candidate SCG is requested and then configured to the UE.

For the case that a UE is served with single connectivity first, CHO including target MCG and candidate SCG means a new PCcell associated with the specific candidate MCG is added, from both network perspective and UE perspective. Therefore, CPA is a rational case in this scenario.

As to the other case that UE is served with dual connectivity first, it’s easy to treat this case as CPC configured with CHO. However, from network perspective, CPC is not reasonable in this case. As mentioned in Observation 1, for both CPA and CPC currently, the precondition is that Pcell/MN is not changed upon the procedure completion. For CHO including target MCG and candidate SCGs for CPC/CPA, candidate SCG is configured for the candidate MCG, but not for the source MCG, in other words, the MCG is changed, and this does not align with the current spec. In some sense, it is a CPA procedure from network perspective. In order to make CPC in this case reasonable, one way is to redefine the definition of CPC, since from UE perspective, it’s PScell change. And the other way is to associate the candidate MCG with the source SCG or a new SCG for the incoming CPC, though it’s MCG’s decision whether to keep or change the SN in TS 37.340, which introduces too much restriction and complexity. Compared with the two solutions, we prefer to redefine the CPC definition to simplify the discussion. The definition of Conditional PScell change definition in TS 37.340 can de revised as “a PSCell change procedure that is executed only when PSCell execution condition(s) are met with or without PCell change”.

Proposal 1: CPA should be discussed in CHO including target MCG and candidate SCG.

Proposal 2: The current definition of CPC is extended to include the PCell change case to support the scenario of CHO including target MCG and candidate SCG.

Proposal 2b: The definition of CPC is revised as “a PSCell change procedure that is executed only when PSCell execution condition(s) are met with or without PCell change”.

Besides, most current agreements focus on CPC case, while as we discussed above, CPA is a rational case, therefore, the agreements need to be updated to include CPA case.

Proposal 3: Current agreements including evaluation order and execution order for CHO including target MCG and candidate SCG is updated to include CPA case but not only CPC case:

RAN2 agrees to support the simultaneous evaluation of CHO and CPC/CPA in Rel-18

- For the CHO+CPC/CPA case:

When both CHO and CPC/CPA conditions are met, both CHO and CPC/CPA cell change is executed.

Baseline: The UE waits until both CHO and CPC/CPA conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

Rel-18 CHO execution

In last meeting, the evaluation order of CHO and CPAC was discussed, and it was agreed that for the case when both CHO and CPC conditions are met, both CHO and CPC cell change is executed. For the case that CHO execution condition is satisfied while CPAC is not satisfied, there are two solutions:

Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

The baseline solution allows only MCG/PCell change without SCG/Pscell change since Pcell is responsible to the coverage and it’s necessary to ensure UE to have valid connection and avoid failure happens. Considering the motivation of this objective of mobility WI is to achieve dual connectivity for the UE when CHO complete and increase the throughput, while the baseline solution may cause that only MCG/Pcell changes, and the resource reservation for CPA/CPC is wasted in some sense. Besides, the network may configure Rel-18 CHO including target MCG and candidate SCG only, UE will delay the PCell change when the CPAC condition is not met, while this delay may result in RLF.

To avoid the RLF due to wait for CPAC execution condition satisfying mentioned above, network can configure an extra threshold or condition for UE to execute CHO part configured with CPAC. For example, when UE finds the channel quality is above the threshold, UE can continue waiting for CPAC execution condition met, else, UE perform CHO part configuration and handover to the target PCell.

As to the CPC/CPA execution condition valuation when CHO execution condition is met, based on TS 38.300, the UE releases stored CHOconfigurations after successful completion of RRC handover procedure, therefore, at least the CPA/CPC execution condition can be evaluated during the period from CHO execution to handover complete. And considering that the CPA/CPC configuration is specific to the candidate MCG, we think it’s acceptable to be evaluated after access to the new serving cell to avoid resource wastes and Xn interaction, especially when the execution condition of CPAC is decided by candidate MN, and it can be configurable.

Proposal 4: To avoid RLF due to waiting for CPAC execution condition met, an RSRP threshold or another condition can be configured for the Rel-18 CHO execution, i.e., UE doesn’t wait for the CPAC condition met.

Proposal 4b: If only CHO configuration of Rel-18 CHO is executed, there are two possible  options for the CPAC evaluation：

Option 1: CPAC is evaluated during the period from CHO execution to handover complete.

Option 2: Besides the evaluation in Option1, CPAC is further evaluated after the handover complete, and this can be under network configuration.

Signalling structure

In both CHO and CPAC, the UE is configured with CHO/CPAC configurations which includes execution condition associated with RRCreconfiguration of the candidate cell. The main IE looks as follows:

- CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated condExecutionCond/condExecutionCondSCG and condRRCReconfig.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement or MN initiated inter-SN CPC. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For CHO, if network configures condEventD1 or condEventT1 for a candidate cell network configures a second triggering event condEventA3, condEventA4 or condEventA5 for the same candidate cell. Network does not configure both condEventD1 and condEventT1 for the same candidate cell.

condExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated inter-SN CPC. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For each condReconfigId, the network always configures either condExecutionCond or condExecutionCondSCG (not both).

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |

Since CPA here is triggered due to CHO in some sense, therefore, it’s spontaneous to treat CPA configurations as part of CHO configurations included in condRRCReconfig, however, in current spec the RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config, and in last meeting, RAN2 has agreed to support the simultaneous evaluation of CHO and CPAC in Rel-18 and UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure. Therefore, to achieve the simultaneous evaluation and avoid unpacking, at least the condExecutionCond for CPA should not be included in the condRRCReconfig for CHO.

Observation 2: To achieve the simultaneous evaluation and avoid unpacking, at least the condExecutionCond for CPA should not be included in the condRRCReconfig for CHO.

Moreover, the next question is whether condRRCReconfig for CPA should be separated from condRRCReconfig for CHO. Based on the IE design for conditional reconfiguration, it can be seen that the condReconfigId is used to identify the execution condition and configurations of a candidate cell of CHO/CPAC.

Considering the association between the CHO and CPA procedure in R18, one possible solution is to use the same condReconfigId to associate the condExecutionCond for CHO and condExecutionCond for CPA, while the condRRCReconfig for CPA is included in condRRCReconfig for CHO, as shown in Figure 1 (a). Via this way, the association relationship can be indicated in an implicit way. And the other solution is to reuse the current IE design, and introduce an explicit indication to indicate the association of CHO candidate and CPA candidate, as shown in Figure 1 (b). Compared to the former solution, the latter one is clearer and straightforward to standard the related UE behaviour. Besides, as we agreed the baseline solution for CHO and CPAC execution, in which a complementary CHO configuration may be configured to achieve CHO-only execution, the latter way can avoid duplicate configuration and extra signalling overhead.

Figure 1 signalling structure for simultaneous evaluation

Proposal 5: It is proposed to use the current IE design, while configuration of candidates for CHO and candidates for CPA are separately configured for UE and an explicit indication is introduced to associate the CPA candidates to the right CHO candidate.

Procedure

In current spec, CHO configuration of a specific candidate cell composes of execution condition generated by serving cell and configurations generated by the candidate cell. And it’s the same situation for CPA that the execution condition for PScell addition is generated by MN (serving cell), while the candidate cells provided the potential SCG configurations.

Observation 3: In both CHO and CPA, the execution condition is generated by MN (serving cell).

Based on this, in the case of CHO including target MCG and candidate SCGs for CPAC, it’s rational that, the source node generates the execution condition for CHO. As we discussed in Section 2.1, from network perspective, CPC in CHO CHO including target MCG and candidate SCGs is actually an CPA procedure, therefore, the target MCG can generate the execution condition for CPAC, which can simplify the design. Besides, the MCG configurations including RACH resources for CHO is also generated by the target MCG, and target SCG generates the SCG configurations including RACH resources.

Proposal 6: In CHO including target MCG and candidate SCGs for CPA:

- CHO execution condition is generated by the source node；

- CHO RACH resource is generated by target MCG(s);

- CPAC execution condition is generated by target MCG(s);

- CPAC RACH resources is generated by target SCG(s).

Based on TS 37.340, in SN addition (including CPA) procedure, the MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message, without modifying it. Similarly, source node will not modify the CHO configuration generated by CHO candidate cells.

Observation 4: Conditional configuration of CPA and CHO generated by candidate cells cannot be modified by other nodes.

What’s more, in Rel-16/17 CHO and CPA/CPC discussion, the maximum number of conditional candidates is strict to 8, considering the limitation of the UE’s measurement capability. With simultaneous evaluation of CHO and CPA, the UE may need to evaluate and measure more objectives, RAN2 is kindly asked whether the related UE capability, i.e., measurement capability like maximum measurement ID should be enhanced. And from our point of view, the UE’s capability is also one factor for the network to decide whether CHO including target MCG and candidate SCGs for CPC/CPA can be configured.

Proposal 7: The related UE capability, i.e., measurement capability like maximum measurement ID is enhanced for simultaneous evaluation.

Besides, though the CPA here is initiated from the target MN, and the target MN provides the execution conditions for candidate SN, CPA in this case is go with CHO, which is initiated by source MN/serving cell. Thus, we think source MN can take more responsibilities, for example, indicate to target MNs whether CPA can be configured and provide some necessary information for the further CPA procedure.

Proposal 8: Source MN can indicate candidate MN whether CPA can be configured and provide some necessary information for further CPA procedure, while candidate MN decides its CPA candidates (candidate SNs/SCGs) and provide CPA configurations, including the execution conditions and RACH resources from candidate SNs to candidate MN.

Based on the discussion above, the following procedure is proposed for CHO including target MCG and target SCG for CPA as depicted in Figure 1:

Figure 1 CHO including target MCG and SCG for CPA

- The source MN make the decision of CHO and send CHO request, maybe based on the UE’s measurement report, the UE’s capability, etc. There may be CPA indication together with CHO request, to indicate whether CPA can be configured and the maximum number of the target SCG can be configured.

- The target MN (T-MN) coordinate with target SNs to provide CPA configuration (and this may be based on source MN’s indication and/or its own decision).

- The target MN provides CHO and CPA configurations to the source MN.

- In case UE is served with dual connectivity before CHO, there’s release of the source SN (S-SN).

- The source MN provides CHO and CPA configurations to UE.

- The UE evaluates CHO and CPAC execution conditions simultaneously.

- UE performs CHO execution only or both CHO and CPA execution.

Proposal 9: It’s proposed that RAN2 captures the above figure and procedure in TS 37.340.

Conclusions

In this contribution, we analyse the open issues of open issues of CHO including target MCG and candidate SCGs for CPC/CPA. Following is our observations and proposals.

Observations:

Observation 1: For Rel-16/17 CPA and CPC, the PCell/MN is not changed.

Observation 2: To achieve the simultaneous evaluation and avoid unpacking, at least the condExecutionCond for CPA should not be included in the condRRCReconfig for CHO.

Observation 3: In both CHO and CPA, the execution condition is generated by MN (serving cell).

Observation 4: Conditional configuration of CPA and CHO generated by candidate cells cannot be modified by other nodes.

Proposals:

Proposal 1: CPA should be discussed in CHO including target MCG and candidate SCG.

Proposal 2: The current definition of CPC is extended to include the PCell change case to support the scenario of CHO including target MCG and candidate SCG.

Proposal 2b: The definition of CPC is revised as “a PSCell change procedure that is executed only when PSCell execution condition(s) are met with or without PCell change”.

Proposal 3: Current agreements including evaluation order and execution order for CHO including target MCG and candidate SCG is updated to include CPA case but not only CPC case:

RAN2 agrees to support the simultaneous evaluation of CHO and CPC/CPA in Rel-18

- For the CHO+CPC/CPA case:

When both CHO and CPC/CPA conditions are met, both CHO and CPC/CPA cell change is executed.

Baseline: The UE waits until both CHO and CPC/CPA conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

Proposal 4: To avoid RLF due to waiting for CPAC execution condition met, an RSRP threshold or another condition can be configured for the Rel-18 CHO execution, i.e., UE doesn’t wait for the CPAC condition met.

Proposal 4b: If only CHO configuration of Rel-18 CHO is executed, there are two possible  options for the CPAC evaluation：

Option 1: CPAC is evaluated during the period from CHO execution to handover complete.

Option 2: Besides the evaluation in Option1, CPAC is further evaluated after the handover complete, and this can be under network configuration.

Proposal 5: It is proposed to use the current IE design, while configuration of candidates for CHO and candidates for CPA are separately configured for UE and an explicit indication is introduced to associate the CPA candidates to the right CHO candidate.

Proposal 6: In CHO including target MCG and candidate SCGs for CPA:

- CHO execution condition is generated by the source node；

- CHO RACH resource is generated by target MCG(s);

- CPAC execution condition is generated by target MCG(s);

- CPA RACH resources is generated by target SCG(s).

Proposal 6: In CHO including target MCG and candidate SCGs for CPA:

- CHO execution condition is generated by the source node；

- CHO RACH resource is generated by target MCG(s);

- CPAC execution condition is generated by target MCG(s);

- CPAC RACH resources is generated by target SCG(s).

Proposal 7: The related UE capability, i.e., measurement capability like maximum measurement ID is enhanced for simultaneous evaluation.

Proposal 8: Source MN can indicate candidate MN whether CPA can be configured and provide some necessary information for further CPA procedure, while candidate MN decides its CPA candidates (candidate SNs/SCGs) and provide CPA configurations, including the execution conditions and RACH resources from candidate SNs to candidate MN.

Proposal 9: It’s proposed that RAN2 captures the above figure and procedure in TS 37.340.

References

- RP-222332 Revised WID on Further NR mobility enhancements, Media Tek;

- R2_121bis_meeing_report;

- R2_121_meeting_report_v1;

- 3GPP TS 38.300 NR and NG-RAN Overall Description; Stage 2;

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.