3GPP RAN WG2 Meeting #122	R2-2305645

# Incheon, Korea, May 22-26, 2023

Agenda Item:	7.2.5

Source:	InterDigital Inc.

Title:	Discussion on positioning for RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

Document for:	Discussion

# Introduction

In RAN#98, the following descriptions for WID were agreed [1]. Signalling designs for positioning of RedCap UEs, bandwidth aggregation and carrier phase-based positioning are needed to support the aforementioned functionalities.

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution, discussion topics related to RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning in RAN2 are addressed. The progress on the aforementioned topics made in RAN1#112 [2] and their impacts on RAN2 work are discussed in this contribution.

# Potential enhancements for positioning for RedCap UEs

Frequency hopping has been agreed to be specified in the Rel. 18 WID. Specification support related to physical layer need to be identified first. For example, for TX hopping to transmit PRS or SRS for positioning, details related to signalling of frequency hopping should be discussed in RAN2. Such signalling details may include new assistance data and triggering/activation/deactivation commands. Frequency hopping can be represented by hopping pattern ID, duration of a hop, hopping bandwidth. RAN1 needs to clarify how frequency hopping can be implemented. Based on the RAN1 work, RAN2 can proceed with the study of signalling details.

The following agreements were made in RAN1#112b-e [2]. Tx hopping for SRS for positioning is supported. In addition, for DL RX hopping, measurement gap was agreed to be supported. Based on the agreements made in RAN1 so far, more progress related to details of configuration for Tx and Rx hopping needs to be made in RAN1 so that RAN2 can start the work on signalling design.

Agreement For RedCap UEs, SRS for positioning Tx frequency hopping is configured within one SRS for positioning resource. Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One [or more] measurements where each measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

Proposal 1: RAN2 should wait for RAN1 to finalize potential enhancements for positioning for RedCap UEs

# Potential enhancements for simultaneous transmission or reception

In NRCP agenda item, the RAN1 discussed simultaneous transmission or reception between UEs. The motivation for such transmission or reception is to enable differential processing or calibration. As shown in the agreement, made in RAN1#112b-e, below, the LMF can send a request to the UE to request measurement of DL PRS resources within a time window. RAN2 may need to specify details of an LPP message to be used for the request.

Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

Based on the progress made in RAN1, RAN2 can wait until more progress on the content of the measurement report is made in RAN1. Thus the following proposal is made.

Proposal 2: RAN2 should wait for RAN1 to finalize potential enhancements for carrier phase positioning and simultaneous reception or transmission.

# Bandwidth aggregation for positioning

Aggregation of bandwidth for positioning is beneficial for increasing accuracy for timing based positioning. The following agreement and conclusion were made in RAN1#112 and RAN1#112b-e, respectively. As shown in the conclusion, the content of the on-demand can be discussed in RAN2.

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

One of the request for on-demand bandwidth aggregation can be the number of frequency layers to be aggregated. The UE can send a request to the LMF to configure requested number of frequency layers for PRS. It is still up to the LMF whether to configure the requested number of frequency layers.

Proposal 3: RAN2 studies the content of the on-demand request from the UE to the LMF for PRS frequency layer aggregation.

RAN1 made the following agreement. According to the agreement, the LMF can indicate to the UE whether to aggregated measurements from PFLs. RAN2 can start signalling design for the indication. Such a mechanism will allow the LMF to enable or disable aggregation of PFLs based on the environment.

Agreement For PRS resources aggregated across PFLs for DL-TDOA and multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 DL PRS measurement of single PFL with the necessary update. - FFS: In a measurement report element, single RSRP or single RSRPP is reported - In a measurement report element, PFL aggregation indication is supported to indicate whether/which measurement is aggregated - Support new signaling in location information request message to indicate UE whether to perform joint measurement across aggregated PFLs - Single RSTD reference in assistance data and measurement report is used for PRS bandwidth aggregation measurement - FFS RSTD reference is aggregated or not

Proposal 4: RAN2 studies signalling designs to indicate whether to perform joint measurement across aggregated PFLs

# Conclusion

Proposal 1: RAN2 should wait for RAN1 to finalize potential enhancements for positioning for RedCap UEs

Proposal 2: RAN2 should wait for RAN1 to finalize potential enhancements for carrier phase positioning and simultaneous reception or transmission.

Proposal 3: RAN2 studies the content of the on-demand request from the UE to the LMF for PRS frequency layer aggregation.

Proposal 4: RAN2 studies signalling designs to indicate whether to perform joint measurement across aggregated PFLs.

# Reference

[1] RP-230809, “New WID on Expanded and Improved NR Positioning, ” RAN#99, Rotterdam, Netherlands, March, 2023.

[2] RAN1 Chairman’s note, RAN1#112b-e, April, 2023.

[3] RAN1 Chairman’s note, RAN1#112, Athens, Greece, Feb, 2023.