3GPP TSG-RAN WG2 #122	R2-2305650

Incheon, Korea, 22 – 26 May 2023

Agenda item:		7.4.3	NR-DC with selective activation cell of groups

Title:		Configurations for selective SCG activation

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#121, there had been progresses below [1].

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell. In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case). R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

Further in RAN2#121bis-e, there were some more progress [2]:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format. For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC Assume for now that there is only one reference configuration. The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported. UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

In this contribution, we discuss some remaining issues for selective SCG activation from the past meetings and provide our views.

# 2. Discussion

2.1	Subsequent CPC reusing CPA configuration

In RAN2#121, there was agreed assumption that a CPA conditional configuration can be used for CPC (but with different triggering conditions). This would mean to provide an additional execution condition to be used for the subsequent CPC, but nothing more than that. The consequence should be no delta configuration applied for such subsequent CPC reusing the CPA configuration.

Proposal 1: RAN2 to confirm that when a CPA configuration is used for the subsequent CPC with a different condition, the UE applies the full configuration without using a reference configuration.

2.2	Execution condition for MN-initiated procedure

In RAN2#121bis-e, it was agreed that the MN generates the execution condition for the initial CPAC in the MN-initiated SCG selective activation, while for subsequent CPC, still there are two options on the table:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

As this is the MN-initiated procedure, the Option 1 should be the baseline. The argument supporting the Option 2 is that the candidate SN may want to generate the execution condition which is to be applied when the candidate cell(s) of the candidate SN is the serving PSCell and this way could give some flexibility.

However, it should be noted that this is the MN-initiated procedure, i.e. reusing MN-initiated CPC with removing RRC Reconfiguration procedure for the subsequent CPC (as shown in Fig.1). This would mean that the intention of the feature should be to apply the MN-initiated CPC for the subsequent CPC as well and thus the execution condition should be also along with the MN-initiated CPC. If there is need for the candidate SN to generate the execution condition, that can be considered for the SN-initiated procedure. There is no need to mix two different procedures. Indeed, it just cause additional complexity for the MN-initiated procedure. Therefore, we still propose to go with the Option 1.

Proposal 2: RAN2 to agree that only the MN generates the execution conditions for all subsequent CPCs.

Fig.1: Comparison of MN-initiated procedures in Rel-17 and Rel-18 (expectation)

Another issue is whether the conditional event A3/A5 are supported for MN-initiated procedure. Assuming the Option 1 (i.e. MN generates the execution conditions for all subsequent CPCs), it is straightforward that the A4 is only supported but not A3/A5. This is because the procedure itself is still MN-initiated CPC and there is no A3/A5 support for MN-initiated CPC. There is no point (or strong motivation) to support the A3/A5.

Proposal 3: RAN2 to agree that only the event A4 is supported for MN-initiated procedure.

2.3	Execution condition for SN-initiated procedure

Next, we discuss the SN-initiated procedure. It was agreed that the source SN generates the execution conditions for the initial CPC for SN initiated SCG selective activation. Same as the MN-initiated, it is FFS if the candidate SN may generate or modify execution conditions for subsequent CPCs. As the execution condition for SN-initiated CPC is generated by the source SN, it may make sense to consider this approach. for the SN-initiated procedure unlike the MN-initiated (for which we propose not to support this).

The main argument is that the execution condition using the event A3/A5 should be based on “source PSCell”. This would mean that after the first/previous CPC, the source PSCell is managed by the target SN (i.e. selected candidate SN) and the candidate SN should generate the execution condition in advance, i.e. at initial preparation phase. Otherwise, the RRC reconfiguration is necessary upon CPC and this is not aligned with the intention of the selective SCG activation.

Proposal 4: RAN2 to agree that candidate SNs generate the execution conditions for subsequent CPC from its candidate cell(s) and provide them to the MN.

Proposal 4a: RAN2 to agree that upon CPC, the UE applies the execution condition contained in the RRC Reconfiguration of the target SN (i.e. selected candidate SN).

2.4	MCG configuration

During the post-121 email discussion, it had been pointed out by some companies that it should be discussed whether the MCG configuration is included in the reference configuration or not. This is because in the legacy inter-SN CPC and CPA, the MCG configuration to be applied upon CPC/CPA can be also included. In the RAN2#121bis-e, this was discussed but left open.

We think this is the valid point. If we consider the selective SCG activation as the extension of the legacy CPC/CPA, it is straightforward to assume that the MCG configuration can be provided. On the other hand, the reference configuration is provided to the candidate SNs, while MCG configuration is not required for the candidate SNs. The MCG configuration is not necessarily included in the reference configuration. Therefore, it can be assumed that the MCG configuration is included in the RRC Reconfiguration message containing the Rel-18 CPC configurations.

Proposal 5: RAN2 to agree that the MN can include the MCG configuration in the RRC Reconfiguration message containing the Rel-18 CPC configurations.

# 3. Conclusion

In this contribution we discussed some remaining issues for selective SCG activation from the past meetings and made the following proposals.

Proposal 1: RAN2 to confirm that when a CPA configuration is used for the subsequent CPC with a different condition, the UE applies the full configuration without using a reference configuration.

Proposal 2: RAN2 to agree that only the MN generates the execution conditions for all subsequent CPCs.

Proposal 3: RAN2 to agree that only the event A4 is supported for MN-initiated procedure.

Proposal 4: RAN2 to agree that candidate SNs generate the execution conditions for subsequent CPC from its candidate cell(s) and provide them to the MN.

Proposal 4a: RAN2 to agree that upon CPC, the UE applies the execution condition contained in the RRC Reconfiguration of the target SN (i.e. selected candidate SN).

Proposal 5: RAN2 to agree that the MN can include the MCG configuration in the RRC Reconfiguration message containing the Rel-18 CPC configurations.

# References

[1] RAN2#121, Chair notes

[2] RAN2#121bis-e, Chair notes