3GPP TSG-RAN WG2 Meeting #122	R2-2305651

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on DTX/DRX

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made in the previous RAN2 meeting on Cell DTX/DRX:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

The this contribution, we discuss the remaining issues.

# 2	Discussion

## 2.1	Configuration of cell DTX/DRX

On the cell DTX/DTX configurations and activation/deactivation, the two proposals from the email discussion [312] was left not treated due to time limit:

Proposal 6: An aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within Cell DTX on-duration. FFS extension of Cell DTX active time beyond Cell DTX on-duration. (15/25) Proposal 7: The periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

In our view, it should be possible to configure UE’s DRX cycles shorter than Cell DTX/DRX periodicity esp. if it intends to be used when the Cell DTX/DRX is not activated, but no reconfiguration is needed when Cell DTX/DRX is activated since the Cell DTX/DRX activation/deactivation can be done dynamically, while it does not make sense to require the NW to reconfigure the UE’s DRX configuration whenever Cell DTX/DRX activation/deactivation happens. Thus, the alignment should be up to NW implementation.

Also note that, based on this agreement “10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.  UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.”, thus even if the Cell DTX and UE DRX is not fully aligned, there would be no impact to the UE power consumption as long as the NW ensures there is enough scheduling opportunities taking both NW’s active time and the UE’s active time into consideration, thus it should have no specification impact.

Proposal 1: alignment of UE’s C-DRX and NW cell DTX is up to NW implementation.

## 2.2	SR and retransmissions

Wrt. the FFS on SR, if agreed to allow SR during Cell DTX non-active time, it should be ensured that the UE will monitor PDCCH for SR responses after SR is sent. Otherwise, it would not make sense. It could be further discussed whether there could be some gap before the UE start monitoring to allow NW wake up the Tx if the cell is also in Cell DTX non-active period since typically there would be some alignment for DTX and DRX configuration and potential impact on SR prohibit timer.

For retransmission, we understood the Cell DTX/DRX pattern is mainly for NW energy saving during low load scenario when there is nothing to be transmitted/received, but as soon as when there is transmission for certain UE, potential retransmissions should not be artificially delayed for that UE.

Proposal 2: if to allow SR during the cell non-active period, the UE shall monitor responses for SR as well even if during non-active period.

Proposal 3: retransmission can be scheduled in case it falls out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

# 3	Conclusion

Further details on Cell DTX/DRX are discussed in this contribution with the following proposals proposed:

Proposal 1: alignment of UE’s C-DRX and NW cell DTX is up to NW implementation.

Proposal 2: if to allow SR during the cell non-active period, the UE shall monitor responses for SR as well even if during non-active period.

Proposal 3: retransmission can be scheduled in case it falls out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.