3GPP TSG-RAN WG2 Meeting #122	R2-2305653

Incheon, 22 – 26 May 2023

Agenda item:	7.5.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	BSR enhancements for XR

WID/SID:	NR_XR_enh-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreement was made in RAN2 #121bis-e [1]:

- Bulk agreements 1. As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. 3. Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g., pre-definition vs RRC configuration) is made. 5. At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. 8. New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. Online 6. Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. 4. As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. 2a. Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). 2b. Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

This contribution gives more details on the BSR tables and delay information report for XR.

# 2	Discussion

## 2.1	Design of new BS Table(s)

The main objective of including new BS Table(s) for XR applications is to reduce the quantization error. Then, determining a quantization error metric provides an alternative to compare the packets’ size accuracy given by legacy and new BS table(s) for XR applications. For that purpose, we use the following equation:

QE=reportedpacketsizeB-actualpacketsizeP

In an ideal case the reported packet size would be identical to the actual packet size, meaning QE equals to zero. Then, for some total number of packets N, the total quantization error can be defined in percent as:

QEtotal=NQENB×100 %

If legacy tables are used (Table 6.1.3.1-1 for a 5-bit BSR (32 bins or code points) or Table 6.1.3.1-2 for 8-bit (256 bins or code points) BSR from TS 38.321), we obtain a quantization error of 16.7% and 3.1%, respectively. An example of determining the quantization error for XR video traffic of 10 Mbps mean data rate, using legacy tables are depicted in Figure 1 (5-bit) and Figure 2 (8-bit). The X axis shows the packet sizes for a truncated gaussian traffic model as per TR 38.838. The left-hand Y axis corresponds to the black solid line plotting the probability density function (PDF) of the traffic model. The blue circles indicate the table entries on each table. The shaded regions between two blue points (bins) is the integrated probability of a packet falling is that table range. This means that most of the XR packets fall in just two table bins. The right-hand Y axis corresponds to the cumulative quantization error per bin, which is shown with the red crosses. Each red cross is aligned to the right-hand side of the bin range.

Figure 1 – 5-bit BSR table utilization from TS 38.321

Figure 2 – 8-bit BSR table utilization from TS 38.321

Observation 1: Legacy BSR tables provide a quantization error of 16.7% and 3.1% for 5-bit and 8-bit tables, respectively.

For linear distribution, we will assume simple minimum and maximum values, as defined for the traffic model, at 10 Mbps@60FPS in TR 38.838. In that case, the values are:

XR traffic minimum (TRmin): 10417 bytes,

XR traffic maximum (TRmax): 31250 bytes.

We consider modifying the simple linear definition presented in R2-2301517 to avoid the division operation at the UE while providing the same performance. It is possible by determining two values: BSmin and STEP, which can be derived using implementation specific algorithms at the gNB. The values are defined as follows:

BSmin=TRmin ,

STEP = TRmax-TRminN-1.

Then, the placement of the values into the BS table is done as follows:

BS0=BSmin  and   BSi=BSi-1+STEP   for   i=1,2,…,N

Using such simple definition and assuming the gNB signaled BSmin=10417 and STEP = 82, the resulting table is:

| Index | BS value | Index_1 | BS value_2 | Index_3 | BS value_4 | Index_5 | BS value_6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 10417 | 64 | ≤ 15665 | 128 | ≤ 20913 | 192 | ≤ 26161 |
| 1 | ≤ 10499 | 65 | ≤ 15747 | 129 | ≤ 20995 | 193 | ≤ 26243 |
| 2 | ≤ 10581 | 66 | ≤ 15829 | 130 | ≤ 21077 | 194 | ≤ 26325 |
| 3 | ≤ 10663 | 67 | ≤ 15911 | 131 | ≤ 21159 | 195 | ≤ 26407 |
| 4 | ≤ 10745 | 68 | ≤ 15993 | 132 | ≤ 21241 | 196 | ≤ 26489 |
| 5 | ≤ 10827 | 69 | ≤ 16075 | 133 | ≤ 21323 | 197 | ≤ 26571 |
| 6 | ≤ 10909 | 70 | ≤ 16157 | 134 | ≤ 21405 | 198 | ≤ 26653 |
| 7 | ≤ 10991 | 71 | ≤ 16239 | 135 | ≤ 21487 | 199 | ≤ 26735 |
| 8 | ≤ 11073 | 72 | ≤ 16321 | 136 | ≤ 21569 | 200 | ≤ 26817 |
| 9 | ≤ 11155 | 73 | ≤ 16403 | 137 | ≤ 21651 | 201 | ≤ 26899 |
| 10 | ≤ 11237 | 74 | ≤ 16485 | 138 | ≤ 21733 | 202 | ≤ 26981 |
| 11 | ≤ 11319 | 75 | ≤ 16567 | 139 | ≤ 21815 | 203 | ≤ 27063 |
| 12 | ≤ 11401 | 76 | ≤ 16649 | 140 | ≤ 21897 | 204 | ≤ 27145 |
| 13 | ≤ 11483 | 77 | ≤ 16731 | 141 | ≤ 21979 | 205 | ≤ 27227 |
| 14 | ≤ 11565 | 78 | ≤ 16813 | 142 | ≤ 22061 | 206 | ≤ 27309 |
| 15 | ≤ 11647 | 79 | ≤ 16895 | 143 | ≤ 22143 | 207 | ≤ 27391 |
| 16 | ≤ 11729 | 80 | ≤ 16977 | 144 | ≤ 22225 | 208 | ≤ 27473 |
| 17 | ≤ 11811 | 81 | ≤ 17059 | 145 | ≤ 22307 | 209 | ≤ 27555 |
| 18 | ≤ 11893 | 82 | ≤ 17141 | 146 | ≤ 22389 | 210 | ≤ 27637 |
| 19 | ≤ 11975 | 83 | ≤ 17223 | 147 | ≤ 22471 | 211 | ≤ 27719 |
| 20 | ≤ 12057 | 84 | ≤ 17305 | 148 | ≤ 22553 | 212 | ≤ 27801 |
| 21 | ≤ 12139 | 85 | ≤ 17387 | 149 | ≤ 22635 | 213 | ≤ 27883 |
| 22 | ≤ 12221 | 86 | ≤ 17469 | 150 | ≤ 22717 | 214 | ≤ 27965 |
| 23 | ≤ 12303 | 87 | ≤ 17551 | 151 | ≤ 22799 | 215 | ≤ 28047 |
| 24 | ≤ 12385 | 88 | ≤ 17633 | 152 | ≤ 22881 | 216 | ≤ 28129 |
| 25 | ≤ 12467 | 89 | ≤ 17715 | 153 | ≤ 22963 | 217 | ≤ 28211 |
| 26 | ≤ 12549 | 90 | ≤ 17797 | 154 | ≤ 23045 | 218 | ≤ 28293 |
| 27 | ≤ 12631 | 91 | ≤ 17879 | 155 | ≤ 23127 | 219 | ≤ 28375 |
| 28 | ≤ 12713 | 92 | ≤ 17961 | 156 | ≤ 23209 | 220 | ≤ 28457 |
| 29 | ≤ 12795 | 93 | ≤ 18043 | 157 | ≤ 23291 | 221 | ≤ 28539 |
| 30 | ≤ 12877 | 94 | ≤ 18125 | 158 | ≤ 23373 | 222 | ≤ 28621 |
| 31 | ≤ 12959 | 95 | ≤ 18207 | 159 | ≤ 23455 | 223 | ≤ 28703 |
| 32 | ≤ 13041 | 96 | ≤ 18289 | 160 | ≤ 23537 | 224 | ≤ 28785 |
| 33 | ≤ 13123 | 97 | ≤ 18371 | 161 | ≤ 23619 | 225 | ≤ 28867 |
| 34 | ≤ 13205 | 98 | ≤ 18453 | 162 | ≤ 23701 | 226 | ≤ 28949 |
| 35 | ≤ 13287 | 99 | ≤ 18535 | 163 | ≤ 23783 | 227 | ≤ 29031 |
| 36 | ≤ 13369 | 100 | ≤ 18617 | 164 | ≤ 23865 | 228 | ≤ 29113 |
| 37 | ≤ 13451 | 101 | ≤ 18699 | 165 | ≤ 23947 | 229 | ≤ 29195 |
| 38 | ≤ 13533 | 102 | ≤ 18781 | 166 | ≤ 24029 | 230 | ≤ 29277 |
| 39 | ≤ 13615 | 103 | ≤ 18863 | 167 | ≤ 24111 | 231 | ≤ 29359 |
| 40 | ≤ 13697 | 104 | ≤ 18945 | 168 | ≤ 24193 | 232 | ≤ 29441 |
| 41 | ≤ 13779 | 105 | ≤ 19027 | 169 | ≤ 24275 | 233 | ≤ 29523 |
| 42 | ≤ 13861 | 106 | ≤ 19109 | 170 | ≤ 24357 | 234 | ≤ 29605 |
| 43 | ≤ 13943 | 107 | ≤ 19191 | 171 | ≤ 24439 | 235 | ≤ 29687 |
| 44 | ≤ 14025 | 108 | ≤ 19273 | 172 | ≤ 24521 | 236 | ≤ 29769 |
| 45 | ≤ 14107 | 109 | ≤ 19355 | 173 | ≤ 24603 | 237 | ≤ 29851 |
| 46 | ≤ 14189 | 110 | ≤ 19437 | 174 | ≤ 24685 | 238 | ≤ 29933 |
| 47 | ≤ 14271 | 111 | ≤ 19519 | 175 | ≤ 24767 | 239 | ≤ 30015 |
| 48 | ≤ 14353 | 112 | ≤ 19601 | 176 | ≤ 24849 | 240 | ≤ 30097 |
| 49 | ≤ 14435 | 113 | ≤ 19683 | 177 | ≤ 24931 | 241 | ≤ 30179 |
| 50 | ≤ 14517 | 114 | ≤ 19765 | 178 | ≤ 25013 | 242 | ≤ 30261 |
| 51 | ≤ 14599 | 115 | ≤ 19847 | 179 | ≤ 25095 | 243 | ≤ 30343 |
| 52 | ≤ 14681 | 116 | ≤ 19929 | 180 | ≤ 25177 | 244 | ≤ 30425 |
| 53 | ≤ 14763 | 117 | ≤ 20011 | 181 | ≤ 25259 | 245 | ≤ 30507 |
| 54 | ≤ 14845 | 118 | ≤ 20093 | 182 | ≤ 25341 | 246 | ≤ 30589 |
| 55 | ≤ 14927 | 119 | ≤ 20175 | 183 | ≤ 25423 | 247 | ≤ 30671 |
| 56 | ≤ 15009 | 120 | ≤ 20257 | 184 | ≤ 25505 | 248 | ≤ 30753 |
| 57 | ≤ 15091 | 121 | ≤ 20339 | 185 | ≤ 25587 | 249 | ≤ 30835 |
| 58 | ≤ 15173 | 122 | ≤ 20421 | 186 | ≤ 25669 | 250 | ≤ 30917 |
| 59 | ≤ 15255 | 123 | ≤ 20503 | 187 | ≤ 25751 | 251 | ≤ 30999 |
| 60 | ≤ 15337 | 124 | ≤ 20585 | 188 | ≤ 25833 | 252 | ≤ 31081 |
| 61 | ≤ 15419 | 125 | ≤ 20667 | 189 | ≤ 25915 | 253 | ≤ 31163 |
| 62 | ≤ 15501 | 126 | ≤ 20749 | 190 | ≤ 25997 | 254 | ≤ 31245 |
| 63 | ≤ 15583 | 127 | ≤ 20831 | 191 | ≤ 26079 | 255 | ≤ 31327 |

Truncated Gaussian distribution with 5-bit and 8-bit table can be found in [2]. The achieved quantization errors can be summarized below with legacy table, truncated Gaussian, and linear distribution:

Table 1 - Quantization error for some proposed table types compared to legacy.

|  | 5-bit BS Table | 8-bit table BS Table | Source |
| --- | --- | --- | --- |
| Current 3GPP | 16.7% | 3.1% | TS 38.321 |
| Truncated Gaussian Table following traffic | 1.7% | 0.2% | [2] |
| Linear | 1.7% | 0.2% | [3] and this contribution |

Observation 2: 8-bit BS table provides much lower quantization error than 5-bit BS table.

Observation 3: no real difference with truncated gaussian distribution and linear distribution as long as with enough granularity and the min and max are correctly set.

Since truncated Gaussian distribution is most close to XR traffic distribution and linear distribution provide the same quantization error, there seems no need to consider other more complicated distributions for the table.

Proposal 1: 8 bits with linear distribution is adopted for the new BS tables.

Then, wrt. the options of static table or RRC configured parameter-based table, our preference for table(s) generation is Option 2b (RRC configured parameter-based BSR Tables). The RRC configuration can be either BSmin and BSmax, or BSmin and Step. The former indicates the XR data’s minimum and maximum values to be adopted for the code point generation. The latter indicates the XR data’s minimum value, and the step size to be added at each code point, which depends on the chosen distribution. The latest code point will correspond to the maximum value of XR data. The possible values for the parameters could consider the typical data rate of XR, e.g. 10 Mbps and 20 Mbps, and the frame rate of 30, 45, 60, 72, 90, 120 fps.

Proposal 2: BSmin and Step are configured per LCG as the RRC parameters to generate the new BS tables.

Proposal 3: the possible values for BSmin and Step will be decided based on typical XR data rate.

## 2.2	Co-existence of the new table and the legacy table

Since there could also be other traffic other than the video traffic, e.g., voice, pose, RRC signaling etc., legacy tables are still to be used for those LCH/LCGs. Besides, when the remaining data of the video traffic becomes out of range of the new table, it makes sense to fallback to legacy table to provide enough information for lower range BS values.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE with new BSR table and the legacy BSR table are identified with different LCIDs, thus the NW knows reported LCGs used legacy table or the new table without other explicit indication.

## 2.3	Delay Information

In the RAN2 #120 meeting, the following were agreed on delay information:

RAN2 will introduce data volume information associated with delay information (e.g., remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g., what exactly is reported) and how to ensure this information is up-to-date e.g., considering UL scheduling delay.

This delay information, defined as “remaining time” is the result of the difference between the PDB or PSDB and the time data has been in the buffer (i.e., buffering time). UEs signal it to the network to enable it to learn about the UL traffic timing patterns and accordingly schedule/configure resource allocations for its UEs in coherence with latency constraints. In case a DRB includes PDUs with or without PDU set that have arrived at different times, UEs could use the longest buffering time to calculate the remaining time. The latter also applies in case PDU sets are used to transmit data.

Proposal 6: Define new BSR format to report remaining time information per LCG.

Proposal 7: the shortest remaining time of data buffered is reported as the remaining time for the LCG.

Proposal 8: both independent PDUs or PDUs conforming a PDU set are supported.

The value can be reported by sending an integer or index pointing to a predefined table. The table includes different threshold ranges such that the UE selects the one fitting its remaining time.

Proposal 9: The remaining time can be reported as an integer or an index pointing out the threshold fitting its remaining time from a predefined table.

If the reported remaining time is lower than a predefined threshold, a delay information report is triggered to give the NW enough time to schedule the required resources such that the PDB or PSDB could be fulfilled. Whether it is reported as part of BSR MAC CE or separately can be decided later.

Proposal 10: The remaining time report is triggered when the remaining time of data buffered for a LCG becomes below threshold.

An additional aspect to be considered is the accuracy of the reported remaining time at the gNB, including the scheduling delay as presented in [4]. The up-to-date remaining time at the gNB should include the calculation of the time at the initial transmission such that the UE and the gNB have the same understanding. In case retransmission occur, the gNB is capable to determine the timing of the initial transmission.

Proposal 11: The remaining time is determined based on the timing of the initial transmission of the MAC CE containing the report.

# 3	Conclusion

This contribution has reviewed possible new BSR tables for XR and has proposed the following:

Observation 1: Legacy BSR tables provide a quantization error of 16.7% and 3.1% for 5-bit and 8-bit tables, respectively.

Observation 2: 8-bit BS table provides much lower quantization error than 5-bit BS table.

Observation 3: no real difference with truncated gaussian distribution and linear distribution as long as with enough granularity and the min and max are correctly set.

Proposal 1: 8 bits with linear distribution is adopted for the new BS tables.

Proposal 2: BSmin and Step are configured per LCG as the RRC parameters to generate the new BS tables.

Proposal 3: the possible values for BSmin and Step will be decided based on typical XR data rate.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE with new BSR table and the legacy BSR table are identified with different LCIDs, thus the NW knows reported LCGs used legacy table or the new table without other explicit indication.

Proposal 6: Define new BSR format to report remaining time information per LCG.

Proposal 7: the shortest remaining time of data buffered is reported as the remaining time for the LCG.

Proposal 8: both independent PDUs or PDUs conforming a PDU set are supported.

Proposal 9: The remaining time can be reported as an integer or an index pointing out the threshold fitting its remaining time from a predefined table.

Proposal 10: The remaining time report is triggered when the remaining time of data buffered for a LCG becomes below threshold.

Proposal 11: The remaining time is determined based on the timing of the initial transmission of the MAC CE containing the report.

# Reference

[1] R2-2304202, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

[2] R2-2211600, BS Tables for XR, Nokia, Nokia Shanghai Bell

[3] R2-2301517, Capacity improvements, Nokia, Nokia Shanghai Bell

[4] R2-2303010,  Discussions on delay information reporting, Fujitsu