3GPP TSG-RAN WG2 Meeting #122	R2-2305655

# Incheon, South Korea, 22– 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Num | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR on timing resiliency and URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Rapporteur), Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-core |  |  |  |  |  | Date: |  |  | 2023-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN timing synchronization status update has been agreed for Timing resiliency and URLLC WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Running CR capturing the agreements for Timing resiliency and URLLC: - Descriptions added for RAN timing synchronization status update procedure including the SIB9 update and the UE getting connected to get the detailed timing information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A stage 2 overview of the agreed mechanisms for Timing resiliency and URLLC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 16.8	Support for Time Sensitive Communications

Time Sensitive Communications (TSC), as defined in TS 23.501 [3], is a communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. Examples of such services are the ones in the area of Industrial Internet of Things, e.g. related to cyber-physical control applications as described in TS 22.104 [39].

To support strict synchronization accuracy requirements of TSC applications, the gNB may signal 5G system time reference information to the UE using unicast or broadcast RRC signalling with a granularity of 10 ns. Uncertainty parameter may be included in reference time information to indicate its accuracy. The UE may indicate to the gNB a preference to be provisioned with reference time information using UE Assistance Information procedure. Propagation delay compensation (PDC) mechanisms may be applied based on RTT or TA, and can be performed at the UE or gNB side. When performed at UE side, the PDC mechanisms are controlled via RRC signalling by the gNB.

The RTT-based PDC mechanism is achieved by using Rx-Tx time difference measurements of a single pair of configured TRS/PRS and SRS. The following figure describes the signalling procedures of UE-side RTT-based PDC:

Figure 16.8-1: Signalling Procedure of UE-side RTT-based PDC

1.	The gNB provides measurement configurations to the UE;

2a/b.	The gNB transmits TRS or PRS to the UE for measurements, and the UE transmits SRS to the gNB for measurement;

3a/b.	Both the UE and the gNB perform Rx-Tx time difference measurements;

4.	The gNB provides its Rx-Tx time difference measurement to the UE;

5.	The UE performs PDC based on Rx-Tx time difference measurements from itself and the gNB.

The following figure describes the signalling procedures of gNB-side RTT-based PDC:

Figure 16.8-2: Signalling Procedure of gNB-side RTT-based PDC

1.	The gNB provides measurement configurations to the UE;

2a/b.	The gNB transmits TRS or PRS to the UE for measurements, and the UE transmits SRS to the gNB for measurement;

3a/b.	Both the UE and the gNB perform Rx-Tx time difference measurements;

4.	The UE reports its Rx-Tx time difference measurement to the gNB;

5.	The gNB performs PDC based on Rx-Tx time difference measurements from itself and the UE.

The gNB may also receive TSC Assistance Information (TSCAI), see TS 23.501 [3], from the Core Network, e.g. during QoS flow establishment, or from another gNB during handover. TSCAI contains additional information about the traffic flow such as burst arrival time, burst periodicity, and survival time. TSCAI knowledge may be leveraged in the gNB's scheduler to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants, and/or to improve the associated link reliability to meet the survival time requirement (see TS 22.104 [39]).

To support uplink periodic traffics of services with survival time requirement, configured grant resources can be used such that the mapping relation between the service and the configured grant is known to both gNB and UE, thus allowing the gNB to use configured grant retransmission scheduling (addressed by CS-RNTI) to trigger survival time state entry for the corresponding DRB. Upon survival time state entry, all RLC entities configured for the DRB are activated by the UE for duplication to prevent failure of subsequent messages and hence fulfilling the survival time requirement.  If CA or DC duplication for the DRB is already activated, the DRB should enter survival time state when any retransmission grant for any of its active LCHs is received.

For NG-RAN timing synchronization monitoring support, the gNB may receive RAN timing synchronization status information request from the AMF, see TS 23.501 [3]. The RAN timing synchronization status information includes the gNB node-level information about timing synchronization operation status. The gNB timing synchronization status may comprise one or more of the following information elements: synchronization state, traceability to UTC, traceability to GNSS, clock frequency stability, PTP clockClass, clock accuracy, parent time source, as defined in Table 5.27.1.12-1 in TS 23.501 [3].

Additionally, the gNB may receive clock quality reporting control information for a UE from the AMF, see TS 23.501 [3]. The clock quality reporting control information contains the clock quality detail level (i.e., “metrics” or “acceptable/not acceptable indication”) and clock quality acceptance criteria for the UE (if the clock quality detail level equals "acceptable/not acceptable indication”). Based on the clock quality reporting control information, the gNB determines how to provision clock quality information to the UE:

-	If the clock quality detail level equals "clock quality metrics", the gNB provides clock quality metrics supported by the gNB to the UE, i.e., one or more of the following information elements: synchronization state, traceability to UTC, traceability to GNSS, clock frequency stability, clock accuracy, parent time source, as defined in Table 5.27.1.12-1 in TS 23.501 [3].

-	If the clock quality detail level equals "acceptable/not acceptable indication", the gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1 in TS 23.501 [3] with the exception of PTP clockClass.

To provision clock quality information to the UEs, a gNB uses unicast RRC signaling:

-	For UEs in the RRC CONNECTED state, the gNB uses unicast RRC signaling.

-	UEs that are not in the RRC_CONNECTED state first needs to establish or resume the RRC connection to receive the gNB timing synchronization status information from the gNB via unicast RRC signaling.

The following figure describes the signalling procedure of gNB reporting clock quality information to a UE:

Figure 16.8-3: Signalling procedure of gNB reporting clock quality information to a UE

0.	The gNB node is pre-configured for the thresholds for each timing synchronization status attribute as described in clause 5.27.1.12 in TS 23.501 [2]. If there is a change on its primary source so that the thresholds are exceeded or met again, the NG-RAN node detects a change on its timing synchronization status (e.g., degradation, failure, recovery).

1.	The gNB notifies a change on its timing synchronization operation using Event ID in SIB9.

2.	The UE in RRC_INACTIVE or RRC_IDLE determines if there is clock quality information update available at the gNB based on SIB9 information. For a UE in RRC_CONNECTED status, steps 2-3 can be skipped.

3.	If there is a RAN timing synchronization status update available, the UE initiates RRC Setup or RRC Resume procedure.

4.	The gNB determines clock quality information reporting to the UE (e.g., metrics or “acceptable/not acceptable”).

5.	The gNB sends the clock quality information to the UE via unicast RRC signaling.

End of Changes