3GPP TSG-RAN WG2 Meeting #122	R2-2305656

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.23.2

Source:	Nokia, Nokia Shanghai Bell

Title:	5GS network Timing synchronization status and reporting

WID/SID:	TRS_URLLC-NR-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made in the previous RAN2 meeting wrt. the objective on 5GS network timing synchronization status and reporting:

Agreements - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message - Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain - 5G Clock Quality information is carried in DLInformationTransfer message. - FFS if update of event ID is informed to UE by SI modification procedure - For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS. - The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive

In this contribution, we further address the highlighted open issues.

# 2	Discussion

## 2.1	SI update for event ID change

On the issue of whether event ID update is done with normal SI modification procedure, or it should not as part of SI modification to avoid impact to other UEs not requiring the information. For the latter, possible options are 1) to require the UE requiring the information keeps on reading SIB9 for possible update, which is too power consuming, or 2) to have some indication e.g. in the paging short message similar to ETWS indication. Both are possible but the latter option might be a bit overkill: It would consume one bit (see below) from the remaining 4 bits.

## 6.5 Short Message Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1). Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit. Table 6.5-1: Short Messages Bit; Short Message. 1; systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8.. 2; etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification.. 3; stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1.. 4; systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period.. 5 – 8; Not used in this release of the specification, and shall be ignored by UE if received..

With the assumption that the clock quality change only happens very rarely or in environments where the SI update is not problematic (both of which, in our view, are the baseline assumption), using the normal SI modification procedure could be sufficient.

Proposal 1: update of event ID is informed to UE by normal SI modification procedure.

## 2.2	IDLE/Inactive mode

For the UE to change to RRC_CONNECTED from IDLE/INACTIVE, CT1 24.501 specification has the following:

##### 5.2.3.2.3 ATTEMPTING-REGISTRATION-UPDATE #### The UE in 3GPP access: #### […] m) shall initiate a registration procedure for mobility and periodic registration update if the UE supports the reconnection to the network due to RAN timing synchronization status change and receives an indication of a change in the RAN timing synchronization status. #### 5.3.1.4 5GMM-CONNECTED mode with RRC inactive indication […] If the UE supporting the reconnection to the network due to RAN timing synchronization status change has been requested to reconnect to the network upon receiving an indication of a change in the RAN timing synchronization status (see subclauses 5.4.4.2, 5.5.1.2.4, and 5.5.1.3.4) and the UE receives an indication of a change in the RAN timing synchronization status, the UE in 5GMM-CONNECTED mode with RRC inactive indication shall request the lower layers to transition to RRC_CONNECTED state.

From the text, the AS layer sends an indication of a change in the RAN timing synchronization status to the NAS layer (i.e., when the value of the reference report ID (gNB ID in SIB1 and the Event ID in SIB9 as supported in Rel-18) is different than the stored value at the UE. When this indication is received at NAS layer, the NAS layer checks the condition of the UE to reconnect to the network are satisfied (i.e., the UE reconnection indication has been provided to the UE). If satisfied, the NAS layer maps the access attempt to an access category and request the RRC layer to move to RRC_CONNECTED.

Proposal 2: Confirm the AS layer of the UE determines if there a change of event ID and gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer requests the RRC layer to move to RRC_CONNECTED.

On the access category for the access for UAC, during CT1#141E the following CT1 reply LS was agreed [C1-232942] assuming existing AC 3 (MO_sig) will be used while they also asked SA1 if there is a service requirement to use a new access category. No action needed from RAN2 point of view for now.

## 2.3	Connected mode

Based on description from the latest TS 23.501 v18.1.0, the gNB knows which UEs need timing synchronization status update when the clock quality change from the UE context:

#### 5.27.1.12 Support for network timing synchronization status monitoring […] For 5G access stratum time synchronization service, clock quality reporting control information manages the NG-RAN timing synchronization status notifications to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to NG-RAN, AMF also includes the clock quality reporting control information provided by the TSCTSF or received from UDM. […] When determining the clock quality metrics for a UE and when determining whether clock quality is acceptable or not acceptable for a UE, NG-RAN considers whether propagation delay compensation is performed while UE capabilities and internal inaccuracies are assumed to be budgeted by the client network operator when agreeing the required clock accuracy with the 5G network operator. To provision clock quality information to the UEs, the NG-RAN uses unicast RRC signalling: - For UEs in RRC Connected state, the NG-RAN uses unicast RRC signalling. - The UE that is not in RRC_CONNECTED state may establish or resume the RRC connection to receive the clock quality information from the NG-RAN.

In RAN3, during RAN3#119BE the following impacts were agreed in R3-231998 for NGAP specification (TS 38.413), considering the SA2 agreements to support gNB timing synchronization status monitoring:

#### 9.3.1.220 Time Synchronisation Assistance Information This IE indicates 5G access stratum time distribution parameters as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Time Distribution Indication; M; ENUMERATED (enabled, disabled, …); -. Uu Time Synchronisation Error Budget; C-ifEnabled; INTEGER (1..1000000, …); Expressed in units of 1ns.; -. Clock Quality Reporting Control Information; O; 9.3.1.x1; Yes; ignore. #### 9.3.1.x1 Clock Quality Reporting Control Information This IE indicates the clock quality reporting control information as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Clock Quality Detail Level; M. >clock quality metrics. >>[FFS]. >acceptance indication. >>Clock Quality Acceptance Criteria; M; 9.3.1.x2. #### 9.3.1.x2 Clock Quality Acceptance Criteria This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. [FFS].

The gNB is configured with the clock quality detail level for each UE which tells to the gNB whether the gNB should send clock quality metrics or just acceptable/not acceptable indication. For acceptable/not acceptable level, the gNB must be configured with some threshold for each clock quality metric that tells to the UE whether the clock quality is acceptable or not (i.e., Clock Quality Acceptance Criteria to be further defined in RAN3). This information is stored in the gNB as part of the UE context. The AMF provides this IE within “Time Synchronisation Assistance Information” IE (e.g., included in INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, or PATH SWITCH REQUEST ACKNOWLEDGE messages).

Observation 1: The gNB receives clock quality detail level and clock quality acceptance criteria (if applicable) from the AMF to determine the reporting towards the UE(s).

For a UE in RRC_CONNECTED, current assumption in SA2 is that the RAN timing synchronization status report (either clock quality metrics or acceptable/unacceptable clock quality indication) can always be sent to all UEs in RRC_CONNECTED that have clock quality information configured in their context, not just when the threshold changes from gNB perspective.

Observation 2: The gNB can always send RAN timing synchronization status report to the RRC_CONNECTED UEs that have clock quality information configured in their context.

Proposal 3: For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation.

# 3	Conclusion

Remaining issues on Timing synchronization status and reporting are discussed in this contribution with the following proposals made:

Proposal 1: update of event ID is informed to UE by normal SI modification procedure.

Proposal 2: Confirm the AS layer of the UE determines if there a change of event ID and gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer requests the RRC layer to move to RRC_CONNECTED.

Proposal 3: For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation.