3GPP TSG-RAN WG2 Meeting #122	R2-2305657

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.23.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Reactive RAN feedback determination for UL data streams

WID/SID:	TRS_URLLC-NR-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

On the objective of adapting downstream and upstream scheduling based on RAN feedback:

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

The following agreement was made in the previous RAN2 meeting:

Agreements

=>	We will wait for XR to progress and see if we can use the existing mechanism. RAN2 will strive to re-use existing mechanism or rely on gNB to determine the information (i.e. aim to not introduce new UE specific BAT reporting)

We further discuss the issue in this contribution and try to conclude the issue according to the workplan.

# 2	Discussion

During the discussion in the previous meeting, there have been different views on how the adjustment for BAT works for upstream. Some companies assumed it can be left to UE implementation to adjust in the application layer without CN involvement while some assumed it is controlled by CN based on RAN feedback. Since the objective in the WID is clear that it covers both downstream and upstream, we do not think we need to reopen the discussion in RAN2. The only question to RAN2 is whether we introduce UE report. While RAN3 can discuss the RAN feedback to CN part based on available information, with or without the UE report.

Proposal 1: confirm it is feasible for gNB implementation to derive BAT offset based on available information.

As discussed in the previous meeting, the delay information introduced for XR could provide good enough information about data arrival time. Besides, UE assistance information data arrival reporting for UL is also under discussion in XR WI.

3. UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

According to the workplan, RAN2 is supposed to conclude the discussion for objective 3 on BAT offset deviation. We propose to confirm no UE report will be introduced for this WI and the objective can be closed from RAN2 point of view.

RAN2#122 - Continue discussing objectives 1b and 1c. - Conclude the discussion for objective 3 on BAT offset derivation. - Update running CRs with new agreements.

Proposal 2: confirm no UE report is needed for objective 3 on BAT offset derivation and we can close the objective in RAN2.

# 3	Conclusion

The objective on BAT offset derivation is discussed in this contribution with the following proposals made:

Proposal 1: confirm it is feasible for gNB implementation to derive BAT offset based on available information.

Proposal 2: confirm no UE report is needed for objective 3 on BAT offset derivation and we can close the objective in RAN2.