3GPP TSG-RAN WG2 Meeting #122	R2-2305679

Incheon, 22nd – 26th May 2023

Agenda item:	   7.4.3

Source:		Lenovo

Title:		Discussion on issues related to SCG selective activation

Document for:		Discussion and Decision

# Introduction

In this paper, we further discuss the relevant issues related SCG selective activation.

# 2	Discussion

## 2.1 General

In the last RAN2 meeting, RAN2 decided to focus on SCG for selective activation cell of groups. This would translate to MCG being same for a while (providing coverage), UE is configured with more than one SCG configurations to the UE, as shown in the below figure:

Figure 1

Following sub-topics need to be investigated by RAN2:

- Provisioning of candidate SCG Configurations to the UE: There’s no reason for this configurations to be provided to the UE using non-RRC means; legacy CPAC configuration can be taken as baseline.

- Provision of measurement configuration: legacy CPAC execution condition configuration can be taken as baseline.

- Security: if different security keys are used for different candidate SCGs during SCG selective activation.

- DL and UL synchronization:  legacy CPAC procedure can be taken as a baseline, while enhancements introduced by L1/L2 mobility (e.g., proactive UL sync) maybe applicable as well.

- Mobility confirmation: In the legacy Rel16 intra-SN CPC and Rel17 CPAC, after successful PSCell switch execution, UE sends a SN RRC Reconfiguration Complete message to NW in different ways. It needs to be discussed if the same approach is reused for intra-SN and inter-SN SCG selective activation respectively.

Among above mentioned sub-topics, a) b) c) are under RAN2 discussion and we further discuss the related issues in the below Section 2.3 to 2.6 respectively. d) and e) have not been discussed yet, and we further explain our thinking in Section 2.7.

With respect to the naming of the SCG selective activation, since the core idea behind is to allow consecutive CPC execution without RRC reconfiguration, we can name it as consecutive CPC. Even in case of “CPA” selective activation, in our understanding that means the consecutive CPC after CPA.

RAN2 is suggested to name the SCG selective activation procedure as consecutive CPC.

## 2.2 Scenario

RAN2#119bis Agreements: Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

In the RAN2#119bis meeting, RAN2 has confirmed the need to support “CPA” selective activation of cell groups. On the other hand, the exact meaning of “CPA” selective activation needs further clarification. In our understanding there are two types of “CPA” selective activation.

- Type#1: After CPA execution, configurations of candidate PSCells for CPA are kept for the subsequent conditional PSCell change (without releasing the configuration after execution), and UE stays in DC scenario.

- Type#2: After CPA execution, configurations of candidate PSCells for CPA are kept for the subsequent conditional PSCell addition (without releasing the configuration after execution) when UE switches from DC to standalone scenario.

In the RAN2#121 meeting, Type#1 has been discussed and assumed by RAN2.

RAN2#121 Agreements: R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

Type#2 implies UE will switch frequently between DC scenario and stand-alone scenario, which doesn’t seem a practical scenario for SCG selective activation. In our understanding, the motivation for SCG selective activation is to reduce reconfiguration effort when UE stays in DC and moves among few PSCells and needs to switch PSCells frequently. In addition, switch between DC and standalone scenario also means the switch of bearer or radio resources between MN and SN, which has impact on MCG configurations as well.

When UE disconnects from SN and enters stand-alone connection, configuration related to SCG selective activation is released.

It has been assumed that all three scenarios can be supported, namely SN initiated intra-SN SCG selective activation, MN initiated inter-SN SCG selective activation, and SN initiated inter-SN SCG selective activation.

RAN2#121 Agreements: Assume to support the following scenarios of SCG selective activation: - SN initiated intra-SN SCG selective activation - MN initiated inter-SN SCG selective activation - SN initiated inter-SN SCG selective activation

In case of SN initiated intra-SN SCG selective activation, in our view, some principles for legacy Rel16 intra SN CPC can be taken as the start point. For example, Rel16 intra SN CPC is a procedure without MN involvement, meaning the configuration of intra SN CPC is transparent to MN and can be configured by SRB3 directly.

Similar as Rel16 intra-SN CPC, Rel18 intra-SN SCG selective activation is a procedure without MN involvement, which can be configured by SRB3.

In Release 17, coexistence of different conditional reconfigurations is supported, that UE can be configured with CHO, intra SN CPC and inter SN CPC at the same time. However, we don’t see the motivation to support SCG selective activation and legacy CPC at the same time, since they are more or less serving the same purpose. Thus, it is suggested that RAN2 does not support the coexistence of SCG selective activation and legacy CPC.

RAN2 does not support the coexistence of SCG selective activation and legacy CPC.

## 2.3 Reference Configuration

RAN2#121bis Agreements: For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

In RAN2#121bis meeting, RAN2 agreed that the reference configuration design for SCG selective activation shall follow similar design as LTM. At the same time, it’s agreed that the reference configuration is always explicitly signalled to UE.

Then, following the same principle and to make progress, RAN2 is suggested to agree that for SCG selective activation configuration, the reference configuration is also always explicitly signalled and not automatically derived from any other configuration.

Following LTM conclusion, the reference configuration for SCG selective activation is also always explicitly signalled and is not automatically derived from any other configuration.

In terms of which network node provides the reference configuration, in our understanding, it could be decided by the initiating MN/SN node. In case of MN initiated inter-SN SCG selective activation, it should be MN that generates the reference configuration. And in case of SN initiated intra/inter-SN SCG selective activation, it should be the source SN that generates the reference configuration.

The reference configuration for SCG selective activation is generated by the initiating MN/SN node.

For inter SN SCG selective activation, when the MN provides the reference configuration to the target SN for delta configuration, it can reuse the existing fields in the CG-ConfigInfo inter node message, namely scg-RB-Config and sourceConfigSCG. The target SN does not need to understand if the received reference configuration is the initial source SCG configuration or some separate configuration generated by the initiating MN/source SN.

The existing fields in CG-ConfigInfo message, such as scg-RB-Config and sourceConfigSCG, are used to transfer the reference configuration from MN to target SN for delta configuration for SCG selective activation.

However, in case of SN initiated inter SN selective activation, in legacy CG-Config message, there seems to be no appropriate fields to convey the reference configuration. Thus, RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

## 2.4 About MCG configuration

RAN2#121bis Agreements: The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

There are two FFSs left regarding the MCG configuration during SCG selective activation considering the MCG configuration could change as well during the SCG selective activation, namely:

- if reference configuration could also include a reference configuration for MCG.

- If each target SCG configuration could be associated with a corresponding MCG configuration.

First, it could be beneficial for RAN2 to confirm if MCG configuration could change during the SCG selective activation. Note that in Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs. Thus, it seems straightforward to follow the same principle as in Rel17 inter SN CPC.

In Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs.

RAN2 confirms that in SCG selective activation, MCG configurations (with the same PCell) could be different for different candidate SCGs.

During SCG selective activation, each target SCG configuration could be associated with a corresponding MCG configuration.

To better support the MCG configuration change during SCG selective activation, it seems beneficial to support reference configuration to support delta MCG configuration as well.

RAN2 supports MCG reference configuration for delta MCG configuration during SCG selective activation

## 2.5 Execution condition configuration

RAN2#121bis Agreements: For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported.

For MN/SN initiated inter-SN SCG selective activation, it seems particularly controversial in terms of which RAN node should generate the execution condition for the subsequent CPC during inter-SN SCG selective activation. There are two options with pros and cons:

- Option 1: The initiating MN/SN node should generate all execution conditions for the subsequent SCG selective activation procedure.

  - Pros:

    - similar principle as in Rel17 inter SN CPC

  - Cons:

    - the initiating MN or source SN may not be able to configure perfect execution conditions for all possible PSCell switches considering all possible combination of serving/source PSCell and candidate PSCells

    - The SN measurement configuration could change when UE switches between different candidate SNs

- Option 2: The candidate SN generates the execution conditions for the cases wherein the candidate SN becomes the new serving/source SN, and UE conditionally switches to other SNs.

    - Pros:

      - Comparatively, the SN of a candidate PSCell has better judge on the execution conditions when the candidate PSCell becomes a serving/source PSCell

    - Cons:

      - One candidate SN does not know the existence of other candidate SNs and PSCells.

In our understanding, no matter if it is MN initiated or SN initiated inter SN SCG selective activation, the same option should be supported to have unified solution. Probably enhancements are needed to address the corresponding cons.

For both MN initiated and SN initiated inter SN SCG selective activation, the same solution should be supported, either

  - The initiating MN/SN generates all execution conditions for all possible conditional switches, or

  - Each source/candidate SN generates execution conditions for the case when UE is connected to a PSCell of that SN and will conditionally switch to other PSCells.

If 12-a is agreed, RAN2 further discusses how to keep the measurement configuration unchanged when UE switches between SNs, such that the execution condition configured by the initiating node will stay valid.

if 12-b is agreed, RAN2 further discusses how to let a candidate SN know the existence of other candidate SN PSCells, such that candidate SN can configure the execution condition for UE to switch from a local PSCell to other candidate SN PSCells.

It has been agreed to support A3 and A5 events for in SN initiated Inter-SN SCG selective activation. In our understanding the combination of A3 and A5 events should also be supported same as in legacy CHO/CPAC configuration. RAN2 is suggested to clarify.

In SN initiated Inter-SN SCG selective activation, RAN2 confirms the combination of A3 and A5 is supported as execution condition.

## 2.6	Security

RAN2 agreed to support selective activation without updating the security via RRC reconfiguration.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In our understanding, for intra-SN SCG selective activation, the secondary key can remain the same, while it is also possible that the MN/SN decides to use a new secondary key when switching between SCGs if we stick to the legacy principle.

RAN2 confirms that for intra-SN SCG selective activation the KgNB/sk-counter can remain the same during SCG selective activation unless reconfigured by network via RRC reconfiguration.

For inter-SN SCG selective activation, as explained in Section 2.2, a security hole does exist when a UE needs to come back to a previously visited PSCell/ SCG before the Master Key KgNB has been changed since this would lead to the same sk-counter being used again with the same KgNB, which leads to potential security breach since all other security input parameters (like SN, HFN, Direction etc.) will be reused as well e.g., if in the Figure 1, the UE were to come back to SCG-2 from SCG-3 and they belong to different SNs.

Thus, some enhancement is needed if RAN2 wants to support inter-SN SCG selective activation without security update via another RRC reconfiguration message. One possibility could be that a list of sk-counters are configured for the same candidate PSCell/SCG, then every time UE switches back to this PSCell/SCG, a new sk-counter will be used.

If inter-SN SCG selective activation is supported, RAN2 supports configuring a list of sk-counters for one candidate SCG to avoid security update via RRC reconfiguration during SCG selective activation.

## 2.7	Others

Another aspect is after SCG selective activation execution and UE accesses to a target SCG successfully, how does UE send the SN RRC Reconfiguration Complete message to the network. Note that in legacy Rel16 intra-SN CPC and Rel17 CPAC, different approaches are used:

- For Rel16 intra-SN CPC, after CPC execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured. MN forwards to the SN.

- For Rel17 CPAC, after CPAC execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN. MN forwards to the corresponding target SN.

For Rel18 SCG selective activation, considering both intra-SN SCG selective activation and inter-SN SCG selective activation are going to be supported, the SN RRC Reconfiguration Complete message transmission after PSCell switch can follow similar approaches in the legacy. That is

- For intra-SN SCG selective activation, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

- For inter-SN SCG selective activation (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

For intra-SN SCG selective activation, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

For inter-SN SCG selective activation (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

In the LTM discussion, RAN2 agreed to support early TA acquisition. More specifically, UE could get the TA value for a certain candidate cell in advance and can thus skip the RACH procedure when switching to that candidate cell. As such the cell switching delay can be reduced.

Even though the exact solution for early TA acquisition is still under discussion, RAN2 can discuss, if time allowed, the early DL/UL sync approach introduced for LTM can be extend to the SCG selective activation scenario.

RAN2 can discuss early DL/UL sync for LTM can be extended to SCG selective activation if time allows.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	In Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs.

Based on the discussion above, we propose:

General

Proposal 1	RAN2 is suggested to name the SCG selective activation procedure as consecutive CPC.

Scenario

Proposal 2	When UE disconnects from SN and enters stand-alone connection, configuration related to SCG selective activation is released.

Proposal 3	Similar as Rel16 intra-SN CPC, Rel18 intra-SN SCG selective activation is a procedure without MN involvement, which can be configured by SRB3.

Proposal 4	RAN2 does not support the coexistence of SCG selective activation and legacy CPC.

Reference Configuration

Proposal 5	Following LTM conclusion, the reference configuration for SCG selective activation is also always explicitly signalled and is not automatically derived from any other configuration.

Proposal 6	The reference configuration for SCG selective activation is generated by the initiating MN/SN node.

Proposal 7	The existing fields in CG-ConfigInfo message, such as scg-RB-Config and sourceConfigSCG, are used to transfer the reference configuration from MN to target SN for delta configuration for SCG selective activation.

Proposal 8	RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

MCG Configuration

Proposal 9	RAN2 confirms that in SCG selective activation, MCG configurations (with the same PCell) could be different for different candidate SCGs.

Proposal 10	During SCG selective activation, each target SCG configuration could be associated with a corresponding MCG configuration.

Proposal 11	RAN2 supports MCG reference configuration for delta MCG configuration during SCG selective activation

Execution Condition

Proposal 12	For both MN initiated and SN initiated inter SN SCG selective activation, the same solution should be supported, either

a.	The initiating MN/SN generates all execution conditions for all possible conditional switches, or

b.	Each source/candidate SN generates execution conditions for the case when UE is connected to a PSCell of that SN and will conditionally switch to other PSCells.

Proposal 13	If 12-a is agreed, RAN2 further discusses how to keep the measurement configuration unchanged when UE switches between SNs, such that the execution condition configured by the initiating node will stay valid.

Proposal 14	if 12-b is agreed, RAN2 further discusses how to let a candidate SN know the existence of other candidate SN PSCells, such that candidate SN can configure the execution condition for UE to switch from a local PSCell to other candidate SN PSCells.

Proposal 15	In SN initiated Inter-SN SCG selective activation, RAN2 confirms the combination of A3 and A5 is supported as execution condition.

Security

Proposal 16	RAN2 confirms that for intra-SN SCG selective activation the KgNB/sk-counter can remain the same during SCG selective activation unless reconfigured by network via RRC reconfiguration.

Proposal 17	If inter-SN SCG selective activation is supported, RAN2 supports configuring a list of sk-counters for one candidate SCG to avoid security update via RRC reconfiguration during SCG selective activation.

Others

Proposal 18	For intra-SN SCG selective activation, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

Proposal 19	For inter-SN SCG selective activation (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

Proposal 20	RAN2 can discuss early DL/UL sync for LTM can be extended to SCG selective activation if time allows.

# 4	Reference