3GPP TSG-RAN WG2 Meeting #122 meeting	R2-2305684

Incheon, Korea, 21st – 26th May, 2023

Agenda item:		7.5.2

Source:		Lenovo

Title:		Discussion on PDU sets and data burst awareness in RAN

Document for:		Discussion and Decision

- Introduction

- RAN#99 has updated the XR WID in RP-230786 including XR awareness:

Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

- In this contribution we further discuss the XR-awareness parameters to use in RAN.

- Discussion

2.1	UL Jitter information of PDU set

Last RAN2 meeting has discussed UL jitter and reached the following:

UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

Last SA2 meeting has approved S2-2306201, the DL jitter related text is blue highlighted as below.

5.37.8 UE power saving management 5.37.8.1 General The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX: - UL and/or DL Periodicity; - N6 Jitter Information associated with the DL Periodicity; - Indication of End of Data Burst. 5.37.8.2 Periodicity and N6 Jitter Information associated with Periodicity At reception of measured Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range together with the associated DL periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2. NOTE 3: In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the N6 jitter is larger than a threshold.

According to the CR, we observe that the DL jitter range info per QoS flow is measured by UPF and reported to SMF and finally provided in the TSCAI to NG-RAN by SMF. Accordingly, we consider the UL jitter range information with the associated UL periodicity is measured based on statistics per QoS flow by UE. The jitter range information may be expressed by example of +/- N ms. For example, if the data arrives 2ms earlier compared to an expected timing according to periodicity of arrival timing, the jitter value is -2ms, while if the data arrives 3ms later compared to an expected timing, the jitter value is 3 ms. UE transmits the UL jitter range information e.g., maximum or average value associated with each UL periodicity, e.g., for a LCH or a QoS flow, to NG-RAN via UAI, e.g. in order to assist CG configuration.

Proposal 1: UE transmits the UL jitter range information associated with UL periodicity per logical channel or per QoS flow, to NG-RAN via UAI.

2.2	UL PDU set importance

RAN2 has agreed to support PDU discarding handling with different PSI in presence of UL congestion. And we think that, UE may discard the PDU set with an associated lower importance value in case of UL congestion. To support this, first of all, the PSI of UL PDU set should be aware at UE AS layer, and this can be up to UE implementation to make UE AS layer aware of PSI of UL PDU set. Further details of the PSI-based discarding procedure can be found in our contribution [x]:

- NW configures different PDCP discard timers for different PDU sets with different PSI, and UE applies one shorter timer for the PDU set with low PSI if congestion is detected by UE or indicated by NW. or

- NW configures PSI-based discard mode, and UE activates PDU discard for PDU set with low PSI if congestion is detected by UE/NW.

And if depending on the congestion detection by UE, the congestion detection rule for example of buffered data volume and buffer delay should be configured by NW. It is considered to be challenging for NW to determine a suitable value due to the variable PDU set size and the dynamically scheduling handling by NW. Furthermore, this will bring additional RAN2 impact. Therefore, we prefer the congestion detection is performed by NW.

Proposal 2: To support the UL PDU set discarding of PDU set with low PDU set importance, confirm the UL PSI awareness at UE is up to UE implementation.

2.3	PDU set Integrated Handling Indication

According to SA2 definition, PSIHI indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer. From RAN point view, it’d better successfully transmit the whole PDU set in the PDU set delay budget as possible in normal case. On the other hand, in the failure transmission case or if the discard timer of one PDU in the PDU set expires, the discard timer of the remaining PDUs/SDUs in the PDU set will be considered as expired.. Therefore, NW should configure the PSIHI per DRB to UE for supporting of UL PDU set discarding based on discard timer.

Proposal 3: NW configures the PSIHI per DRB to UE, e.g. for supporting of UL PDU set based discarding.

2.4	PDU set Delay Budget and EDBI (Enhanced Delay Budget Information)

According to SA2 definition, UL PDU Set Delay Budget (PSDB) indicates the time between reception of the first PDU at the UE in UL and the successful delivery of the last arrived PDU of a PDU Set at the UPF in UL.

After receiving PSDB from CN, the NW should further derive the Uu PSDB based on CN PSDB. Generally, NW will schedule PDU set transmissions within the PSDB. For supporting PDCP discard operation, NW also configures the discard timer based on the Uu PSDB for UE.

To efficiently support delay aware-based scheduling, UE will report the up-to-date delay information of buffered data e.g., remaining time of a PDU set to NW e.g. within the buffer status report. Therefore, UE needs to know the PSDB and buffer time of a PDU set. The discard timer length is configured based on the Uu PSDB, in same case, it may be larger than Uu PSDB. As result, the remaining time of a PDU set is just the remaining time of a discard timer.

Proposal 4: RAN2 to discuss to support the remaining delay budget report for a PDU set, UE uses the discard timer length as the allowed Uu PSDB.

2.5	UL End of Data Burst

Last RAN2 meeting has discussed UL end of data burst indication and reached the following:

FFS on whether EoDB signalling is needed.

Like the purpose of the downlink EoDB, if network can be aware of the end of a UL data burst, network can determine to terminate UE’s DRX active time early for the purpose of power saving.

In our understanding, the padding BSR e.g., indicating data volume=0 can be used as an implicit indication of EoDB. Some new trigger condition for the transmission of a “zero-BSR” may be required. Alternatively, UE could indicate within a new indication, e.g. new field within the MAC header or new MAC CE to indicate the end of a data burst to the NW. In any case we see some benefit that gNB is made aware of the end of a data burst in order to optimize UE power saving.

Proposal 5: RAN2 to discuss how to indicate EODB to the gNB for UL, e.g. by means of triggering “zero-BSR” or by some new indication in MAC header or by a new MAC CE.

2.6	DL Data Burst

According to definition in SA2, a data burst is a set of multiple PDUs generated and sent by the application in a short period of time and a Data Burst can be composed by one or multiple PDU Sets. During a Data Burst, periods of data transmission inactivity should not be assumed. And the duration of Data Bursts may vary. For DL, RAN is aware of the last PDU indication of a data burst according to the End of Data Burst indication in the last PDU of a data burst from UPF, it is helpful to timely determine all PDUs of a data burst were received from UPF. After successfully transmitting the data burst to UE, NW can timely indicate UE enter C-DRX inactive mode to save power.

Proposal 6: Confirm that NW can send UE to DRX inactive mode according to the End of Data Burst indication.

2.7	PDU set information over Xn and F1 interfaces

In the following we discuss XR-awareness information provided between RAN nodes in case of DC, CU-DU split and handover case.

    - Handover and dual connectivity

During handover procedure, the PDU set QoS parameter and characteristics may need to be transmitted from source gNB to target gNB. There is a case that only a part of the PDUs of a PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be forwarded and transmitted by the target gNB. In this case, it is beneficial to be aware of the remaining 5G PSDB for transmitting the remaining PDUs of the PDU set for more accurate QoS management and scheduling for target gNB. The PDU set information may be also needed for the forwarding data.

For dual connectivity, the similar information as handover may also needed between MN and SN.

    - CU-DU split

When receiving QoS flow from UPF, the CU performs QoS flow to DRB mapping. After the mapping, the CU may need to provide the PDU set QoS parameter and characteristics of the DRB to the DU via control plane signaling and PDU set information of the DRB to DU via GTP-U extension header since the DU is responsible for the scheduling. For UL, the gNB-DU may also need to forward some PDU set information to CU e.g. to help PDCP re-ordering function in CU.

Proposal 7: The PDU set QoS parameter and characteristics and PDU set information may also need to be transmitted between source gNB and target gNB, MN and SN, or CU and DU.

- Conclusion

In this contribution, we discuss XR-awareness. According to above observations, we have the following proposals:

Proposal 1: UE transmits the UL jitter range information associated with UL periodicity per logical channel or per QoS flow, to NG-RAN via UAI.

Proposal 2: To support the UL PDU set discarding of PDU set with low PDU set importance, confirm the UL PSI awareness at UE is up to UE implementation.

Proposal 3: NW configures the PSIHI per DRB to UE for supporting of UL PDU set discarding.

Proposal 4: RAN2 to discuss to support the remaining delay budget report for a PDU set, UE uses the discard timer length as the allowed Uu PSDB.

Proposal 5: RAN2 to discuss how to indicate EODB to the gNB for UL, e.g. by means of triggering “zero-BSR” or by some new indication in MAC header or by a new MAC CE.

Proposal 6: Confirm that NW can send UE to DRX inactive mode according to the End of Data Burst indication.

Proposal 7: The PDU set QoS parameter and characteristics and PDU set information may also need to be transmitted between source gNB and target gNB, MN and SN, or CU and DU.

- References

RAN2#121bis chairman note.

23.700-60-i00, Study on XR (Extended Reality) and media services(R18)

R2-2303226, Discussion on PDU sets awareness in RAN, Lenovo